3GPP TSG-RAN WG4 Meeting #107	R4-2308726

Inchon, Korea, 22 – 26 May, 2023

Source:	ZTE Corporation

Title:	Discussion on L1-RSRP measurement requirements for simultaneous DL reception from different directions

Agenda Item:	8.8.3.2

Document for:	Approval

- Introduction

In RAN plenary 98 meeting, a revised WID[1] for Rel-18 was approved to further clarify the scope of the  simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier. The main update is to remove the sub bullet related with L3 measurement delay and put the L3 measurement related aspects into the NOTEs. The objective in the revised WID is as follows:

- Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4 layer DL MIMO - Enhanced RF requirements: - Specify RF requirements, mainly spherical coverage requirements, for devices with simultaneous reception from different directions with different QCL TypeD RSs - The legacy spherical coverage requirement for reception from a single direction will be kept - PC3 will be prioritized, other power classes should be considered after the PC3 requirements framework is finalized - Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier - Enhanced RRM requirements: - The following requirements should be studied and specified if necessary: - L1-RSRP measurement delay - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI - Receive timing difference between different directions (different QCL Type D RSs) NOTEs: - The case of single TCI is handled as a second priority. Additional aspects related to single TCI can be further revisited. - The work on L3 measurement related aspects for scheduling/measurement restriction requirements is not precluded. - Further check in RAN #100 whether to include other L3 measurement related aspects and objectives subject to RAN4 progress.

In last RAN4 meeting, RAN4 has spread extensive discussion around the L1-RSRP measurement and some agreements were achieved in [2].

Issue 1-1: Which reference signals to be considered for multi-rx L1 measurement requirements Agreement from GTW: - Multi-Rx L1 measurement requirements are defined under assumption of the following reference signals availability - CSI-RS + CSI-RS: CSI-RS reference signals are transmitted from the two TRPs - SSB + SSB: SSB signals are transmitted from the two TRPs - FFS - SSB + CSI-RS: SSB and CSI-RS signals are transmitted from different TRPs - FFS if same or different RS combinations would apply for different RRM requirements - FFS whether to consider simultaneous and/or non-simultaneous RS transmission from different TRP to define RRM requirement Issue 1-2: Beam sweeping/Sharing/Scaling factor reduction Agreement from GTW: - Introduce enhanced RRM requirements based on faster beam sweeping with multi-Rx chains based on UE capabilities - Option 1: Introduce beam sweeping factor reduction - Other options not precluded - FFS on specific conditions when enhancement apply - FFS if any signalling is needed - FFS on details of UE capabilities Agreements: How to capture the enhanced requirements (based on the agreement of enhanced requirements) in the spec due to faster beam sweeping can be discussed during CR stage. Issue 1-2-3: Whether to consider reducing the sharing factor for L3 and L1 Agreements: NO, do not consider enhancement to sharing factor for L3 and L1 in this WI Issue 1-3-4: Which reference signals to be considered for GBBR configuration Agreements: SSB+SSB or CSI-RS + CSI-RS can be supported for GBBR. Way forward: FFS if SSB+CSI-RS is supported for GBBR. Issue 1-4-3: L1-RSRP measurement accuracy requirements It was agreed to discuss in performance part

In this document, we provide our further analysis focus on L1-RSRP measurement around simultaneous multi-panel reception.

- Discussion

Regarding to L1 measurement enhancement under multi-panel Rx, it has been approved that there is not any room to reduce the sample number in 104bis meeting. So we can explore from the following aspects:

- The exact combinations of simultaneous reception around L1-RSRP measurement

- The fast beam sweeping during L1-SRSP measurement

- Group based reporting requirements

- Besides all above, some issues were discussed in the measurement restriction and scheduling restriction aspects. So we discuss them in another document from us.

2.1 The exact combinations of simultaneous reception around L1-RSRP measurement

- During last meeting, it is approved that: the scenarios for mTRP simultaneous reception is to be covered as part of scheduling restrictions/measurement restrictions, L1 measurement, and TCI state switching agenda items. So before discuss other aspects on L1-RSRP measurement, we want to firstly provide our views on the exact combinations.

In RAN4 106 meeting, it has been approved that R17 group-based reporting is used as a prerequisite to define requirement for simultaneous reception, such simultaneous reception includes Data/Data, RS/RS and Data/RS cases. With such prerequisite, NW can identify which beam pairs can be used for simultaneous reception therefore when NW configures the RS or schedules the data, the available beam pairs can be considered. However before receiving the group-based report, the NW is not sure about the beam pairs, so the NW would not schedule the data which has be simultaneously received with other data or RS. The UE would detect the RS in TDM manner

Observation 1: Before receiving the group-based report, the NW is not sure about the beam pairs so would not schedule the data which has be simultaneously received with other data or RS. The UE would detect the RS in TDM manner.

- So when we discuss the combinations or scenarios of simultaneous reception, which means the NW has received the group-based reporting from UE. Given that valid group-based reporting is received by the NW, then the following combinations of simultaneous scheduling/configuration are possible:

- Data + Data

- L1-RSRP SSB + L1-RSRP SSB

- L1-RSRP CSI-RS + L1-RSRP CSI-RS

- L1-RSRP CSI-RS + L1-RSRP SSB depends on RAN1 LS reply

Proposal 1: Given that valid group-based reporting is received by the NW, then the following combinations of simultaneous scheduling/configuration are possible:

- Data + Data

- L1-RSRP SSB + L1-RSRP SSB

- L1-RSRP CSI-RS + L1-RSRP CSI-RS

- L1-RSRP CSI-RS + L1-RSRP SSB depends on RAN1 LS reply

2.2 The fast beam sweeping during L1-SRSP measurement

During RRM measurement and L1-RSRP measurement, for SSB based, the UE always need to perform Rx beam sweeping to acquire the Rx beam tracing. The different point is that for RRM measurement, coarse beam is assumed since of the lack of prior information. While for L1-RSRP measurement, it is only oriented to the serving cell, so fine beam is assumed during the SSB based L1-RSRP measurement.

During last meeting, it has approved to introduce the enhanced RRM requirements based on faster beam sweeping based on UE capabilities. While some details are still FFS.

- Introduce enhanced RRM requirements based on faster beam sweeping with multi-Rx chains based on UE capabilities - Option 1: Introduce beam sweeping factor reduction - Other options not precluded - FFS on specific conditions when enhancement apply - FFS if any signalling is needed - FFS on details of UE capabilities

For the UE configured with multiple antenna panels, since of some prior information acquired by the UE, so for Rx beam sweeping of L1-RSRP measurement, maybe one antenna panel has been down-selected through the previous RRM measurement. Under such assumption, the Rx beam sweeping of L1-RSRP measurement is only performed on a single antenna panel. Another UE implementation is that still all antenna panels are referred to during Rx beam sweeping of L1-RSRP measurement.

- Assumption 1: L1-RSRP measurement is performed only by a single panel down-selected by RRM measurement

- Assumption 2: L1-RSRP measurement is performed by multiple panels

Observation 2: For the case of a single panel has been down-selected by previous RRM measurement(named as Assumption 1, the Rx beam sweeping during L1-RSRP measurement is performed only by a single panel. While for other case(named as Assumption 2), the Rx beam sweeping during L1-RSRP measurement is performed by multiple panels.

Combined with the group-based report, once the UE has reported the beam pair(s) to NW, which means the UE can receive such beam pair(s) simultaneously. Under such assumption, it can be assumed that for the UE reported SSB/CSI-RS or other SSB/CSI-RS QCL-typeDed with the reported SSB/CSI-RS within a group pair, so as to realize simultaneous reception with another beam direction, the UE would receive such SSB/CSI-RS only based on a single panel rather than multiple panels.

Under Assumption 1 furthermore the L1-RSRP measurement occasions for TRP1 and TPR2 are overlapping in time domain, it is convenient to realize simultaneous two L1-RSRP measurement procedures for two TRPs. As shown in the below figure 1. In this case, UE can perform two L1-RSRP measurements for the two TRPs in parallel. while for the UE not capable of simultaneous multi-panel Rx, it can only perform the two L1-RSRP measurement sequentially, as shown in below Figure 2.

- Figure 1. Perform L1-RSRP measurement for two TRPs in parallel

- Figure 2. Perform L1-RSRP measurement for two TRPs sequentially

The total measurement period can be reduced since of the relaxation on measurement restriction. However regarding to the beam sweeping factor, we can not see direct enhancement.

Proposal 2: Under Assumption 1, i.e. L1-RSRP measurement is performed only by a single panel down-selected by RRM measurement, no room to reduce the scaling factor of Rx beam sweeping but the measurement restriction can be relaxed.

Under Assumption 2, as shown in Figure 3, it is obvious that the fully beam sweeping needs multiple panels’ participation, for the UE capable of simultaneous multi-panel Rx, UE can perform partial beam sweeping by each panel at the same time as below. Furthermore, such L1-RSRP measurement seems can perform before or after the group-based reporting, i.e. not restricted by the group-based reporting. So the beam sweeping factor can be reduced by a half, i.e. from N to N/2. Further more, one condition is that each panel can cover half of the fully beam directions. If the beam coverage of multiple panels are different, e.g. one panel covers 2 beam directions and another panel covers 6 beam directions, as shown in figure 4, under such situation, the beam sweeping factor can not relax from N to N/2, but only relax from N to maximum[N1, N2], here N1and N2 refer to the number of beams covered by each panel respectively.

- Figure 3. Each L1 measurement needs beam sweeping including multiple panels

- Figure 4. Each panel covers different number of beam directions

Proposal 3: Under Assumption 2, i.e. L1-RSRP measurement is performed by multiple panels, UE can perform partial beam sweeping by each panel at the same time. So the beam sweeping factor can be reduced.

- For the case of each panel can cover half of the fully beam directions, the beam sweeping factor can be reduced by a half, i.e. from N to N/2.

- For other cases, the beam sweeping factor can be reduced from N to maximum[N1, N2], here N1and N2 refer to the number of beams covered by each panel respectively.

As the above analysis, regarding the condition when enhancement apply, we believe the two aspects should be considered:

- The UE perform the L1-RSRP measurement for a single TRP based on multiple panels;

- At one measurement occasion, the UE is not scheduled to receive data from the other TPR;

- Not binding to the group-based report, i.e. such enhancement is allowed before the group-based reporting(maybe this is not a type of condition);

- UE is capable of this enhancement.

Proposal 4: The conditions of the enhanced fast beam sweeping during L1-RSRP measurement can be considered from the following aspects:

- The UE perform the L1-RSRP measurement for a single TRP based on multiple panels;

- At one measurement occasion, the UE is not scheduled to receive data from the other TPR;

- Not binding to the group-based report, i.e. such enhancement is allowed before the group-based reporting(maybe this is not a type of condition);

- UE is capable of this enhancement.

Regarding the corresponding signalling and UE capability, to our understand, the common solution is applicable here, i.e. when the UE reports supporting such UE capability, then the NW need to indicate the UE whether enable such enhancement through the NW signalling given that the above conditions are met. Once such enhancement is enabled, then UE should perform the L1-RSRP measurement based on the enhanced requirement. Regarding the exact enhanced requirement, in last meeting, it is approved to discuss during CR stage.

Proposal 5: Regarding the corresponding signalling and UE capability, to our understand, the common solution is applicable here, i.e. when the UE reports supporting such UE capability, then the NW need to indicate the UE whether enable such enhancement through the NW signalling.

2.3 Group based reporting requirements

To our understand, whether need to define requirements specially for group based reporting, which was discussed in last meeting but failed to reach consensus since RAN4 still has many uncertainty about the group-based reporting, so we send LS to RAN2 for further clarification. Even we still need to wait for RAN1’s reply, but we can continue some related discussion to align companies’ understanding.

The reason of specific measurement requirements for group-based reporting maybe necessary lies in the following aspects:

- If the mixed RSs(SSB in Set 1 and CSI-RS in Set 2) based measurement is clarified as allowed by RAN1, then until right now, RAN4 does not define any L1-RSRP measurement requirements based on the mixed RSs, so RAN4 needs to discuss how to define L1-RSRP measurement requirements for such mixed RS scheme.

- Since different from legacy L1-RSRP reporting, beam pair(s) should be reported in the group-based reporting, so the whether the legacy sample number is enough or need to enhance, some discussion is needed. Considering the legacy sample number is defined to determine a single beam quality.

- Group-based reporting is pure RAN1 enhancement, not any related conditions involved in, so as to apply such reporting better in simultaneous multiple panel reception, whether some conditions are needed, and the applicability of the measurement requirements should also be discussed.

- Therefore, it is necessary to discuss the potential new measurement requirements for group-based reporting from the above aspects.

Proposal 6: The group-based reporting is pure RAN1 enhancement, Since different from the legacy L1-RSRP measurement, it is necessary to discuss whether define new measurement requirements for group-based reporting from the following aspects:

- The potential mixed RSs measurement scheme;

- The sample number;

- The applicability condition.

- Conclusion

In this contribution, we have the following observations and proposals around L1-RSRP measurement for simultaneous DL reception from different directions:

Observation 1: Before receiving the group-based report, the NW is not sure about the beam pairs so would not schedule the data which has be simultaneously received with other data or RS. The UE would detect the RS in TDM manner.

Proposal 1: Given that valid group-based reporting is received by the NW, then the following combinations of simultaneous scheduling/configuration are possible:

- Data + Data

- L1-RSRP SSB + L1-RSRP SSB

- L1-RSRP CSI-RS + L1-RSRP CSI-RS

- L1-RSRP CSI-RS + L1-RSRP SSB depends on RAN1 LS reply

Observation 2: For the case of a single panel has been down-selected by previous RRM measurement(named as Assumption 1, the Rx beam sweeping during L1-RSRP measurement is performed only by a single panel. While for other case(named as Assumption 2), the Rx beam sweeping during L1-RSRP measurement is performed by multiple panels.

Proposal 2: Under Assumption 1, i.e. L1-RSRP measurement is performed only by a single panel down-selected by RRM measurement, no room to reduce the scaling factor of Rx beam sweeping but the measurement restriction can be relaxed.

Proposal 3: Under Assumption 2, i.e. L1-RSRP measurement is performed by multiple panels, UE can perform partial beam sweeping by each panel at the same time. So the beam sweeping factor can be reduced.

- For the case of each panel can cover half of the fully beam directions, the beam sweeping factor can be reduced by a half, i.e. from N to N/2.

- For other cases, the beam sweeping factor can be reduced from N to maximum[N1, N2], here N1and N2 refer to the number of beams covered by each panel respectively.

Proposal 4: The conditions of the enhanced fast beam sweeping during L1-RSRP measurement can be considered from the following aspects:

- The UE perform the L1-RSRP measurement for a single TRP based on multiple panels;

- At one measurement occasion, the UE is not scheduled to receive data from the other TPR;

- Not binding to the group-based report, i.e. such enhancement is allowed before the group-based reporting(maybe this is not a type of condition);

- UE is capable of this enhancement.

Proposal 5: Regarding the corresponding signalling and UE capability, to our understand, the common solution is applicable here, i.e. when the UE reports supporting such UE capability, then the NW need to indicate the UE whether enable such enhancement through the NW signalling.

Proposal 6: The group-based reporting is pure RAN1 enhancement, Since different from the legacy L1-RSRP measurement, it is necessary to discuss whether define new measurement requirements for group-based reporting from the following aspects:

- The potential mixed RSs measurement scheme;

- The sample number;

- The applicability condition.

- Reference

- RP-223527 “Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception”, Qualcomm Incorporated.

- R4-2306319 “WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1)”, Ericsson.