3GPP TSG-RAN WG4 Meeting #107									                          R4- 2308748

Incheon, South Korea, May 22 – 26, 2023

Agenda item:			8.20.2.2.3

Source:	Ericsson

Title:	TP to TR 38.858 on Feasibility of FR1 UE aspects

Document for:	Approval

# Introduction

In the last RAN4#106bis-e meeting, TPs from different companies addressing the feasibility of UE aspects were consolidated and approved in [1]. This paper captured the agreements in the WF [2] and proposed some updates and comments for TR 38.858 on the Feasibility of UE aspects, i.e., 10.6.

# References

- R4-2305970 TP to TR 38.858 on Feasibility of FR1 UE aspects.

- R4-2305969 WF for SBFD feasibility study and requirements impact from UE aspect.

# Text Proposal

## <<Start of Change for TR 38.858>>

#### 10.6.1.1	UE-UE co-channel inter-subband CLI modeling

Editor's note: This section captures the CLI modeling.

##### 10.6.1.1.1 Overview and analysis framework

The objectives of UE-UE co-channel inter sub-band CLI modeling is to analyze the impact of interference that occurs between two UEs in close proximity, operating on adjacent sub-bands within the same channel. This interference occurs when UL transmission of an aggressor UE in the channel interferes with the DL reception of a victim UE in the same channel.

For this SI RAN4 has decided to use typical UE parameters in the analysis, as opposed to worst-case parameters most often used for minimum performance requirements.  The evaluation of gNB performance improvements is underway, taking into account a population of typical UEs rather than worst-case UEs.

The sub-band co-channel selectivity is a measure of the ratio of the receive power on the assigned sub-band to the receive power on the adjacent sub-band after FFT operation. In an ideal scenario, the UL transmission of the aggressor UE should not impact the DL reception of the victim UE due to the OFDM wave orthogonality. However, non-ideal FFT suppression can cause interference to the victim UE, particularly when the UL sub-band has frequency errors and is not time-synchronized with the DL sub-band. The analysis indicates that the IBE interference is higher and dominates the sub-band co-channel selectivity, and frequency and time offset are not significant factors influencing UE-UE interference. It is worth noting that the RF degradations can cause inter-subband interference as well and the impact will depend on the targeted Rx IM and EVM performance. The measurement data submitted by [one] company shows the subband selectivity of FR1 UE can be 33 dB. Nonetheless, this interference will not be any worse than the selectivity value. For this reason, the 33 dB was agreed for modeling the inter-sub-band selectivity.

For legacy UE, no sub-band filtering is considered.

To model the AGC and NF modeling for co-channel CLI in a system level simulation, a fixed value noise figure of 9 dB is used.

Apart from the selectivity, it is important to mention that degradation can be caused by transmitter leakage from the UL sub-band into the DL sub-band. For co-channel case, the leakage was agreed to be modelled using IBE based model with granularity of 1 RB. Additionally, the IQ image contribution for the IBE model for co-channel CLI can be ignored for the DUD configuration since the image is fully contained in the uplink sub-band.

##### 10.6.1.1.2 UE co-channel Tx model

Inband emissions

For UE co-channel Tx model, RAN4 has decided to use the IBE requirements from 38.101-1 clause 6.4.2.3 as shown in Table 10.6.1.1-1 in the feasibility study. This model consists of three parts, General, IQ image, Carrier leakage. In the system level simulation, the general and IQ image parts shall be considered, while the carrier leakage part can be ignored in the feasibility study. For DUD configuration, the IQ image from the uplink is fully contained in the UL sub-band and does not land in the DL subband, thus the IQ image can also be ignored in the simulation. The granularity of this model is 1RB and it is not pursued to simplify this model to a frequency flat model. It is understood these requirements are minimum performance requirements as opposed to typical requirements. RAN4 has agreed to use typical requirements for the UE parameters,  Since typical values for the UE parameters were not determined, the formulation from the MPS is being used.

Table 10.6.1.1-1: Requirements for in-band emissions in TS 38.101-1

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28 | Image frequencies when output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 | Image frequencies when output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm | Carrier leakage frequency (NOTES 4, 5) |
|  |  | -25 | 0 dBm ≤ Output power ≤ 10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power < 0 dBm |  |
|  |  | -10 | -40 dBm ≤ Output power < -30 dBm |  |

##### It should also be assumed the LO location is in the center of the channel for the purposes of system studies in RAN4. The LO location is important as it allows placement of the image.10.6.1.1.3 UE co-channel Rx model

For UE co-channel Rx model, currently there is no corresponding RF requirement for this model. In the feasibility of UE co-channel Rx model, the definition of Sub-band/In-channel selectivity is introduced for SBFD feasibility study purpose:

- For one input level and one jammer level, Sub-band/In channel selectivity is the ratio of the receive power on the assigned sub-band to the receive power on the adjacent sub-band after FFT operation.

For legacy UE, no sub-band filtering is considered, and no rejection/attenuation due to RF/BB filtering is assumed and only the selectivity and performance of the FFT is studied. For now, 33 dB was proposed in RAN4 for subband/in-channel selectivity considering FFT operation.

For new SBFD aware UE, it needs further study on whether sub-filtering can be considered or not. The subband/in-channel selectivity needs further study.

Thermal self-noise performance

RAN4 decided on a simple fixed-value noise figure model for the UE receiver. Generally, the receiver noise figure will vary with the input power level, however the single value noise figure model was considered to be sufficient for the purpose of system studies for SBFD. RAN4 decided on a NF of 9 dB.

Effect of jammer – non-thermal self-noise aspect

There are a few factors to consider in determining the in-subband interference in the presence of a co-channel jammer. With an in-channel adjacent-subband interferer the 3rd order distortion, reciprocal mixing, residual sideband, quantization noise, phase noise, ADC distortion, and analog filtering should be considered.

Measurements were made of a UE receiver for various signal levels, interferer levels, interferer offsets, sub-band bandwidths, and interferer bandwidths. The measurements included the entire receiver, which includes everything through the FFT. 120 various conditions were measured.

The interference in the victim sub-band can be modelled as 33 dB below the total input power level. Interference is approximately frequency flat across the victim.

FFT leakage and selectivity

In the SBFD system an aggressor UE (UE2) operating in the UL sub-band may interfere with a UE (UE1) receiving in the adjacent DL sub-band. The UE2 UL may arrive at UE1 misaligned in time or frequency, and potentially causing UE1 DL SINR degradation in the FFT.

The simulation involved the conversion of the OFDM waveform to a spectrum using FFT, along with the introduction of timing and frequency errors. A 5 RB guard band was assumed. The analysis revealed that time-misalignment was the main cause of the spectral leakage, and even minor timing errors resulted in leakage.

It appears reasonable to consider the leakage as a single average value, the data shows about 33 dB down from the jammer level would be appropriate. The leakage effect should be taken into consideration and compared to the interference caused by the aggressor IBE. IBE interference is higher than the FFT leakage so RAN4 has decided to exclude any factor for FFT leakage. Further, RAN4 has concluded no factor is needed for FFT selectivity.

Figure 10.6.1.1.3-1: FFT leakage with time and frequency misaligned blocker (5 RB guard band)

##### 10.6.1.1.4 Discussion process

- 10.6.1.1.4.1 RAN4 Agreement

### 10.6.2	Summary

Editor's note: This section captures the conclusion of feasibility.

[

For co-channel interference case, the RF effect is dominant, and the frequency and time offset are not significant factors influencing UE-UE interference. The leakage is modelled using IBE based model.

As for the adjacent channel case, it was decided to assume power-dependent ACLR of the aggressor UE and selectivity of the victim UE when modeling adjacent channel interference.

For legacy UE, no sub-band filtering is considered,

A fixed value noise figure of 9 dB has been used to model the AGC and NF modeling for co-channel and adjacent channel CLI in a system level simulation.

]

## <<End of Change>>