3GPP TSG-RAN WG4 Meeting #107	R4-2308755

Incheon, Korea, May 22nd - May 26th, 2023

Source	: Nokia, Nokia Shanghai Bell

Title	: Remaining issues on unified TCI switching delay requirement

Agenda item	: 5.2.3.1

Document for	: Approval

Introduction

RAN4 #106 continued the discussions on the remaining issues of Rel-17 feMIMO WI with RRM core requirements. The outcome of the unified TCI switching delay requirements was captured in the WF in [11].

A few aspects are still open related to FeMIMO such as Time tracking, TCI state with SSB as PL-RS and the definition of PL-RS.

The aspect of PL-RS has its roots in Rel-16 MIMO where RAN4 defined requirements for Pathloss reference signal switching delay in section 8.14. Those requirements were based on the RAN1 wording and introduced the condition ‘maintained’ however without defining ‘maintained’. We have addressed Rel-16 related discussion in a separate paper [12] with the proposed definitions in a CR [13]

For facilitating the discussions, we note that RAN4 sent an LS to RAN1 asking about the number of PL-RS to be maintained by the UE [2]. This LS was replied in [3].

In this document, the following issues are discussed:

- Whether UE need to track UL time/frequency if source RS in UL TCI state is not in the DL active TCI list (Issue 1-1-3)

- MAC-CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2 (Issue 1-2-1)

- Definition of ‘maintained PL-RS’ (Issue 1-3-1)

Discussion

Time tracking if source RS in UL TCI state is not in the DL active TCI list

RAN4 has been discussing whether the UE needs to track the time for the active UL TCI state. The discussion continued in RAN4#106 meeting without any conclusion. Only outcome captured:

- To be confirmed:

  - [Do not define requirements for the case if source RS in UL TCI state is not in DL active TCI state list]

Not defining any requirements for this scenario basically means that UE behavior is not defined – for Rel-17. However, one may expect that the question remains open in coming (ongoing) releases.

In Rel-17, the scenario for UL m-TRP (e.g. where the TRPs are from different cells) considers that the propagation delay difference between the signals from different TRPs is at most the CP length (±CP/2). This guarantees that the UE is capable of correctly receiving from two TRPs. Assumption is that these two TRPs are not located at the same physical location.

Rel-17 mTRP conditions ensures that UE can receive from the two TRPs.

Currently, for the DL TCI state switching and DL time tracking, according to specification TS 38.306, the UE is required to perform time/frequency tracking for each TCI state in the list of active TCI states. Therefore, the agreement in RAN4 #104-bis-e is that no additional time frequency tracking is needed provided the source RS of the UL TCI state is in the active TCI state list.

No additional time frequency tracking is needed provided the source RS of the UL TCI state is in the active TCI state list (agreement in RAN4 #104-bis-e)

However, it is a UE capability how many TCI states the UE support being in the list of active TCI states for non-unified TCI framework (legacy). This can be between 1 and 8. Currently, the same capabilities for unified TCI state have not yet been defined for the unified TCI states.

In Rel-16, the DL/UL TCI state change relied on channel reciprocity. In Rel-17, for the unified TCI state work this may not always be applicable, and the following situation is possible:

- The DL TCI state and the UL TCI state are associated to different TRPs which may not be collocated.

Figure 1 - Propagation delay difference in an intra-cell m-TRP scenario

Up to Rel-16 the DL/UL relied on channel reciprocity.

In Rel-17, unified TCI state framework enables decoupled DL TCI state and UL TCI state handling.

Related to DL time tracking, let’s consider that the DL propagation delay difference is equal to CP, and that the UE is deriving timing from DL-RS associated to TRP 1 while the UE is transmitting to TRP 2, as in Figure 1. Currently, according to UE transmit timing requirements in TS 38.133, Table 7.1.2-1 the UE shall follow the DL frame timing change of the reference cell in connected state and keep the UE transmit timing error within +-Te. The reference cell is referring to the cell in which the UE is transmitting in UL.

The reference cell in section 7 is referring to the cell in which the UE is transmitting in UL.

The Te values are a fraction of the CP length for each SCS, as in the table below, considering that the SCS of the SSB and the SCS of the uplink signals are the same:

| SCS [kHz] | CP Length [μs] | Te[μs] | Te/CP [%] |
| --- | --- | --- | --- |
| 15 | 4.7 | 0.39 | 8.3% |
| 30 | 2.3 | 0.26 | 11.3% |
| 120 | 0.59 | 0.11 | 19% |

The UE UL transmit timing requirements are based on that the UE is tracking the DL frame timing of the reference cell, which is the cell towards which the UE is transmitting in UL. The current requirements do not account the new aspect introduced by unified TCI state work which includes mTRP operations.

There is no mentioning in the UE requirements related to tracking of the DL frame timing from any specific TRPs of a cell. From current requirements it is obvious that the UE cannot use the frame timing from one TRP (e.g. TRP1) and use that as reference for UL transmit timing to another TRP (here TRP2). The UE shall use the DL frame timing from TRP towards which the UE is transmitting in UL as reference (in this example UE shall use DL RS from TRP2 when transmitting to TRP2).

UE shall use the DL frame timing of the TRP towards which the UE is performing UL transmission.

The UL transmit timing requirement is in place to ensure the network reception of the UL signals transmitted by the UE. If the UL transmit timing is not as expected, reception of the UL transmissions from the UE may be degraded or may not be received at all by the gNB. This will of course also impact the UE performance and overall performance.

The UL transmission is relative to the DL reception in the cell/TRP (towards which the UL transmission is targeted). Hence, for not collocated deployments it cannot be assumed that any received DL frame timing from the cells/a TRP of the cell can be used as reference for UL transmission to another cell/TRP.

The current specification is not clear related to ‘active’ TCI state for UL for non-unified TCI state as the assumptions are based on UL/DL reciprocity. For unified TCI states the UE behavior is not clear regarding how a UE manages active UL TCI in a similar manner to active DL TCI.

For DL case, if target TCI state is in the active TCI state list for PDSCH/PDCCH, the UE is tracking fine time and frequency synchronization on the resource RS. As captured in 38.306, DL TCI case clearly specifies maxNumberActiveTCI-PerBWP in tci-statePDSCH, and a UE should track time and frequency sync.

tci-StatePDSCH TS38.306 - maxNumberActiveTCI-PerBWP indicates the maximum number of activated TCI-states per BWP per CC, including control and data. If a UE reports X active TCI state(s), it is not expected that more than X active QCL type D assumption(s) for any PDSCH and any CORESETs for a given BWP of a serving cell become active for the UE. The UE shall include this field. Note the UE is required to track only the active TCI states.

Hence, when DL TCI state change is indicated with a target TCI state in the active TCI state list, the UE can switch to the target TCI state and can immediately receive and demodulate the PDCCH/PDSCH, after the TCI state switch delay without need for reception of additional SSB/TRS.

A UE shall acquire and keep time and frequency tracking of the cell and to the source RS associated to the UL TCI for UE to perform any UL transmission. Keeping time and frequency tracking of the cell is also the condition necessary for the UE to perform DL reception.

A UE is not allowed to transmit PUCCH/PUSCH/SRS (or any UL signal) independently from or without firstly perform DL synchronization to the cell/TRP towards which the UE transmits. This is clear from UE transmit timing requirements in section 7.1. The maxNumberActiveTCI-PerBWP under tci-StatePDSCH is about supported number of activated TCI-states per BWP per CC for both DL for non-unified TCI states and associated UL.

For unified TCI states the UE can be configured with a number of TCI states which can be then activated using a MAC command (Unified TCI state activation/deactivation MAC CE). It is possible to activate up to 8 joint and/or separate TCI states (we assume the actual number supported by the UE is a UE capability).

Each of the activated TCI states can be indicated afterwards for active use by the UE. Hence, the UE shall use the indicated TCI states for reception and transmission. Such indication is done using DCI. The DCI indicated activated TCI states are indicated TCI states and an indicated TCI state can be of any type of:

- PDCCH,

- PDSCH,

- CSI-rs,

- PUCCH,

- PUSCH,

- SRS

Focusing on the UL the UL spatial filter is derived from the RS of DL QCL Type D of the TCI state (the DL RS associated with the indicted UL TCI state).

The activation of UL TCI states is done by MAC CE. Once UL TCI state is activated, it can be taken into use by DCI indication (as per the usual unified TCI state framework).

To enable fast TCI change of the activated TCI states using DCI – and adding a TCI state to the indicated TCI states, the UE is required to track the DL timing of the activated states including any DL RS associated to an UL TCI state. This is the same requirement as was defined in Rel-15.

A UE shall acquire and keep time and frequency tracking of TCI states in the active TCI state list.

A UE shall acquire and keep time and frequency tracking of the DL RS associated to the UL TCI state in the active TCI states.

An UL TCI state will never be configured without an associated DL RS. Hence, any UL TCI state in the activated TCI state list will always have an associated DL RS configured.

For the scenario when MAC activates a new TCI state in the active TCI state list, it may happen that the network indicates an UL TCI state before the UE has acquired the accurate time tracking based on the associated DL RS.

If source DL-RS associated with the indicated UL TCI state is unknown (e.g. it is not in the DL active TCI list), additional time for time/frequency tracking should be allowed for the indicated UL TCI state.

It is our understanding that the UE shall track time and frequency of the associated DL RS for an UL TCI state when the UL state is in the active TCI state list.

UE shall track time and frequency of the associated DL RS for an UL TCI state when the UL state is in the active TCI state list.

However, as this is rather essential also for Release 18 work, RAN4 can also clarify with RAN1 whether the UE is required to time track of DL-RSs associated to UL TCI states in the active TCI state list. Additionally, RAN4 can ask RAN1 if there is always an associated DL RS configured with an UL TCI state.

Check with RAN1 if UE is required to do DL time tracking for an UL TCI state in the active TCI state list.

However, for the issue:

- Whether UE need to track UL time/frequency if source RS in UL TCI state is not in the DL active TCI list

We this as a network configuration error (there would be no DL RS for T/F tracking associated with an UL TCI state). And RAN4 usually do not define anything which is regarded as network configuration errors.

Definition of maintained PL-RS related discussion

The aspect of PL-RS has been discussed for quite many meetings. However, in last meeting RAN4 almost reached agreements on the detailed wording.

The aspect of PL-RS includes multiple different parts:

- If the UE is configured with more than 4 PL-RS

- Common understanding of PL-RS and definition of maintained PL-RS

- MAC CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2

RAN4 sent an LS to RAN1 which has been replied in LS reply [3] on active TCI state list for UL TCI.

Following we discuss the open issues by initially addressing the LS reply from RAN1.

Number of maintained PL-RS

RAN4 sent an LS to RAN4 in August meeting for which RAN1 has now sent reply:

Question 1: What is the relationship between qd and active UL (or joint) TCI state list? How does network configure the RS resource indexes qd? Answer 1: According to 38.213, “the RS index for obtaining the downlink pathloss estimate for PUSCH, PUCCH, and SRS transmission is provided by PL-RS associated with or included in the indicated TCIState or UL-TCIstate except for SRS transmission that is not provided followUnifiedTCIstateSRS” [...] ”if followUnifiedTCIstateSRS is not provided for a SRS resource set and for a SRS resource from the SRS resource set, [...] a RS index qd for obtaining a pathloss estimate for the SRS transmission is provided by PL-RS associated with or included in the TCIState or UL-TCIState of an SRS resource with lowestSRS-ResourceId in the SRS resource set” Question 2: Is UE expected to maintain the pathloss RSs of all the pathloss reference RS in the active UL (or joint) TCI list? Answer 2: A UE is expected to maintain the pathloss RSs of all active UL (or joint) TCI states. But the total number of different path-loss RSs of all active UL (or joint) TCI states is not expected to be larger than 4. Question 3: What is the UE behavior when number of active UL (or joint) TCI states is larger than 4 if corresponding pathloss RSs need to be maintained? Answer 3: The UE is not required to maintain more than 4 PL-RS. The same PL-RS can be used in more than one UL or Joint TCI states.

From the LS reply it is clear that the qd RS index for obtaining the DL pathloss (i.e. the DL PL-RS) for PUSCH, PUCCH and SRS transmission is provided by network configuration. Specifically, for PUSCH, it is provided the PL-RS associated with or included in the TCI state or UL-TCIstate.

PL-RS for obtaining the DL pathloss estimate for UL transmission is always provided for PUSCH, PUCCH and SRS.

From the LS it is also clear that the UE is required to maintain the pathloss RS (PL-RS) of all active UL TCI states and joint TCI states. It is also clear that currently the UE is not required to maintain a total of more than 4 different PL-RS for all active UL TCI states and joint TCI states.

UE is required to maintain the pathloss RS (PL-RS) of all active UL TCI states.

UE is required to maintain the pathloss (PL-RS) of all active joint TCI states

UE is not required to maintain a total of more than 4 different PL-RS for all active UL TCI states and joint TCI states.

However, RAN1 is silent on what it means that the UE is to ‘maintain’ the PL-RS.

RAN1 specification is not clear on what ‘maintain’ means.

Finally, RAN1 also clearly states that the UE is not required to maintain more than 4 different PL-RS for UL TCI states. However, RAN1 also notes that the same PL-RS can be used for more than one UL or joint TCI state.

Definition of maintained

The RAN1 replies are useful in clarifying the RAN4 discussion but does not solve all open issues. However, based on the reply we can initially conclude that provided the UE is not having more than 4 different PL-RS for all active UL TCI states for PUSCH, PUCCH and SRS transmissions, the UE is required to maintain the PL-RS and hence the PL-RS will always be maintained.

If UE is not having more than 4 different PL-RS for all activated UL TCI states for PUSCH, PUCCH and SRS transmissions, the PL-RS will always be maintained.

Based on the RAN1 reply, we suggest using this as a condition for defining when a PL-RS is assumed to be maintained.

The PL-RS is maintained provided UE is not having more than 4 different PL-RS for all activated UL TCI states for PUSCH, PUCCH and SRS transmissions.

TP for PL-RS maintained for Release 17

The definition of ‘PL-RS is maintained’ as discussed and proposed for Rel-16 eMIMO could also be applicable for Rel-17 FeMIMO:

### 8.16.3	MAC-CE based uplink TCI state switch delay

The requirements in this clause shall apply for UL TCI state switch using separate UL TCI state or joint TCI state of unified TCI state switch framework.

In case that source RS in UL TCI state or joint TCI state is associated with a PCI different from that of the serving cell, the requirements in this clause shall apply if the cell with different PCI satisfies the known cell condition defined in 8.16.1. If the known cell condition is not met, longer delay may be expected.

In case of joint TCI state switch, UE is not expected to transmit on UL before UE completes the DL and UL TCI state switch.

For separate UL TCI state switch or joint TCI state switch for PUCCH or PUSCH, or semi-persistent/aperiodic/periodic SRS, when beamCorrespondenceWithoutUL-BeamSweeping is set to 1, upon receiving PDSCH carrying MAC-CE activation command in slot n on serving cell,

-	The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3Nslotsubframe,µ + NM* (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) / NR slot length.

-	If target TCI state is unknown,

-	The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3Nslotsubframe,µ + (TL1-RSRP + Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) / NR slot length.

Where,

-	THARQ (in slot) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

-	NM = 1, if the target PL-RS is not maintained by the UE, 0 otherwise.

[-	PL-RS is maintained provided:

-	the target PL-RS is associated with or included in the UL or joint TCI states in the active TCI list for PUSCH/PUCCH/SRS transmissions

-	There are no more than 4 different RS configured as PL-RS per serving cell among all active UL TCI states (UL TCI state or joint) TCI states for PUSCH/PUCCH/SRS transmissions

-	The target pathloss reference signal remains detectable during TCI state switching period

-	SNR of the target pathloss reference signal≥-3dB

-	The associated SSBs with the target pathloss reference signal remain detectable during the TCI state switching period

-	SNR of the associated SSB ≥-3dB

-	Conditions for known path loss reference signal in section 8.14.2 are fulfilled.Note:	The requirements specified in this clause are not applicable if more than 4 different RSs are configured as PL-RS per serving cell among all active UL (or joint) TCI states.]

-	Tfirst_target-PL-RS is time to first pathloss RS transmission after L1-RSRP measurement when target TCI state is unknown.

-	Tfirst_target-PL-RS is time to first pathloss RS transmission after MAC CE command is decoded by the UE for known TCI State.

-	Ttarget_PL-RS is the periodicity of the target pathloss reference signal which would be SSB or NZP CSI-RS when PL-RS is associated with serving cell

-	Ttarget_PL-RS is the periodicity of the target pathloss reference signal which would be SSB when PL-RS is associated with PCI different from serving cell

-	T L1-RSRP is the time for Rx beam refinement in FR2, defined as

-	TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1,

-	with the assumption of M=1

-	with TReport = 0

-	TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2

-	CSI-RS based L1-RSRP measurement only apply for TCI state switch when source RS is associated with serving cell

-	configured with higher layer parameter repetition set to ON

-	with the assumption of M=1 for periodic CSI-RS

-	for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

-	with TReport = 0

- Editor note: when PL-RS is SSB in FR2, the delay requirement is FFS.

-	The UE shall be able to transmit with the old UL TCI state until slot n+ THARQ + 3Nslotsubframe,µ.

Agree on the TP for PL-RS maintained for Release 17.

Having now discussed the 2 of the 3 issues:

- Whether UE need to track UL time/frequency if source RS in UL TCI state is not in the DL active TCI list (Issue 1-1-3)

- Definition of ‘maintained PL-RS’ (Issue 1-3-1)

Next, we look in more details into the 3rd issue - MAC-CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2 (Issue 1-2-1).

MAC CE based UL TCI state switching delay

Having defined what PL-RS maintained means we next look at another open issue discussed in RAN4, namely whether beam sweeping should be needed for PL-RS measurement in FR2 when the PL-RS signal is SSB.

As mentioned, PL-RS is used for UE to determine the UL transmit power.

In Rel-15 and 16, PL-RS maintenance relies on DL and UL channel reciprocity. If DL reception is triggered using a beam selection (TCI state), the UL beam is selected by the channel reciprocity. In Rel-17, the unified TCI framework also allows for scenarios in which the UL TCI states are switched independently from DL TCI states.

However, the UL TCI state is always configured with a PL-RS (according to RAN1 LS and configuration) and the switch delay could depend on whether the PL-RS associated with the UL TCI state is maintained by the UE or not when the UL TCI state is activated. The DL path loss measurement for an UL TCI need be acquired by the UE to enable the UE to determine the UL transmit power.

As mentioned, in R16 we only considered TCI switch among DL TCI states from serving cell and UL/DL reciprocity was assumed when the requirements were defined for a change in UL spatial relation. Hence, because UE always measure the serving cell PL-RS, the UE would always have a maintained PL-RS.

According to the RAN1 LS, PL-RS is always required to be maintained by the UE for all active UL (or joint) TCI states for at least 4 different activated PL-RS.

RRC configuration allows that the PL-RS can be SSB or CSI-RS. If an SSB is configured as PL-RS and UE is not configured with more than 4 different PL-RS, the UE is, according to RAN1 specifications, required to maintain the PL-RS – including SSB based PL-RS, if SSB is configured as source for PL-RS.

PL-RS source can be SSB or NZP-CSI-RS

A UE with up to 4 activated PL-RS, is according to RAN1 specifications, required to maintain all the PL-RSs.

As the SSB PL-RS is required to be maintained by the UE (assuming UE is having no more than 4 different PL-RS activated) there is no need for beam sweeping for PL-RS measurements in FR2 if the PL-RS is SSB.

There shall be no beam sweeping for PL-RS measurements in FR2 if the PL-RS is SSB (assuming UE is having no more than 4 different PL-RS).

As RAN1 does not discuss UE behavior for the case where UE is having with more than 4 different PL-RS for all active UL (or joint) TCI states, RAN4 should not either discuss requirements for such scenario.

Conclusion

In this contribution we provide our observations and proposals on remaining issues on unified TCI switching requirements. We propose:

Whether UE need to track UL time/frequency if source RS in UL TCI state is not in the DL active TCI list:

- If source DL-RS associated with the indicated UL TCI state is unknown (e.g. it is not in the DL active TCI list), additional time for time/frequency tracking should be allowed for the indicated UL TCI state.

- UE shall track time and frequency of the associated DL RS for an UL TCI state when the UL state is in the active TCI state list.

Check with RAN1 if UE is required to do DL time tracking for an UL TCI state in the active TCI state list.

Definition of ‘maintained PL-RS’

- The PL-RS is maintained provided UE is not having more than 4 different PL-RS for all activated UL TCI states for PUSCH, PUCCH and SRS transmissions.

Agree on the TP for PL-RS maintained for Release 17.

MAC-CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2

There shall be no beam sweeping for PL-RS measurements in FR2 if the PL-RS is SSB (assuming UE is having no more than 4 different PL-RS).

We have provided a CR for capturing the proposed TP in [14]

- 4. References

- R4-2217203, WF on FeMIMO unified TCI state, RAN4 #104-bis-e, Intel

- R4-2214972, LS on active TCI state list for UL TCI, RAN4, #104-e

- R1-2210719, Reply LS on active TCI state list for UL TCI, RAN1 #110-bis-e

- R4-2214481 WF on FeMIMO RRM impact for unified TCI, RAN4 #104-e

- R4-2211203, WF on FeMIMO RRM impact for unified TCI state, RAN4#103-e

- R4-2214481, WF on FeMIMO RRM impact for unified TCI, Intel

- R4-2214482, WF on FeMIMO RRM requirements for inter-cell beam management, Huawei

- R4-2220713, WF on Rel-17 FeMIMO RRM core requirement maintenance, Intel

- R4-2302587, CR for definition of PL-RS maintained, Nokia, Nokia Shanghai Bell

- R4-2302589, CR for definition of PL-RS maintained and time tracking, Nokia, Nokia Shanghai Bell

- R4-2303145, WF on Rel-17 FeMIMO RRM core requirement maintenance, Intel, Samsung

- R4-2308751, Discussion on eMIMO and maintained PL-RS, Nokia, Nokia Shanghai Bell

- R4-2308752, CR for definition of PL-RS maintained in section 8.14, Nokia, Nokia Shanghai Bell

- R4-2308756, CR for definition of PL-RS maintained and time tracking for unified TCI, Nokia, Nokia Shanghai Bell