3GPP TSG-RAN WG4 Meeting # 107 	R4-2308797

Incheon, South Korea, 22nd May – 26th May 2023

Agenda Item:	5.2.10.3

Source: 	Ericsson

Title:	Remaining issues related to PRS-RSRPP accuracy requirement

Document for:	Approval

# Introduction

PRS-RSRPP accuracy requirement was discussed in RAN4#106 and agreed to define accuracy requirement for higher side conditions, i.e., Es/Iot ≥ 0dB. In this contribution we present our view on remaining issues related to PRS-RSRPP accuracy requirement and proposal based on simulation results for 1 sample PRS-RSRPP.

# Discussion

The following two issues need to be addressed before accuracy requirement for PRS-RSRPP can be concluded:

- RF calibration margin applicable to PRS-RSRPP measurement, and

- Accuracy requirement (for Nsample = 4 and Nsample = 1) based on simulation results submitted by companies.

# RF calibration margin applicable to PRS-RSRPP measurement

RF calibration margin needs to be accounted for when specifying the absolute accuracy requirement for PRS-RSRPP measurement for Nsample = 4 and Nsample = 1. In our view RF calibration margin value used for Rel. 16 PRS-RSRP absolute accuracy requirement can be reused for PRS-RSRPP absolute accuracy requirement and same RF calibration margin shall be applied to PRS-RSRPP measurement for Nsample = 4 and Nsample = 1.

Proposal 1: Reuse PRS-RSRP RF calibration margin of 2.5dB to define absolute accuracy requirement for PRS-RSRPP in FR1 for both Nsample = 4 and Nsample = 1.

Proposal 2: Reuse PRS-RSRP RF calibration margin of 4dB to define absolute accuracy requirement for PRS-RSRPP in FR2 for both Nsample = 4 and Nsample = 1.

# Accuracy requirement

As agreed in RAN4#106, simulation results for 1 sample PRS-RSRPP measurement accuracy at higher side condition is presented in table below. Based on the simulation results, we propose values reported in Table 1 is used to define accuracy requirement for 1 sample PRS-RSRPP accuracy in FR1 and FR2 when SINR = 0dB.

Proposal 3: Values proposed in Table 1 is used to define accuracy requirement for 1 sample PRS-RSRPP accuracy in FR1 and FR2 when SINR = 0dB.

Table 1. 1 sample PRS-RSRPP accuracy in FR1 and FR2 for SINR = 0dB.

| FR | SCS (kHz) | PRS BW (PRBs) | 5% [0dB] | 95% [0dB] |
| --- | --- | --- | --- | --- |
| FR1 | 15 | 52 | -0,44 | 0,47 |
|  |  | 104 | -0,35 | 0,35 |
|  | 30 | 48 | -0,37 | 0,38 |
|  |  | 132 | -0,23 | 0,19 |
|  | 60 | 64 | -0,85 | 0,23 |
|  |  | 132 | -0,84 | 0,15 |
| FR2 | 60 | 64 | -0,85 | 0,23 |
|  |  | 132 | -0,84 | 0,15 |
|  | 120 | 64 | -2,44 | 0,09 |
|  |  | 128 | -2,4 | 0,06 |

# Summary

In this paper we presented our views on remaining issues related to PRS-RSRPP accuracy requirement and propose the following to be considered for discussion:

Proposal 1: Reuse PRS-RSRP RF calibration margin of 2.5dB to define absolute accuracy requirement for PRS-RSRPP in FR1 for both Nsample = 4 and Nsample = 1.

Proposal 2: Reuse PRS-RSRP RF calibration margin of 4dB to define absolute accuracy requirement for PRS-RSRPP in FR2 for both Nsample = 4 and Nsample = 1.

Proposal 3: Values proposed in Table 1 is used to define accuracy requirement for 1 sample PRS-RSRPP accuracy in FR1 and FR2 when SINR = 0dB.

Table 1. 1 sample PRS-RSRPP accuracy (without RF calibration margin) in FR1 and FR2 for SINR = 0dB.

| FR | SCS (kHz) | PRS BW (PRBs) | 5% [0dB] | 95% [0dB] |
| --- | --- | --- | --- | --- |
| FR1 | 15 | 52 | -0,44 | 0,47 |
|  |  | 104 | -0,35 | 0,35 |
|  | 30 | 48 | -0,37 | 0,38 |
|  |  | 132 | -0,23 | 0,19 |
|  | 60 | 64 | -0,85 | 0,23 |
|  |  | 132 | -0,84 | 0,15 |
| FR2 | 60 | 64 | -0,85 | 0,23 |
|  |  | 132 | -0,84 | 0,15 |
|  | 120 | 64 | -2,44 | 0,09 |
|  |  | 128 | -2,4 | 0,06 |

# References

- R4-2303194, “Ad-hoc meeting minutes for R16+17 POS and MGE”, Huawei.

- R4-2105750, “WF on NR Positioning Performance Requirements”, Intel.