3GPP TSG RAN WG4 Meeting # 107	R4-2308817

Incheon, Korea, May 22nd  –  May 26th , 2023

Agenda Item:	5.2.10.3

Source: 	Ericsson

Title:	Discussion on Rel-17 MR-DC remaining issues

Document for:	Discussion

# Introduction

From RAN4 #106 meeting Rel-17 maintainence on MR-DC remaining topics listed [1] and certain CR[2] were postponed due to the discussion can not reach agreements.

In this paper we provide our view on certain open issues.

# Discussion

2.1 SCG activation/deactivation related open issues

Issue 5-1-3: Tsearch in RACH-less based PSCell activation delay

- Proposal:

  - For the RACH-less based PSCell activation the condition when Tsearch = 0ms applies while the TCI state is known.

  - For RACH-less based PSCell activation, if bfd-and-RLM are configured with value true and TCI state is known or if the target cell is a known FR2 PScell, Tsearch = 0 ms. if the target cell is a known FR2 PScell. Otherwise, there are no requirements if PSCell is unknown.

Issue 5-1-4: Tsearch in RACH based PSCell activation delay

- Proposal:

  - For RACH based PSCell activation, if the target cell is a known NR FR2 PSCell, Tsearch = 0 ms. If the target cell is an unknown FR2 PSCell configured with bfd-and-RLM with value true and no RLM has occurred, then Tsearch = [12]* Trs ms, otherwise if Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms

Issue 5-1-5: Known condition for PSCell activation

- Proposal:

  - Capture the condition that when bfd_and_RLM with value true is configured and the TCI state is known the PSCell is known

The main discussion within the Tsearch value for RACH-less SCG activation and RACH based activation are lies within 2 aspects

- What are the procedure to trigger the RACH and RACH-less activation

- From UE and Network perspective what are the side conditions needed.

To our understanding the procedure for UE switch from scg-state deactivated to activated only have either RACH based or RACH-less, as according to TS38.331 clause 5.3.5.3 Reception of an RRC Reconfiguration.

Also the SCG activation have two subscenarios, 1st activation without scg-state or activation from scg-state deactivated.

1>	else if the RRCReconfiguration message was received via SRB1 within the nr-SCG within mrdc- SecondaryCellGroup (UE in NR-DC, mrdc-SecondaryCellGroup was received in RRCReconfiguration or RRCResume via SRB1):

2>	if the scg-State is not included in the RRCReconfiguration or RRCResume message containing the RRCReconfiguration message:

3>	perform SCG activation as specified in 5.3.5.13a;

3>	if reconfigurationWithSync was included in spCellConfig in nr-SCG:

4>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

3>	else if the SCG was deactivated before the reception of the NR RRC message containing the RRCReconfiguration message:

4>	if bfd-and-RLM was not configured to true before the reception of the RRCReconfiguration or RRCResume message containing the RRCReconfiguration message; or

4>	if lower layers indicate that a Random Access procedure is needed for SCG activation:

5>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

4>	else the procedure ends;

3>	else the procedure ends;

2>	else

3>	perform SCG deactivation as specified in 5.3.5.13b;

3>	the procedure ends;

NOTE 2a:	The order in which the UE sends the RRCReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation.

The first activation without scg-state will always follow Random Access procedure . The so-called RACH-less activation will only happen when there is a scg-state within the RRC message which indicate it has been deactivated before and the RLM&BFD to be configured is a prerequist.

The main debate during Rel-17 which side conditions to apply during RACH-less activation are due to 3 different SNR values to apply within 3 different side condition.

- SNR requirement: RLM/BFD configured no RLM/BFD declared: -10dB TS 38.133 clause 8.11

- SNR requirement: PSS/SSS detection Ês/Iot ≥ -6dB TS 38.133 clause 10.1

- SNR requirement for legacy feature PScell addition: Ês/Iot ≥ -2dB TS 38.133 clause 8.9.2

Some companies claims that RLM/BFD condition SNR ≥-10dB value is too low to guarantee RACH activation, also for the legacy feature PScell addition, the Ês/Iot ≥ -2dB was introduced to guarantee the radio condition.

The main issue we see here is that what procedure will be followed if target cell become unknown however bfd and RLM is being configured. The unknown condition will only happen when UE has not sent a valid measurement report during the last 5 seconds or the target cell SSB detection failed. From procedure wise UE shall either search for the new detectable SSB which interpret the requirement wise as a fall back to the RACH based procedure.

Proposal 1: Tsearch is the time for AGC settling and PSS/SSS detection. For RACH-less activaiton If the target cell is known and TCI state is known Tsearch = 0 ms for both FR1 and FR2.

Proposal 2: For RACH-less activation if target cell become unknown, the RACH based activation requirement will be followed.

For TCI unknown case, as the TCI states known is defined there is no beam failure being declared, our interpretation of TCI state unknonw is there is beam failure being declared here say the SNR is even worse than -10dB, our interpretation this will follow the SCG failure procedure instead of RACH based activation as we don’t think this high SNR value can be improved by go through a random access procedure.

Proposal 3: If Beam failure has been declared or TCI become unknown during SCG activation procedure, the SCG failure procedure from TS 38.331 clause 5.7.3 shall be followed.

Issue 5-3-1: Applicability of RACH-less SCG activation delay requirements

- Proposals

  - Option 1: The RACH-less SCG activation delay requirements are applicable under the condition that PL-RS of PSCell to use is identical to the associated SSB or CSI-RS of the TCI states for PDCCH/PDSCH reception in PSCell

  - Option 2:

The PL-RS discussion also has been brought up for several rounds without consensus. Certain aspect needs to be considered to evalue whether PL-RS will have impact on RACH-less SCG activation delay.

For RACH-less based SCG activation, UE would transmit SR on PUCCH or PUSCH on PSCell after PSCell activation. To determine the transmission power, UE needs to maintain PL-RS to estimate downlink pathloss.

Firstly current requirement specifies the RACH-less SCG activation with already taken in consideration of uncertainty delay. Based on TS38.133 clause 8.17.2 defined an extra delay component TIU to guarantee the 1st transmission of PUSCH or SR on PUCCH.

TIU : When RACH-less based PSCell activation is configured, it is the uncertainty in acquiring the first PUSCH transmission occasion [or SR on PUCCH].

This uncertainty is different from waiting for the RACH occation from the network, it is totally cover the uncertainty UE may have in acquiring the first transmission. UE fail to maintain or obtain the PL-RS due to any type of reason shall fall into this category of uncertainty.

Secondly, regarding whether there is uncertainty for UE need extra time to find the PL-RS to measure (for example, not identical to SSB or mapped with unknown NZP-CSI-RS). Existing specification TS38.133 clause 8.14 have defined the PL-RS known condition for PL-RS switching delay and shall apply old PL-RS during this activation of new PL-RS period without explicitly require UE to maintain PL-RS measurement.

This indicates UE will also have a PL-RS to measure, regardless of old or new, correct or wrong. As current specification does not test UL power transmission accuracy especially not for the 1st transmission, we fail to understand the reason why complicate this issue.

Also UE apply RACH-less activation delay were based on certain pre-conditions for example UE maintain the L3 measurement during deactivated SCG and maintaining sync, uplink 1st transmission power shall be adjusted during this procedure unless UE performe mobility procedure which will trigger new random access procedure not following the RACH-less activation in this case.

Proposal 4: RACH-less activation delay shall maintain the previous agreement as PL-RS monitoring is not required by specification and uncertainty of any 1st UL transmission is already covered in the requirement.

# Summary and Conclusion

In this contribution we have provided our views on study of improvement on FR2 Scell and SCG setup/resume. The following proposals are made:

Proposal 1: Tsearch is the time for AGC settling and PSS/SSS detection. For RACH-less activaiton If the target cell is known and TCI state is known Tsearch = 0 ms for both FR1 and FR2.

Proposal 2: For RACH-less activation if target cell become unknown, the RACH based activation requirement will be followed.

Proposal 3: If Beam failure has been declared or TCI become unknown during SCG activation procedure, the SCG failure procedure shall be followed.

Proposal 4: RACH-less activation delay shall maintain the previous agreement as PL-RS monitoring is not required by specification and uncertainty of any 1st UL transmission is already covered in the requirement.

# References

- R4-2302761 “Topic summary for [106][203] R17_maintenance_RRM”, Intel

- R4-2302596 “CR on Efficient activation/de-activation mechanism for one SCG”, Nokia