3GPP TSG-RAN WG4 Meeting #107	R4-2308818

Incheon, Korea, May 22nd  –  May 26th , 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3319 | rev | 1 | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on SCG activation/deactivation RRM requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh3-Core |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Rel-17 enhancements on FR1-FR1 NR-DC RRM requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editiory and Tsearch for RACH and RACH-less activaiton |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-17 The agreement will not be captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------NEXT CHANGE----------------------------

## 8.17	SCG Activation and Deactivation Delay

### 8.17.1	Introduction

This clause defines requirements for the delay within which the UE shall be able to activate one SCG and deactivate on SCG.

The requirements shall apply for NR-DC with an NR PCell, PSCell or SCell.

### 8.17.2	SCG Activation Delay Requirement

The requirements in this clause shall apply for the UE configured with one deactivated SCG in NR-DC and when PScell in one SCG is being activated.

The delay within which the UE shall be able to activate the deactivated SCG depends upon the specified conditions.

Upon receiving SCG activation command in slot n, the UE shall be capable to transmit PRACH preamble or PUCCH or PUSCH towards PSCell no later than in slot n+Tactivation_timeNR slot length ,

where:

Tactivation_time = TRRC_delay + Tprocessing + Tsearch + T∆ + TIU + 2 ms

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2].

Tprocessing is the SW processing time needed by UE, including RF warm up period. When PSCell is activated from deactivated state, if any PSCell parameter is modified, Tprocessing = 20ms. Otherwise, Tprocessing = 5 ms.

Tsearch is the time for AGC settling and PSS/SSS detection.

For RACH based PSCell activation, if the target cell is a known NR FR2 PSCell, Tsearch = 0 ms. If the target cell is an unknown FR2 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms.

For RACH-less based PSCell activation, if TCI state is known, Tsearch = 0 ms if the target cell is a known FR2 PScell. The RACH based activation will be followed if target cell become unknown. SCG failure procedure from TS38.331 clause 5.7.3 will be follow if TCI state is unknown..

T∆ is time for fine time tracking and acquiring full timing information of the target PSCell. T∆ = 1*Trs ms.

TIU: When RACH based PSCell activation is configured, it is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TIU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3].

When RACH-less based PSCell activation is configured, it is the uncertainty in acquiring the first PUSCH transmission occasion or SR on PUCCH.

Trs is the SMTC periodicity of the PSCell if the UE has been provided with an SMTC configuration for the target cell in SCG activation command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5.

In FR2, the PSCell is known if it has been meeting the following conditions:

-	During the last 5 seconds before the reception of the SCG activation command:

-	the UE has sent a valid measurement report for the PSCell being activated and

-	One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3.

-	One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tactivation_time according to the cell identification conditions specified in clause 9.3.

otherwise it is unknown.

If the UE is configured to perform BFD while the SCG is deactivated

-	The TCI state is known if the following conditions are met:

-	During the period from the PSCell deactivation to the completion of PSCell activation, while PSCell was deactivated,

-	UE has not detected beam failure

-	Otherwise, the TCI state is unknown.

The PCell interruption specified in clause 8.2 is allowed only during the RRC reconfiguration procedure [2].

### 8.17.3	SCG Deactivation Delay Requirement

The requirements in this clause shall apply for a UE which is configured with at least PCell and PScell.

Upon receiving RRC-based SCG deactivation command in subframe n, the UE shall accomplish the deactivation actions specified in TS 38.331 [2] no later than in slot n+TRRC_delayNR slot length:

where

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2].

The PCell interruption specified in clause 8.2 is allowed only during the RRC reconfiguration procedure [2].

--------------------END OF CHANGES--------------------------