3GPP TSG RAN WG4 Meeting #107	R4-2308821

Incheon, Korea, May 22nd  –  May 26th , 2023

Agenda Item:	8.25.3

Source: 	Ericsson

Title:	NR-DC with selective activation of cell groups via L3 enhancements

Document for:	Discussion

# Introduction

From RAN4 #106 bis e-meeting Rel-18 NR-DC with selective activations via L3 enhancements are listed in [1]. In this paper we provide our view on certain open issues.

Background: WID objective #2 for information:

2. To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements:

- To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4]

Note 4: A harmonized RRC modelling approach for objectives 1 and 2 could be considered to minimize the workload in RAN2.

# Discussion

From RAN4#106 bis-emeeting, several agreements were reached. In this paper we provide our view on the left open issues.

Issue 1-6: whether to consider UE is not evaluating the execution condition of other candidate PSCells while executing CPC

- Candidate solutions:

  - Option 1: Yes. Event_DU for the subsequent change can be set to zero, if the execution condition for another PSCell is fulfilled while the UE is completing the PSCell change.

  - Option 2: No. According to TS37.340 and RAN2 agreements, UE is not evaluating the execution condition of other candidate PSCells while executing CPC, and the evaluation is continued after finishing the PSCell addition or change.

To our understanding there could be multiple candidate PScells fulfill the condition however the execution of the PScell change will be in a sequential manner. As the Event DU defined as the undertainty time from when UE transmits SN RRCReconfigurationcomplete message for the previous PSCell addition or change until a condition exists at the measurement reference point which will trigger the subsequent conditional PSCell change.

The TEvent_DU from the 1st CPC is the time from UE have decode the 1st RRC message with measurement configuration, UE starts to detect the SSB from the candidate target cells have multiple candidates SSB being measured for example SSB power level RSRP etc. Then the event will be triggered if this type of threshold is being fulfilled for certain period of the time.

There could be 2 scenarios for the 1st activation of CPC, only 1 candidate fulfills condition or more than one candidate fulfill the condition.

The main differences between subsequent CPC and the 1st activation of CPC is that UE was being indicated by the network to keep the previous configuration from the 1st CPC activation. In another words, the subsequent CPC is one of the rest being fulfilled candidates at the 1st CPC acitvation. In this case, there is no evaluation needed.

However so far there is no agreement on whether UE shall not evaluating other candidates while executing the current CPC, as the candidates being configured could be non-collocated or far away from being detected, we see the reason to keep the Event_DU for evaluation and uncertainty.

Proposal 1: Event_DU can be set to zero, however to cover the full range of scenarios, we proposa to keep the Event_DU.

# Summary and Conclusion

In this contribution we have provided our views on NR-DC with selective activations via L3 enhancements. The following proposals are made:

Proposal 1: Event_DU can be set to zero, however to cover the full range of scenarios, we proposa to keep the Event_DU.

# References

- R4-2306356 “WF for NR mobility enhancements RRM requirements”, Apple