3GPP TSG-RAN WG4 Meeting #107              	R4-2308831

Incheon, South Korea, 22th May-26th May, 2023

Agenda item:	8.13.5.3

Source:	Samsung

Title:	View on PDSCH requirements with multi-Rx reception

Document for:	Discussion

- Introduction

In RAN#97-e meeting, the updated WID on enhanced NR support for high-speed train scenario in frequency range 2 (FR2) was approved in [1]. In the last RAN4 meeting, the deployment, test scenario, channel model and test scope for demodulation requirements were under discussion. The related agreements were captured in the WF [2].  Regarding the deployment scenario and test scope of PDSCH requirement for multi-Rx reception are made as

- Deployment and Channel Model for Demodulation requirement with simultaneous Rx reception in open space scenario - Deployment Scenario for FR2 HST simultaneous multi-Rx reception requirements for Open Space Scenario - RAN4 consider the Bi-directional deployment scenario for PDSCH demodulation requirements with simultaneous multi-Rx reception - Consider the scenario the timing different between TRPs is exceeding CP - RAN4 will evaluate the performance impact for this scenario with PC6 UE and specify corresponding requirements with receiver implementation agonistic approach. - RAN4 will discuss the test applicable rules if needed. - Consider both scenario A and scenario B as starting point for performance evaluation - FFS whether single requirement or separate requirements need to be introduced for scenario A and scenario B in Bi-directional deployment scenario for PDSCH demodulation requirements - Aligned with RF session conclusion, not consider uni-directional scenario in Rel-18 FR2 HST WI performance requirements. - RRH locations parameters - Reuse the current RRH location parameters’ values of FR2 HST-DPS performance requirements evaluation for DL demodulation performance study of FR2 HST simultaneous multi-panel reception as a starting point for evaluation - Beam serving coverage - Reuse the beam serving range of uni-directional FR2 HST-DPS performance evaluation as the left-beam serving range of an RRH in bi-directional deployment, and right-beam serving range is symmetrical to the left-beam serving range about the RRH as a starting point for evaluation - Further study of coverage - General assumption for CPE with multi-panel reception Rx processing for FR2 HST - Assuming UE processing with 2x2 per TRP as baseline - General assumption for channel modelling and interference - No interference in between CPE panels assumption is baseline - The baseline assumption CPE panels are co-located i.e., distance between two CPE panels is 0 (m). - Companies are encouraged to further evaluate the performance impact with non-co-located - No limitation on CPE implementation - Channel Model for FR2 HST simultaneous multi-panel reception performance evaluation - Assuming CPE processing with 2x2 per TRP per panel separately - Option 1: Reuse the existing FR2 HST DPS channel model to perform FR2 HST simultaneous multi-panel reception performance evaluation as a starting point for evaluation - FFS on the modelling the propagation delay as a part of channel model - FFS on how to extend the doppler shift model to simultaneous multi-panel reception - Option 2: Reuse HST-SFN scheme A to study the UE receive signals from two RRHs simultaneously - multi-Rx simultaneous reception requirement with single carrier - General for requirement - For FR2 HST requirements with simultaneous reception (2 panels), RAN4 could consider reusing applicable agreements reached in the UE Demodulation work item for FR2 multi-RX as starting point to avoid overlap - Test scope of multi-Rx simultaneous requirement - RAN4 only define PDSCH demodulation performance requirements for Rel-18 HST FR2 UEs capable of simultaneous two-panel reception. - Whether to introduce a new correlation matrix for HST FR2-1 in a Multi-TRP and Multi-Rx context for OTA demodulation performance requirement - Assuming no inter-TRP and inter-panels interference as baseline for Rel-18 FR2 HST WI. No new correlation matrix for FR2-1 HST is introduced - UE processing assumption for the FFT window - Number of FFT window: RAN4 need to further evaluate performance with below candidate options - Option 1: single FFT window for FR2 HST demodulation requirements definition for two panels - Option 2: independent FFT window for each panel for the scenario the reception time difference between different TRPs is larger than one CP. - Transmission scheme - Prioritize mDCI with full-overlapping resources based mTRP scheme for DL demodulation performance study of FR2 HST simultaneous multi-panel reception - FFS on sDCI

In this contribution, the view on deployment scenario and test setup for PDSCH requirement with multi-Rx reception are provided.

# 2	Discussion

## 2.1	Deployment for PDSCH requirement with multi-Rx reception

For Rel-18 FR2 HST WI, the target is to enable the multi-Rx reception for UE capable of tow panel. The RRH locations parameters should be same as the Rel-17 FR2 HST deployment scenario with UE single panel reception.  According to RF discussion and agreement in the last meeting, RAN4 consider the Bi-directional deployment scenario for PDSCH demodulation requirements with simultaneous multi-Rx reception, as illustrated in the following figure.

Figure 1: Bi-directional deployment for UE with multi-panel reception

Aa shown in Figure 1, the UE with multi-panel will be served with two beams from two adjacent RRHs. Since it is agreed that UE with two panels can process the signal separately and no inter-TRP and inter-panels interference modeling.  The UE will connect to the 2nd nearest RRH, similar as Rel-17 FR2 HST, which can solve the coverage hole issue, the region around RRH with 2*D_offset. The D_offset is used to indicate the location of the beams switching points with different RRHs, similar as FR2 HST-DPS in Uni-directional deployment scenario. Before or after the switching points, the right panel of UE is serving from the left panel of nearest RRH deployed in the right side and the left panel of UE is serving from the right panel of nearest RRH deployed in the left side.

Beam serving coverage

Regarding the beam serving coverage, RAN4 has agreed to reuse the beam serving range of uni-directional FR2 HST-DPS performance evaluation as the left-beam serving range of an RRH in bi-directional deployment, and right-beam serving range is symmetrical to the left-beam serving range about the RRH as a starting point. Meanwhile, based on RF core requirement, the same one-directional spherical coverage in both forward and backward directions for bi-directional scenarios.

Then, the same value of Ds_offset can be used from Un-directional scenario for Bi-directional scenario to determine beam serving coverage for each beam for different panels deployed in RRH.

Since both scenario A and scenario B are considered in Bi-directional deployment scenarios, then, the following value of Ds_offset should be considered

| Scenario | Ds (meter) | Dmin (meter) | Ds_offset (meter) |
| --- | --- | --- | --- |
| A | 700 | 10 | 10 |
| B | 700 | 150 | 100 |

The related beam serving coverage for left panel and right panel for each RRH can be considered as

- Left panel: [(k-1) *Ds- Ds_offset, k*Ds- Ds_offset] for the kth RRH, k=0, 1,…,

- Right panel: [k *Ds+ Ds_offset, (k+1)*Ds+ Ds_offset] for the kth RRH, k=0,1,…

Proposal 1: The following value of Ds_offset can be considered to determine the beam serving coverage for each RRH in Bi-directional scenario, for scenario A and B, separately

| Scenario | Ds (meter) | Dmin (meter) | Ds_offset (meter) |
| --- | --- | --- | --- |
| A | 700 | 10 | 10 |
| B | 700 | 150 | 100 |

The related beam serving coverage for left panel and right panel for each RRH can be considered as

- Left panel: [(k-1) *Ds- Ds_offset, k*Ds- Ds_offset] for the kth RRH, k=0, 1, …

- Right panel: [k *Ds+ Ds_offset, (k+1) *Ds+ Ds_offset] for the kth RRH, k=0,1, …

## 2.2	Channel model for PDSCH requirement with multi-Rx reception

Regarding HST-SFN scheme A, it was introduced in Rel-17 FeMIMO, with considering two visible RRH. The model defined is mainly targeting for FR1. Meanwhile, the relative delay and power for each tap, excepting for doppler, are involved into channel model, since the PDSCH detection from joint transmission from each RRH is required.

Compared with HST-SFN scheme A, since assuming CPE processing with 2x2 per TRP per panel separately, and no inter-TRP and inter-panels interference modeling. Then, the relative delay and power for tap from each RRH is not needed

Proposal 2: No need to model the relative propagation delay and power for tap from the visible RRH

Therefore, the existing FR2 HST DPS channel model defined in Uni-directional scenario should be feasible fo channel modeling for each panel in the Bi-directional scenario.

In Rel-17 FR2 HST, the DPS channel model in Uni-directional scenario with single panel reception was introduced as

cosθt=Dsoffset +Ds-vtDmin2+Dsoffset+Ds-vt2, 0<t≤Dsv cosθt=cosθt modDsv, t>Ds/v

Based on the beam serving coverage of left panel and right panel of each RRH as mentioned in the section 2.1, the related channel can be considered for Bi-directional deployment scenario

- Doppler shift fst=fd cosθt (Hz) from the left panel of RRH for PDSCH received at right panel of UE is given by cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2, 0<t≤Dsv cosθt=cosθt modDsv, t>Ds/v - Doppler shift fst=fd cosθt (Hz) from the right panel of RRH for PDSCH received at left panel of UE is given by cosθt=-(Ds-Dsoffset +vt)Dmin2+Ds-Dsoffset +vt2, 0<t≤2*Ds_offsetv cosθt=-(Dsoffset +vt)Dmin2+Dsoffset +vt2, 2*Ds_offsetv<t≤Dsv cosθt=cosθt modDsv, t>Ds/v

Proposal 3: The following channel model can be considered for PDSCH requirement with multi-Rx reception in Bi-directional deployment scenario.

- Doppler shift fst=fd cosθt (Hz) from the left panel of RRH for PDSCH received at right panel of UE is given by cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2, 0<t≤Dsv cosθt=cosθt modDsv, t>Ds/v - Doppler shift fst=fd cosθt (Hz) from the right panel of RRH for PDSCH received at left panel of UE is given by cosθt=-(Ds-Dsoffset +vt)Dmin2+Ds-Dsoffset +vt2, 0<t≤2*Ds_offsetv cosθt=-(Dsoffset +vt)Dmin2+Dsoffset +vt2, 2*Ds_offsetv<t≤Dsv cosθt=cosθt modDsv, t>Ds/v

## 2.3	Test setup for PDSCH requirement with multi-Rx reception

Number of FFT

According to agreement made in FR2-multi-Rx WI, no joint preprocessing/ decoder is considered for mDCI for RF requirement. which means that each panel can receiver and proceed the signal transmission by each RRH independently, Therefore, single FFT operation for each panel should be benefit for HST scenario

Meanwhile, based on the RRM core requirement, Rel-18 FR2 PC6 UE should support simultaneous data reception from two panels with MRTD more than the CP length. Given the assumption that each panel can process the signal separately, one FFT is available for each panel, then, there should be no issue for MRTD is larger than the CP length

Therefore, we think there is no limitation of number of FFT window. Independent FFT window for each panel for the scenario the reception time difference between different TRPs is larger than one CP.

Proposal 4: Independent FFT window for each panel for the scenario the reception time difference between different TRPs is larger than one CP.

Transmission scheme

With receiver assumption of CPE can process the signal independently per each panel, then mDCI based transmission scheme should be more feasible for Rel-18 FR2 HST transmission scheme.

For sDCI based transmission scheme, the different layers transmitted by each RRH belongs to one CW, which means the information before decoder for each panel should be combined into one decoder, even with assumption that UE will process 2x2 per panel separately. Although the issue of MRTD larger than the CP length can be solved by each panel with separated FFT window, large power imbalance of each layer transmitted by each RRH are still existing. Therefore, we think the gain of sDCI based transmission scheme is limited compared with mDCI based transmission scheme.

Layer combination

Since the assumption is UE can process with 2x2 per RRH separately, and no inter-TRP/inter-CPE interference modeling, then, the channel model for each panel can be regarded as Uni-directional. 2+2-layer combination should be no issue.

Proposal 5: Consider 2+2-layer combination with full-overlapping scheduling based on mDCI transmission scheme for requirement definition of multi-Rx reception

# 3	Conclusion

In this contribution, the view on the test scenario, channel modelling and test setup for FR2 HST PDSCH requirement with multi-Rx simultaneous reception in Bi-directional deployment for open space scenario was provided

Proposal 1: The following value of Ds_offset can be considered to determine the beam serving coverage for each RRH in Bi-directional scenario, for scenario A and B, separately

| Scenario | Ds (meter) | Dmin (meter) | Ds_offset (meter) |
| --- | --- | --- | --- |
| A | 700 | 10 | 10 |
| B | 700 | 150 | 100 |

The related beam serving coverage for left panel and right panel for each RRH can be considered as

- Left panel: [(k-1) *Ds- Ds_offset, k*Ds- Ds_offset] for the kth RRH, k=0, 1, …

- Right panel: [k *Ds+ Ds_offset, (k+1) *Ds+ Ds_offset] for the kth RRH, k=0,1, …

Proposal 2: No need to model the relative propagation delay and power for tap from the visible RRH

Proposal 3: The following channel model can be considered for PDSCH requirement with multi-Rx reception in Bi-directional deployment scenario.

- Doppler shift fst=fd cosθt (Hz) from the left panel of RRH for PDSCH received at right panel of UE is given by cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2, 0<t≤Dsv cosθt=cosθt modDsv, t>Ds/v - Doppler shift fst=fd cosθt (Hz) from the right panel of RRH for PDSCH received at left panel of UE is given by cosθt=-(Ds-Dsoffset +vt)Dmin2+Ds-Dsoffset +vt2, 0<t≤2*Ds_offsetv cosθt=-(Dsoffset +vt)Dmin2+Dsoffset +vt2, 2*Ds_offsetv<t≤Dsv cosθt=cosθt modDsv, t>Ds/v

Proposal 4: Independent FFT window for each panel for the scenario the reception time difference between different TRPs is larger than one CP.

Proposal 5: Consider 2+2-layer combination with full-overlapping scheduling based on mDCI transmission scheme for requirement definition of multi-Rx reception

# References

RP- 222272, Revised WID on enhanced NR support for high-speed train scenario in frequency range 2 (FR2)

R4-2305892	WF on demodulation requirements for Rel-18 FR2 HST		Samsung