3GPP TSG-RAN WG4 Meeting #107              	R4-2308833

Incheon, South Korea, 17th April-26th April, 2023

Agenda item:	8.13.5.1

Source:	Samsung

Title:	View on channel modelling for Rel-18 FR2 HST demodulation requirement in tunnel scenario

Document for:	Discussion

- Introduction

In RAN#97-e meeting, the updated WID on enhanced NR support for high-speed train scenario in frequency range 2 (FR2) was approved with the following objective as [1]

- Core requirement - Only train roof-mounted high-power devices with target applicable carrier frequency up to 30GHz and up to 350km/h velocity are considered in this WI - Specify the RF requirements for intra-band carrier aggregation (CA) scenario, and investigate and specify the RRM requirements for intra-band carrier aggregation (CA) scenario [RAN4] - Specify the requirement for simultaneous multi-panel operation for train roof-mounted FR2 high power devices [RAN4]: - Maximum 2 active panels supporting the multi-panel simultaneous reception. - NOTE: Focus on FR2 HST specific requirements, and avoid the overlap with the scope of FR2 multi-Rx DL reception - Study on reference tunnel deployment scenario for FR2 HST and specify the channel model and corresponding core requirements if any [RAN4] - Specify UL timing adjustment solution, including explicit NW signaling assistance, for FR2 HST scenario with large UL/DL propagation delay difference from different RRHs/TRPs to UE [RAN4, RAN2]. - Note: RAN1/RAN2 work can be triggered by RAN4 LS. - Performance requirement - Specify the necessary RRM test cases based on the outcome on corresponding core part. - Investigate and if needed specify the demodulation performance requirements for intra-band carrier aggregation (CA) HST scenario - Specify the necessary demodulation performance requirements for simultaneous multi-panel reception. - NOTE: Focus on FR2 HST specific requirements, and avoid the overlap with the scope of FR2 multi-Rx DL reception - Specify the other necessary RRM and demodulation performance requirements depending on the outcome of core part.

In the last RAN4 meeting, the deployment, test scenario, channel model and test scope for demodulation requirements were under discussion. The related agreements were captured in the WF [1]. Regarding the deployment for demodulation requirements in Tunnel scenario, the following agreement are made

- Deployment for Demodulation requirements in Tunnel scenario - Only consider when CPE is fully inside tunnel as baseline - Option 1 - Both uni- and bi-directional deployment scenarios are considered for FR2 HST-DPS performance requirement evaluation in tunnel channel. For uni-directional deployment, FR2 HST UE moves in the same direction as the RRH serving beam direction - Option 2 - Use Uni-directional deployment for tunnel scenario for HST FR2 - Option 3 - RAN4 to agree additionally on the assumption for bi-directional deployment in the tunnel if simultaneous multi-panel reception is considered - Option 4 - FFS to define requirement based on evaluation of channel model for tunnel scenario

Regarding the channel model for tunnel scenario, the following agreement are made as

- Deployment for Demodulation requirements in Tunnel scenario - Channel Model for demodulation requirement in Tunnel scenario - For UE demod - Reuse the channel model in RAN4 spec 38.101-4, i.e, single path with LoS propagation, for performance requirement study of FR2 HST-DPS in tunnel deployment as baseline - FFS on new channel model is needed for Bi-directional deployment in tunnel scenario - FFS on new channel model (multi-tap) is needed pending on UE location with faraway or close around RRH - FFS on consider multipath fading channel for demodulation performance study - For BS demod: FFS with below candidate options - FFS on consider multipath fading channel for demod (geometry-Ds and Dmin and speed) scenario 3 (tunnel space) in RAN4 spec 38.104 - Reuse FR2 Rel-17 38.104 single path with LoS propagation channel model with - Multipath fading channel

In this contribution, the view on channel modeling for demodulation requirement for FR2 HST scenario in tunnel scenario is provided.

# 2	Discussion

## 2.1	Deployment for demodulation requirements in Tunnel scenario

During the Rel-17 work item, most of focus was devoted into open space deployment scenarios (Scenario-A and B, differentiated by Dmin, which is the perpendicular distance between RRH site and railway track), while limited study was provided on the tunnel scenario due to the limited Rel-17 effort and relative prioritization, which is identified by operators as important high speed train scenario.

Based on the WID, to study on reference tunnel deployment scenario for FR2 HST is expected to be included in this WI. Based on previous meetings, the following agreement was achieved

- HST Tunnel deployment scenario - Scenario#1: single-panel reception UE and DPS transmission scheme - Sceanrio#2: Two-panel simultaneous reception scenario and analysis of corresponding transmission schemes in tunnel deployment has lower priority - FFS: feasibility of uni- and bi-directional RRH deployments in Sceanario#1 (single-panel reception UE and DPS transition schemes) - FFS: deployment assumption at the exit/entrance of the tunnel - Key parameters for tunnel deployment - Ds: the distance separation between two neighboring RRH sites - Ds =700ms - Dmin: the minimum distance between RRH site and train track: - Dmin = 1m

- HST Tunnel deployment scenario - Consider both uni-directional and Bi-directional deployment in the tunnel for Scenario #1 (single-panel reception, DPS transmission scheme) - FFS additional assumptions on tunnel deployment - Option 1: In uni-directional scenarios assume the at least one open-space RRH is deployed close to railway, e.g., with tunnel deployment parameters, and orientations of RRH panels are the same in open-space and in the tunnel. - Option 2: No need to taking exit/entrance of the tunnel into account of tunnel scenario and channel

Meanwhile, based on agreement in the previous meeting in the RRM session for Channel Model

- Channel Model for tunnel deployment - Only considering LoS propagation conditions - FFS: whether RRM session to decide propagation conditions which can be used for demodulation performance requirements - Option 1: Reuse FR2 HST channel model in TS38.101-4 and TS 38.104 - Option 2: Consider multi-path fading model (e.g, with up to 2nd order multi-path components) - Other options are not precluded

- Channel Model inside the tunnel - Continue the discussion of the channel modes in the Demod track - Dmin and Ds agreed in the RRM session can be used for reference [R4-2220396] - FFS whether channel characteristics in tunnel scenario can be assumed to comparable to open space scenario

Based on feasibility study of RRM session, it concludes both Uni-directional and bi-directional deployments in tunnel for scenario#1 (single-panel reception UE and DPS transmission scheme) are feasible, meanwhile two-panel simultaneous reception scenario and analysis of corresponding transmission schemes in tunnel deployment has lower priority.

In Rel-17 FR2 open space, both Uni-directional and bi-directional are considered for demodulation requirement. For deployment for demodulation requirement in tunnel scenario, we think both deployment scenario should be considered at least for single-panel reception with DPS transmission schemes. For two-panel simultaneous reception scenario in tunnel scenario, since it has lower priority in RRM core requirement, then, it can be deprioritized for demodulation requirement.

Regarding the UE moving direction and several beam directions in uni-directional, similar as FR2 open space, it can be assumed that FR2 HST UE moves in the same direction as the RRH serving beam direction, since there is no different from demodulation requirement, only different is the sign of doppler experienced at the UE side. Although the UL Tx timing switching is different for the same direction as the RRH serving beam direction or the opposite direction as the RRH serving beam direction, but it will not impact on the demodulation requirement. Therefore, we propose to focus on both Uni-directional and Bi-directional scenario for demodulation requirement in tunnel scenario. For Uni-directional deployment, FR2 HST UE moves in the same direction as the RRH serving beam direction as baseline, and deprioritized for demodulation requirement for two-panel simultaneous reception in Bi-directional scenario.

Proposal 1: RAN4 considers both Uni-directional and Bi-directional deployment for demodulation requirement for FR2 HST-DPS in tunnel scenario. For Uni-directional deployment, FR2 HST UE moves in the same direction as the RRH serving beam direction

Proposal 2: Deprioritized the demodulation requirement for two-panel simultaneous reception in Bi-directional scenario in the tunnel scenario.

## 2.2	Channel Model for UE demodulation in tunnel scenario

As agreed in RRM session, only LoS propagation conditions is considered for channel model in tunnel scenario. It is also agreed to apply the channel model in RAN4 spec 38.101-4, i.e, single path with LoS propagation, for performance requirement study of FR2 HST-DPS in tunnel deployment as baseline. Meanwhile, Dmin and Ds agreed in the RRM session can be used for reference for channel modes discussion in the Demod session.Therefore, the channel model for tunnel scenario can be referred in open space scenario in scenario-A and scenario B for single-panel reception with DPS transmission schemes as following

Uni-directional scenario

cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2,  0<t≤Dsv

cosθt=cosθt modDsv,  t>Ds/v

| Parameter | Value |
| --- | --- |
|  | TBD |
| Ds | 700 m |
| Ds_offset | [5m] |
| Dmin | 1 m |
| v | 350 km/h |
| fd | 9722 Hz |

Bi-directional scenario

For the Bi-directional, the UE is receiving PDSCH from the 2nd-nearst RRH, the switching point between different RRH is the middle of distance between two RRHs. Therefore, the similar channel can be reused with updated Dmin

cosθt=Ds-vtDmin2+Ds-vt2,  0<t≤(0.5*Ds)/v

cosθt=-vtDmin2+vt2,  (0.5*Ds)/v<t≤Ds/v 
cosθt modDsv,    t>Ds/v

| Parameter | Value |
| --- | --- |
|  | TBD |
|  | 700 m |
|  | 1 m |
|  | 350 km/h |
|  | 9722 Hz |

Based on our company paper study, if the analog beamforming is applied, the observed channel will be more “single-tap-like”. Although it is possible that multi-path component existing in the fading channel, while we think the RMS delay spread is very small. Meanwhile, there is no clear modeling for the 2nd -tap with including the delay and power. Therefore, we do not think it is necessary to consider multi-path fading for requirement.

As for the new channel for UE with different location, from baseband processing aspect, there is no different foreseen. Meanwhile, how can differentiate the UE processing with different channel behavior. Therefore, we don’t think it is necessary to introduce new channel model (multi-tap) when UE is in close proximity of RRH. So, we prefer to apply the same channel model for requirement in tunnel scenario when UE is in side of tunnel, no differentiate the channel pending on the UE location.

Proposal 3: No new channel model (multi-tap) is needed pending on UE location with faraway or closed

Proposal 4: Not consider multipath fading channel for demodulation performance

Propose 5: Reuse the channel model in RAN4 spec 38.101-4, i.e, single path with LoS propagation, for performance requirement study of FR2 HST-DPS in tunnel deployment with updated Ds, Dmin and Ds_offset.

Uni-directional scenario

cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2,  0<t≤Dsv

cosθt=cosθt modDsv,  t>Ds/v

| Parameter | Value |
| --- | --- |
|  | TBD |
| Ds | 700 m |
| Ds_offset | [5m] |
| Dmin | 1 m |
| v | 350 km/h |
| fd | 9722 Hz |

Bi-directional scenario

cosθt=Ds-vtDmin2+Ds-vt2,  0<t≤(0.5*Ds)/v

cosθt=-vtDmin2+vt2,  (0.5*Ds)/v<t≤Ds/v 
cosθt modDsv,    t>Ds/v

| Parameter | Value |
| --- | --- |
|  | TBD |
|  | 700 m |
|  | 1 m |
|  | 350 km/h |
|  | 9722 Hz |

Ds_offset

Regarding the Ds_offset in Uni-directional scenario, with small value of Dmin, it is expected to have small value of Ds_offset compared in the open space scenario. Therefore, we are open to discuss the value Ds_offset, the value of Ds_offset as 5m proposed by companies can be accepted for us.

Proposal 6: The value of Ds_offset as [5]m can be considered for channel model of Uni-directional scenario in Tunnel scenario.

## 2.3	Channel Model for BS demodulation in tunnel scenario

Different with downlink, regardless the deployment or transmission scheme, only single tap is available for each RRH, since there is no multiple RRH combination assumption.

In FR1 HST, both open space and tunnel space channel model were introduced with single tap channel as

, , ,

With different Dmin as 150ms for open space, and 2 ms for tunnel scenario.

During Rel-17 FR2 discussion, different channel models with covering Uni-directional and Bi-directional scenario were agreed,

- Channel Model for Uni-directional RRH deployment cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2, 0<t≤Dsv cosθt=cosθt modDsv, t>Ds/v where 0 ≤Ds_offset<Ds Scenario; Cell radius (m); Ds_offset. Scenario A (Ds =700m, Dmin =10m); 700; 10m. Scenario B (Ds=700m, Dmin=150m); 716; 100m. - Channel Model for Bi-directional RRH deployment cosθt=Ds-vtDmin2+Ds-vt2, 0<t≤(0.5*Ds)/v cosθt=-vtDmin2+vt2, (0.5*Ds)/v<t≤Ds/v cosθt modDsv, t>Ds/v

For performance requirement definition, as agreed, only one set of requirements was defined based on Bi-directional scenario-B model.

For tunnel scenario in FR2, we think the similar channel model can be used with updated the Dmin to align with the deployment scenario, since it will impact on the doppler experienced by gNB and also the switching point of served RRH, although the performance between different Dmin is minor.

Proposal 7: Reuse Rel-17 FR2 38.104 Bi-directional scenario B channel model with updated the value of Dmin as 1m to align the deployment scenario for demodulation requirement in tunnel scenario.

# 3	Conclusion

In this contribution, the view on channel modeling of FR2 HST demodulation requirement in tunnel scenario is provided.

Proposal 1: RAN4 considers both Uni-directional and Bi-directional deployment for demodulation requirement for FR2 HST-DPS in tunnel scenario. For Uni-directional deployment, FR2 HST UE moves in the same direction as the RRH serving beam direction

Proposal 2: Deprioritized the demodulation requirement for two-panel simultaneous reception in Bi-directional scenario in the tunnel scenario.

Proposal 3: No new channel model (multi-tap) is needed pending on UE location with faraway or closed

Proposal 4: Not consider multipath fading channel for demodulation performance

Propose 5: Reuse the channel model in RAN4 spec 38.101-4, i.e, single path with LoS propagation, for performance requirement study of FR2 HST-DPS in tunnel deployment with updated Ds, Dmin and Ds_offset.

Uni-directional scenario

cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2,  0<t≤Dsv

cosθt=cosθt modDsv,  t>Ds/v

| Parameter | Value |
| --- | --- |
|  | TBD |
| Ds | 700 m |
| Ds_offset | [5m] |
| Dmin | 1 m |
| v | 350 km/h |
| fd | 9722 Hz |

Bi-directional scenario

cosθt=Ds-vtDmin2+Ds-vt2,  0<t≤(0.5*Ds)/v

cosθt=-vtDmin2+vt2,  (0.5*Ds)/v<t≤Ds/v 
cosθt modDsv,    t>Ds/v

| Parameter | Value |
| --- | --- |
|  | TBD |
|  | 700 m |
|  | 1 m |
|  | 350 km/h |
|  | 9722 Hz |

Proposal 6: The value of Ds_offset as [5]m can be considered for channel model of Uni-directional scenario in Tunnel scenario.

Proposal 7: Reuse Rel-17 FR2 38.104 Bi-directional scenario B channel model with updated the value of Dmin as 1m to align the deployment scenario for demodulation requirement in tunnel scenario.

# References

RP- 222272, Revised WID on enhanced NR support for high-speed train scenario in frequency range 2 (FR2)

R4-2303173	WF on NR FR2 HST Enhancements RRM requirements		Samsung