3GPP TSG-RAN WG4 Meeting # 107	R4-2308863

# Incheon, KR, May 22 – May 26, 2023

Title: 	Discussions on network signaling assistance for MU-MIMO receiver

Source: 	Huawei, HiSilicon

Agenda item:	8.19.1.1

Document for:	Discussion

Background

In last meeting, a WF [1] was agreed and many open issues on network assistance are left. This paper provides our views on related open issues.

Discussions

Work plan

As pointed out in last meeting, only two meetings are left before the freezing of RAN1 R18 work. Therefore it’s proposed to complete the discussion on DCI related signalling and send the LS if agreed to RAN1 before the end of May meeting to allow sufficient time for RAN1 to further discuss.

Proposal 1: RAN4 to make decision on the DCI based network assistant signalling and send the LS to RAN1 if agreed by the end of the May meeting.

Target scenario of the NWA

Based on the test scope specified in WID, the target study scenario for MU-MIMO advanced receiver is that all  paired UEs are    configured with dmrs-Type=1 and maxLength=1. I.e. Up to 4 DMRS ports. RAN4 have never evaluated the performance for advanced receiver for other more complicated configurations. Our understanding is that NWA at least for DCI related on modulation order should also be limited to the same scenario.

Proposal 2: NWA at least for DCI related should be limited to configurations of dmrs-Type=1 with maxLength=1 .I.e. only up to 4 DMRS ports.

The presence of co-scheduled UE

As pointed out in the last meeting, this information is necessary to be signalled using DCI which can be indicated with one value in DCI bit field specified for modulation order. We would like to mention that presence of co-scheduled UE means that co-scheduled UE(s) which has same DMRS sequence as target UE is existing because non-orthogonal interference UE is agnostic to target UE which can be whitened by using traditional IRC algorithm.

Proposal 3: Use one value in DCI bit field specified for modulation order to indicate the presence of co-scheduled UEs which have same DMRS sequence as target UE

The DMRS port information for the co-scheduled UE

One of issues is signalling on DCI port information, candidate options are listed as follows:

Issue 1-2-2-3: The DMRS port information for the co-scheduled UE GTW agreements on Apr 17th: - Dedicated DCI signaling is not preferred for the DMRS port information - FFS whether assistant RRC signalling can be introduced to reduce the BD complexity and/or maintain reasonable CE performance for target UE - Companies are encouraged to further evaluate BD performance including the detailed assumption: - Number of co-scheduled UE for BD - Time/frequency location of co-scheduled UE Way forward - Study the BD performance together with FDRA information as recommended in Issue 1-2-2-8. - FFS whether to introduce additional assistant RRC signalling to restrict the BD complexity.

It has been agreed to preclude the dedicate DCI related signalling on DMRS port. Regarding the suggestion that jointly indicating DMRS port information with MO, our concern is the bit overhead, at least 3*3=9 bits is needed which is unacceptable to RAN1.  Some companies suggest introducing RRC signalling, our understanding is that the max DMRS port to be detected depends on it’s DMRS port allocation and potential co-scheduled UE’s DMRS port allocation specified by RAN1 spec and this information usually dynamically change, so RRC signalling can’t work.

Proposal 4: Don’t introduce signalling on DMRS port information, which can be left to UE blind detection.

Frequency domain resource allocation information of the co-scheduled UE

We have same comments as DMRS port information, it is impossible and unnecessary to introduce any signalling on this information.  Per PRB/PRG interference presence detection is unavoidable.

Proposal 5: Don’t introduce signalling on frequency domain resource allocation, which can be left to UE blind detection.

DMRS power boosting for the co-scheduled UE

The assumption that all interference UEs have same DMRS power boosting can’t be guaranteed in any spec. Taking following as an example:

Table 2-1: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 1 | 0,1 |
| 3 | 2 | 0 |
| 4 | 2 | 1 |
| 5 | 2 | 2 |
| 6 | 2 | 3 |
| 7 | 2 | 0,1 |
| 8 | 2 | 2,3 |
| 9 | 2 | 0-2 |
| 10 | 2 | 0-3 |
| 11 | 2 | 0,2 |
| 12-15 | Reserved | Reserved |

Target UE is scheduled by value 3 in DCI format 1_1, “Antenna port(s)” field. Meanwhile, this UE detects a co-scheduled UE in port 1 with following two possibilities:

- Case 1: Co-scheduled UE is scheduled by value 4, DMRS power boosting is -3dB

- Case 2: Co-scheduled UE is scheduled by value 1, DMRS power boosting is 0dB

Therefore, this UE has no idea which power level should be applied to PDSCH after channel estimation processing. If this UE applies wrong power boosting to the PDSCH, there would be large performance degradation especially for R-ML receiver. We did a simulation to evaluate the performance degradation. In the simulation, Rank 1+1, 2T2R Medium, 16QAM (MCS13 for target UE) + 16QAM (Interference UE) are assumed, target UE has DMRS power boosting equalling to -3dB (Number of DMRS CDM groups without data=2), co-scheduled UE has DMRS power boosting equalling to 0dB(Number of DMRS CDM groups without data=1). Two cases are simulated:

- Case 1: Target UE assume DMRS power boosting of co-scheduled UE is -3dB (Wrong power boosting assumption)

- Case 2: Target UE assume DMRS power boosting of co-scheduled UE is 0dB(Correct power boosting assumption)

The simulation results are shown in Figure 2-1:

Figure 2-1: Simulation results for different assumptions on DMRS power boosting

We can observe that there is approximate 5dB performance loss for R-ML receiver due to the wrong assumption of power boosting, even for the E-MMSE-IRC receiver, the performance loss is 0.4dB. Hence we strongly suggest introducing 1 DCI (Implicitly indicated with signalling on MO without dedicate bit) or RRC bit to indicate the scenario that DMRS power boosting is not aligned between paired UEs so target UE can disabled the R-ML or E-MMSE-IRC (Depend on UE implementation) receiver directly to avoid the potential performance degradation.

Proposal 6: Introduce 1 DCI (Implicitly indicated with signalling on MO without dedicate bit) or RRC bit in case DMRS power boosting is not aligned between paired UEs.

Modulation order

In last meeting, companies provided following suggestions on signalling design for modulation order which can be summarized as follows:

- Proposal 1: DCI based signaling indicate the MO and DMRS port information jointly.

- Proposal 2: 3 bits DCI based signaling indicate some cases which are suitable for R-ML receiver.  Case 1: All paired UEs across bandwidth scheduled for target UE have same modulation order. Case 2: In each PRB, interference UEs have same modulation order.

| Bitfield | coUE modulation order | UE behavior |
| --- | --- | --- |
| 000 | No UE which has same DMRS sequence with target paired | SU-MIMO (MMSE-IRC) |
| 001 | PRB aligned (Single coUE modulation per PRB) | UE fallback to MMSE-IRC or R-ML with blind detection with only 4 hypotheses (UE dependent) |
| 010 | Bandwidth aligned, QPSK | R-ML without modulation order detection |
| 011 | Bandwidth aligned, 16-QAM | R-ML without modulation order detection |
| 100 | Bandwidth aligned, 64-QAM | R-ML without modulation order detection |
| 101 | Bandwidth aligned, 256-QAM | R-ML without modulation order detection |
| 110 | Bandwidth aligned, 1024-QAM | R-ML without modulation order detection |
| 111 | PRB not aligned (Multiple coUE modulations per PRB) | UE fallback to MMSE-IRC or R-ML with blind detection (UE dependent) |

- Proposal 3: Introduce the RRC based signaling to inform UE the MCS index table(s) used for PDSCH of the co-scheduled UEs, or signal the maximum MCS index table (64QAM, 256QAM or 1024QAM table) used among all the (potential) co-scheduled UEs.

- Proposal 4: Include blind detection opportunities for modulation order detection using semi-static DUT resource allocation, where DUT is not scheduled with PDSCH, but co-scheduled UEs are scheduled with PDSCH.

- Proposal 5: Introduce DCI signaling to assist blind detection, such as 1 bit modulation order set information for each co-scheduled UE to reduce search space, for example, when gNB decide 16QAM for a co-scheduled UE, then gNB transmit 1 bit to indicate the modulation order of this co-scheduled UE is in the set {16QAM, 64QAM}

For proposal 1, our biggest concern is bit overhead, for each port 4 bits is needed to indicate all possible modulation orders, totally 12 bits is needed. We think it is unacceptable to RAN1.

For proposal 3, we think the reduction of complexity is limited since 256QAM is widely used in real work. Our concern is that what UEs to be paired (Or maximum MCS index Table) can be dynamically changed which may lead to frequent RRC reconfiguration. Or we have misunderstanding that P3 intends to indicate the max MCS index Table in the whole cell, if so we can further discuss the benefit.

For proposal 4, by reviewing the proponent’s clarification in the email thread, it seems that P4 means Network semi-statically configures a pattern indicating position of REs which are not used for PDSCH but to be used for MO detection. Firstly, this solution requires RAN1 define a new Rate-matching pattern or transmission scheme since it is not allowed in current spec. Secondly, we don’t see any benefit to reduce the blind detection complexity since the total hypotheses is not reduced.

For proposal 5: Introducing a modulation order set is a good idea to reduce the complexity and potentially improve the performance. UE can benefit from lower modulation order and few modulation types of co-scheduled UEs. But our biggest concern is that we may not have enough time to discuss the detailed modulation set due to the quite limited timeline since the modulation set has very flexible implementation and it’s difficult to unify the views of companies.

For proposal 2: We think this proposal is a good solution to trade off the bit overhead, reducing complexity and improving the performance. Bit 010 to 110 are specified for UEs not capable of modulation order blind detection, bit 001 targets to reduce the complexity for a UE capable of modulation order detection but suffering multiple interference layers by reducing the number of hypotheses to 4 which gives target UE an opportunity to perform R-ML for all interference layers to get the most optimal performance. In other words, making interference layers have same modulation order in each PRB is the only possible potential implementation to enable UEs to perform R-ML algorithm for all interference layers with proper signalling assistance.

Observation 1: Example 2 in proposal 2 can cover both UEs supporting or not supporting modulation order blind detection.

Observation 2: Making interference layers have same modulation order in each PRB is the only possible potential implementation to enable UEs to perform R-ML algorithm for all interference layers with proper signalling assistance.

Furthermore, I don’t agree the concern that cases indicated from 001 to 110 is rare.  At least for a UE with limited RB allocation small bandwidth, it is very likely that all interference have same MO across the whole bandwidth scheduled for itself. Take cases in Figure 2-2 as an example, modulation order of UE1 is PRB aligned with other UEs, so UE1 can perform R-ML for two interference layers by indicating “001”. Modulation order of any of UE2 to UE6 are bandwidth aligned with UE1, so UE2 to UE6 can perform R-ML receiver without blind detection by indicating “011”.

Figure 2-2: Example of case which is friendly to R-ML receiver

Observation 3: For a UE with small bandwidth, it is very likely that all interference have same MO across the whole bandwidth scheduled for itself.

In order to facilitate network to implement MU-MIMO scheduling schemes which can serve more UEs capable R-ML receiver, it’s better to introduce two new UE capabilities, (1) Capability of supporting R-ML receiver with modulation order of interference layers known and (2) Capability of supporting R-ML receiver with modulation order of interference layers blind detection. Network can identify these two types of UEs by capability reporting and apply special MU-MIMO schedule schemes which are benefit to these UEs such as case in Figure 2-2.

Proposal 7: Introduce two new UE capabilities (1) Capability of supporting R-ML receiver with modulation order of interference layers known and (2) Capability of supporting R-ML receiver with modulation order of interference layers blind detection. Network can identify these two types of UEs by capability reporting and apply special MU-MIMO schedule schemes which are benefit to these UEs such as case in Figure 2-2.

We present the performance of MO blind detection for following three cases as per WF [1], additionally, we also simulated the case with Rank 1+2 (Case 3)

- Case1: Rank 1+1, 2T2R, MCS 13 for the target UE, QPSK interference, TDLC300-100 medium, random precoding

- Case2: Rank 2+2, 4T4R, MCS 17 for the target UE, 16QAM interference, TDLA30-10 low, orthogonal precoding

  - UE assume two interference have same modulation order

- Case3: Rank 1+2, 4T4R, MCS 17 for the target UE, 16QAM interference, TDLA30-10 medium A, orthogonal precoding.

Performance of Case 1 is shown in Figure 2-3, we can observe that performance loss is negligible.

Figure 2-3: Simulation results for MO blind detection performance for Rank 1+1

Performance of Case 2 is shown in Figure 2-4, we can observe that performance loss is approximate 1.3dB and the R-ML performance with modulation order blind detection is quite closed to E-MMSE-IRC receiver

Figure 2-4: Simulation results for MO blind detection performance for Rank 2+2

For case 3, four types of receiver are assumed.

- E-MMSE-IRC

- R-ML applied for all layers with MO known. This receiver simulates target UE is signaled the exact modulation order of interference UE

- R-ML applied for all layers with MO BD with prior information that two interference layers have same MO. This receiver simulates target UE is signaled that all interference layers have same MO.

- R-ML with modulation order applied for 1 interference layer and E -IRC with modulation order blind detection applied for the other 1 interference layer. This receiver simulates the UE perform R-ML without signaling assistance.

Simulation results are captured in Figure 2-5 and Table 2-2:

Figure 2-5: Simulation results for MO blind detection performance for Rank 1+2

Table 2-2: Summary of Simulation results for MO blind detection performance for Rank 1+2

| Receiver Type | E-MMSE-IRC | R-ML for 2layers with MO known | R-ML for 2layers with MO BD | 1 layer R-ML with MO BD+ 1layer E-IRC |
| --- | --- | --- | --- | --- |
| SNR(dB)# @70 % of maxTP | 25.0 | 17.7 | 18.3 | 21.1 |

With the signalling indicating all interference layers have same modulation order, the R-ML performance is improved by 3dB compared to R-ML without signalling assistance and has only 0.6dB performance degradation compared to ideal condition. This observation verifies the benefit of signalling of proposal 2.

Observation 4: With the signalling indicating all interference layers have same modulation order, the R-ML performance is improved by 3dB compared to R-ML without signalling assistance and have only 0.6dB performance degradation compared to ideal condition.

Proposal 8: Use proposal 2 in WF [1] as signalling on modulation order.

Time domain resource allocation information of the co-scheduled UE

Even this scenario is outside the WI scope, we can compromise to use 1 DCI (Implicitly indicated with signalling on MO without dedicate bit) or RRC bit to indicate the case that time allocation is not aligned between paired UEs to avoid potential performance degradation.

Proposal 9: Use 1 DCI (Implicitly indicated with signalling on MO without dedicate bit) or RRC bit to indicate the case that time allocation is not aligned between paired UEs.

RS location information of the co-scheduled UE

We can compromise to use 1 DCI (Implicitly indicated with signalling on MO without dedicate bit) or RRC bit to indicate the case that RS location is not aligned between paired UEs to avoid potential performance degradation.

Proposal 10: Use 1 DCI (Implicitly indicated with signalling on MO without dedicate bit) or RRC bit to indicate the case that RS location is not aligned between paired UEs.

PRB bundling size for the co-scheduled UE

Following regulation has been captured in 38.214:

The UE does not expect the precoding of the potential co-scheduled UE(s) in other DM-RS ports of the same CDM group to be different in the PRG-level grid configured to this UE with PRG =2 or 4. The UE does not expect the resource allocation of the potential co-scheduled UE(s) in other DM-RS ports of the same CDM group to be misaligned in the PRG-level grid to this UE with PRG=2 or 4.

It means in each PRG, UE can assume co-scheduled UEs in the same CDM group have same precoding and resource allocation as its own with PRG=2, 4 so this UE can perform joint channel estimation in each PRG. This regulation can be reused to cases with different CDM groups. I.e. we prefer to use following clarification:

Default assumption 1: UE assume in each its PRG, the precoding of the potential co-scheduled UE(s) in all DM-RS ports of different CDM group to be same with PRG=2 or 4.

One invalid scenario of default assumption can be shown in Figure 2-6, assume UE1 is target UE, PRG of UE2 is smaller than UE1 and different PMI is applied to different PRGs of UE2, so UE1 has to fallback to per RB based interference cancellation.

Figure 2-6: Invalid scenario of default assumption1

Default assumption 2: UE assume in each its PRG, the resource allocation of the potential co-scheduled UE(s) in other DM-RS ports of different CDM group to be aligned with PRG=2 or 4.

One invalid scenario of default assumption can be shown in Figure 2-7, assume UE1 is target UE,  two UEs are allocated in PRG3 of UE1 which may cause different PMI and different modulation order which do harm to E-MMSE-IRC/R-ML receiver, so UE1 has to fallback to per RB based interference cancellation.

Figure 2-7: Invalid scenario of default assumption2

Therefore, only both default assumption 1 and default assumption 2 are valid can enable UE to perform interference cancellation with PRG granularity.

Proposal 11: RAN4 to make following assumptions for advanced receiver:

- Default assumption 1: UE assume in each its PRG, the precoding of the potential co-scheduled UE(s) in all DM-RS ports of different CDM group to be same with PRG=2 or 4.

- Default assumption 2: UE assume in each its PRG, the resource allocation of the potential co-scheduled UE(s) in other DM-RS ports of different CDM group to be aligned with PRG=2 or 4.

- Introduce 1 bit RRC signaling to indicate in case default assumption 1 or/and default assumption 2 is invalid.

Conclusion

In this paper, we provide our views on network signalling on advanced receiver requirements definition. The observations and proposals are:

Proposal 1: RAN4 to make decision on the DCI based network assistant signalling and send the LS to RAN1 if agreed by the end of the May meeting.

Proposal 2: NWA at least for DCI related should be limited to configurations of dmrs-Type=1 with maxLength=1 .I.e. only up to 4 DMRS ports.

Proposal 3: Use one value in DCI bit field specified for modulation order to indicate the presence of co-scheduled UEs which have same DMRS sequence as target UE

Proposal 4: Don’t introduce signalling on DMRS port information, which can be left to UE blind detection.

Proposal 5: Don’t introduce signalling on frequency domain resource allocation, which can be left to UE blind detection.

Proposal 6: Introduce 1 DCI (Implicitly indicated with signalling on MO without dedicate bit) or RRC bit in case DMRS power boosting is not aligned between paired UEs.

Observation 1: Example 2 in proposal 2 can cover both UEs supporting or not supporting modulation order blind detection.

Observation 2: Making interference layers have same modulation order in each PRB is the only possible potential implementation to enable UEs to perform R-ML algorithm for all interference layers with proper signalling assistance.

Observation 3: For a UE with small bandwidth, it is very likely that all interference have same MO across the whole bandwidth scheduled for itself.

Proposal 7: Introduce two new UE capabilities (1) Capability of supporting R-ML receiver with modulation order of interference layers known and (2) Capability of supporting R-ML receiver with modulation order of interference layers blind detection. Network can identify these two types of UEs by capability reporting and apply special MU-MIMO schedule schemes which are benefit to these UEs such as case in Figure 2-2.

Observation 4: With the signalling indicating all interference layers have same modulation order, the R-ML performance is improved by 3dB compared to R-ML without signalling assistance and have only 0.6dB performance degradation compared to ideal condition.

Proposal 8: Use proposal 2 in WF [1] as signalling on modulation order.

Proposal 9: Use 1 DCI (Implicitly indicated with signalling on MO without dedicate bit) or RRC bit to indicate the case that time allocation is not aligned between paired UEs.

Proposal 10: Use 1 DCI (Implicitly indicated with signalling on MO without dedicate bit) or RRC bit to indicate the case that RS location is not aligned between paired UEs.

Proposal 11: RAN4 to make following assumptions for advanced receiver:

- Default assumption 1: UE assume in each its PRG, the precoding of the potential co-scheduled UE(s) in all DM-RS ports of different CDM group to be same with PRG=2 or 4.

- Default assumption 2: UE assume in each its PRG, the resource allocation of the potential co-scheduled UE(s) in other DM-RS ports of different CDM group to be aligned with PRG=2 or 4.

- Introduce 1 bit RRC signaling to indicate in case default assumption 1 or/and default assumption 2 is invalid.

Reference

- R4-2302929 WF on advanced receiver for MU-MIMO scenario