3GPP TSG-RAN WG4 Meeting # 107	R4-2308867

# Incheon, KR, May 22 – May 26, 2023

Title: 	Discussions on ATP requirements

Source: 	Huawei, HiSilicon

Agenda item:	9.19.2

Document for:	Discussion

Background

As per [1], some open issues on ATP requirements were left, in this paper, we provide our views on these open issues.

Discussions

Test point T (%) selection

One issue is test point, candidate options are listed as follows:

- Issue A. # of test point(s) - Option 1. Two test points for 2Rx and 4 Rx. - Option 2. Two test points for 2Rx and 1 test point for 4 Rx. - Option 3. Single test point for 2Rx and 4 Rx - Issue B. T(%) and outlier removal for Gspan <= 2.5 dB

Based on the simulation results provided by companies, for 4Rx, almost all SNR points are corresponding to Rank 2, so only one test point is enough. It’s better to keep consistent for 2Rx and 4Rx so we suggest choosing 1 test point for 2Rx also. To cover test points both with Rank1 and Rank2 and reduce the test number, we suggest splitting two test points into cases with different duplex mode. Please following:

- FR1 FDD 2x2: 15% (Rank1)

- FR1 FDD 2x4: 50% (Rank2)

- FR1 TDD 2x2: 15% (Rank1)

- FR1 TDD 2x4: 35% (Rank2)

- FR2 TDD 2x2: 40% (Rank2)

Proposal 1: Choose single test point for 2Rx and 4Rx by splitting two test points into cases with different duplex mode:

- FR1 FDD 2x2: 15% (Rank1)

- FR1 FDD 2x4: 50% (Rank2)

- FR1 TDD 2x2: 15% (Rank1)

- FR1 TDD 2x4: 35% (Rank1)

- FR2 TDD 2x2: 40% (Rank2)

X dB margin

One issue is how to add margin to the impairment results, candidate options are listed as follows:

- Option 1: Apply X dB margin per modulation order to impairment results X = [0.5] dB for QPSK, X = [0.5] dB for 16QAM X = [0.8] dB for 64QAM, X = [0.8] dB for 256QAM Note that the modulation order would be based on median value over companies’ own median statistics on CQI. - Option 2: Apply single X dB margin to impairment results (e.g. X=0.5 dB or else)

Based on our understanding, it’s better to leave XdB to companies’ own determination since companies may have different modulation order at tested SNR point. Companies can add XdB margin to there impairment results and final results are derived from the average of impairment results from all companies without additional margin.

Proposal 2: Companies add XdB margin to there impairment results and final results are derived from the average of impairment results from all companies without additional margin.

Applicability and release independence

One issue is applicability and release independence, candidate options are listed as follows:

- Option 1: The requirement with link adaptation should be applicable for all NR UEs without any new applicability rules, and the requirement should be release independent from Rel-15 - Option 1.a: Proposal considering declaration - Optional for Rel-15 and Rel-16 UEs based on declaration - Mandatory for all Rel-17 and forward UEs.. - Option 2: The absolute physical layer throughput requirement with link adaptation should be applicable from Rel-18 and not release independent from Rel-15.

Our position is not changed, there is no reason to require an old UE to pass a new requirement. Even though no new feature is introduced, potential algorithms may be updated with consideration of supporting more new features introduced in the later releases. It is noted that Rel-18 ATP test has different test purpose and test metric compared to Rel-15 CSI test.

Proposal 3: Support Option 2. The absolute physical layer throughput requirement with link adaptation should be applicable from Rel-18 and not release independent from Rel-15.

Conclusion

In this paper we provide our views on ATP requirements, the proposals are:

Proposal 1: Choose single test point for 2Rx and 4Rx by splitting two test points into cases with different duplex mode:

- FR1 FDD 2x2: 15% (Rank1)

- FR1 FDD 2x4: 50% (Rank2)

- FR1 TDD 2x2: 15% (Rank1)

- FR1 TDD 2x4: 35% (Rank1)

- FR2 TDD 2x2: 40% (Rank2)

Proposal 2: Companies add XdB margin to there impairment results and final results are derived from the average of impairment results from all companies without additional margin.

Proposal 3: Support Option 2. The absolute physical layer throughput requirement with link adaptation should be applicable from Rel-18 and not release independent from Rel-15.

- 4   Reference

- R4-2305915 WF on absolute physical layer throughput requirement