3GPP TSG-RAN WG4 Meeting #107	R4-2308883

# Incheon, 22nd - 26th May, 2023

Title: 	Simulation results on NR BS ATG demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.14.5.3

Document for:	Discussion

Background

During RAN4#106-bis-e meeting, WF [1] for ATG demodulation requirements was approved. In this contribution, we provide our initial simulation results about NR BS ATG demodulation requirements.

Simulation

The ideal simulation results for NR BS ATG demodulation requirements are shown as following Table 2-1.

Table 2-1 Ideal simulation results for NR BS ATG demodulation requirements

| Case number | SCS | Bandwidth | Antenna configuration | Propagation condition | Rank | SNR@70% max TP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MCS13 | MCS17 | MCS24(Table2) |
| 1 | 15kHz | 5MHz | 1x2 | AWGN + 200Hz Doppler | 1 | 3.67 | 6.70 | 19.52 |
| 2 | 15kHz | 5MHz | 1x4 | AWGN + 200Hz Doppler | 1 | 1.16 | 4.24 | 16.57 |
| 3 | 15kHz | 5MHz | 1x8 | AWGN + 200Hz Doppler | 1 | -0.99 | 2.27 | 13.82 |
| 4 | 30kHz | 10MHz | 1x2 | AWGN + 500Hz Doppler | 1 | 3.56 | 6.76 | 19.81 |
| 5 | 30kHz | 10MHz | 1x4 | AWGN + 500Hz Doppler | 1 | 1.07 | 4.30 | 16.90 |
| 6 | 30kHz | 10MHz | 1x8 | AWGN + 500Hz Doppler | 1 | -1.07 | 2.31 | 14.12 |

The maximum throughput cannot achieve the maximum throughput under 256QAM.

Proposals

In this contribution, we provide our initial simulation results about NR BS ATG demodulation requirements. Our observations and proposals are:

- The maximum throughput cannot achieve the maximum throughput under 256QAM.

Reference

R4-2305899, WF for ATG demodulation requirements, RAN4#106-bis-e, CMCC