3GPP TSG-RAN WG4 Meeting #107	R4-2308894

# Incheon, 22nd - 26th May, 2023

Title: 	Discussion on UE demodulation requirements for LTE NTN IOT

Source: 	Huawei, HiSilicon

Agenda item:	9.7.7.1

Document for:	Information

Background

During RAN4#106-bis-e meeting, WF [1] on IoT-NTN UE demodulation and CQI reporting requirements was approved. In this contribution, we share our view about UE demodulation requirements for LTE NTN IOT.

Discussion

NPDCCH repetition factor in test 2

- For NB-IoT NPDSCH demodulation requirements, set NPDCCH repetition factor to be 128 in test 2. - Interested companies can verify this repetition number ensures the NPDCCH BLER becomes low enough.

Our simulation results paper shows that the target SNR @ 70% maximum throughput for the NPDSCH test 2 is -14.32dB. Here we provide the evaluation results for the NPDCCH performance with repetition factor 128 or 256 as following Table 2.1-1.

| SNR [dB] | NPDCCH repetition factor | BLER |
| --- | --- | --- |
| -14.32 | 128 | 0.099 |
| -14.32 | 256 | 0.004 |

NPDCCH performance with repetition factor 128 is not low enough.

We can see that NPDCCH performance with repetition factor 128 is not low enough, so we propose to select NPDCCH repetition factor 256 for NPDSCH test 2.

Select NPDCCH repetition factor 256 for NPDSCH test 2.

Summary

In this contribution, we share our view about UE demodulation requirements for LTE NTN IOT.

- NPDCCH performance with repetition factor 128 is not low enough.

- Select NPDCCH repetition factor 256 for NPDSCH test 2.

Reference

R4-2305960, WF on IoT-NTN UE demodulation and CQI reporting requirements, RAN4#106-bis-e, MediaTek