3GPP TSG-RAN WG4 Meeting #107	R4-2308898

# Incheon, 22nd - 26th May, 2023

Title: 	Simulation results on SAN demodulation requirements for LTE NTN IOT

Source: 	Huawei, HiSilicon

Agenda item:	9.7.7.2

Document for:	Information

Background

During RAN4#106-bis-e meeting, WF [1] on IoT NTN SAN demodulation requirements was approved. In this contribution, we provide our updated simulation results about SAN demodulation requirements for LTE NTN IOT.

Discussion

SAN demodulation requirements for eMTC

PUSCH CE mode A

The ideal simulation results for PUSCH CE mode A demodulation requirements are shown as Table 2.1.1-2 based on the simulation assumption shown as Table 2.1.1-1.

Table 2.1.1-1 Simulation assumptions for PUSCH CE mode A demodulation requirements

| Parameters | CE Mode A |
| --- | --- |
| Max number of HARQ transmissions | 4 |
| RV sequences | 0, 2, 3, 1, 0, 2, 3, 1 |
| Number of PUSCH repetitions | 8 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | Option 1: NTN-TDLC5-5 Option 2: NTN-TDLA100-5 |
| FRC | TS36.104 A3-2 |
| System BW | 1.4MHz only |
| Fraction of maximum throughput | 70% |

Table 2.1.1-2 Ideal simulation results for PUSCH CE mode A demodulation requirements

| Test number | Number of PUSCH repetitions | Antenna configuration | SNR@70% maximum throughput [dB] |  |
| --- | --- | --- | --- | --- |
|  |  |  | NTN-TDLA100-5 | NTN-TDLC5-5 |
| 1 | 8 | 1x1 | -3.78 | -3.38 |
| 2 | 8 | 1x2 | -8.54 | -7.28 |

PUSCH CE mode B

The ideal simulation results for PUSCH CE mode B demodulation requirements are shown as Table 2.1.2-2 based on the simulation assumption shown as Table 2.1.2-1.

Table 2.1.2-1 Simulation assumptions for PUSCH CE mode B demodulation requirements

| Parameters | CE Mode B |
| --- | --- |
| Max number of HARQ transmissions | 2 |
| RV sequences | 0, 0, 0, 0, 2, 2, 2, 2, 3, 3, 3, 3, 1, 1, 1, 1 |
| Number of PUSCH repetitions | 256 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | Option 1: NTN-TDLA100-5 Option 2: NTN-TDLC5-5 |
| FRC | TS36.104 A3-1 |
| System BW | 1.4MHz only |
| Fraction of maximum throughput | 70% |

Table 2.1.2-2 Ideal simulation results for PUSCH CE mode B demodulation requirements

| Test number | Antenna configuration | SNR@70% maximum throughput [dB] |  |
| --- | --- | --- | --- |
|  |  | NTN-TDLA100-5 | NTN-TDLC5-5 |
| 1 | 1x1 | -16.55 | -17.32 |
| 2 | 1x2 | -18.78 | -19.03 |

PUCCH

The ideal simulation results for PUCCH demodulation requirements are shown as Table 2.1.3-2 based on the simulation assumption shown as Table 2.1.3-1.

Table 2.1.3-1 Simulation assumptions for PUCCH demodulation requirements

| Parameters | Values |
| --- | --- |
| Number of PUCCH repetitions | 8 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | NTN-TDLA100-5 |
| System BW | 1.4MHz |

Table 2.1.3-2 Ideal simulation results for PUCCH demodulation requirements

| Test number | Antenna configuration | SNR@70% maximum throughput [dB] |
| --- | --- | --- |
| 1 | 1x1 | -7.65 |
| 2 | 1x2 | -9.64 |

PRACH

The ideal simulation results for PRACH demodulation requirements are shown as Table 2.1.4-2 based on the simulation assumption shown as Table 2.1.4-1.

Table 2.1.4-1 Simulation assumptions for PRACH demodulation requirements

| Number of Tx antennas | Number of Rx antennas | Propagation channel | Frequency offset | Number of repetitions | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | NTN-TDLA100-5 | 270Hz | 8 | YES | YES |  |  |
|  |  |  |  | 16 |  |  | YES | YES |

Table 2.1.4-2 Ideal simulation results for PRACH demodulation requirements

| Test number | Burst format | Antenna configuration | SNR@1% BLER [dB] |
| --- | --- | --- | --- |
| 1 | 0 | 1x1 | -13.75 |
| 2 | 0 | 1x2 | -18.43 |
| 3 | 1 | 1x1 | -13.77 |
| 4 | 1 | 1x2 | -18.29 |
| 5 | 2 | 1x1 | -17.34 |
| 6 | 2 | 1x2 | -21.12 |
| 7 | 3 | 1x1 | -17.49 |
| 8 | 3 | 1x2 | -21.16 |

SAN demodulation requirements for NB-IoT

NPUSCH format 1

The ideal simulation results for NPUSCH format 1 demodulation requirements are shown as Table 2.2.1-2 based on the simulation assumption shown as Table 2.2.1-1.

Table 2.2.1-1 Simulation assumptions for NPUSCH format 1 demodulation requirements

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions | FRC | Repetition number | Fraction of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | 3.75KHz | 1 | Option 1: NTN-TDLA100-1 Option 2: NTN-TDLC5-1 | TS36.104 A16-1 | 4 | 70% |
| 1 | 1 and 2 | 15KHz | 12 | Option 1: NTN-TDLC5-1 Option 2: NTN-TDLA100-1 | TS36.104 A16-5 | 16 | 70% |

Table 2.2.1-2 Ideal simulation results for NPUSCH format 1 demodulation requirements

| Test number | Subcarrier spacing [kHz] | Antenna configuration | SNR@70% maximum throughput [dB] |  |
| --- | --- | --- | --- | --- |
|  |  |  | NTN-TDLA100-1 | NTN-TDLC5-1 |
| 1 | 3.75 | 1x1 | -5.33 | -5.30 |
| 2 | 3.75 | 1x2 | -8.77 | -8.75 |
| 3 | 15 | 1x1 | -3.92 | -4.20 |
| 4 | 15 | 1x2 | -7.88 | -8.49 |

NPUSCH format 2

The ideal simulation results for NPUSCH format 2 demodulation requirements are shown as Table 2.2.2-2 based on the simulation assumption shown as Table 2.2.2-1.

Table 2.2.2-1 Simulation assumptions for NPUSCH format 2 demodulation requirements

| Number of TX antennas | Number of RX antennas | Propagation conditions | Number of allocated subcarriers | Subcarrier spacing | Repetition number |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | NTN-TDLA100-1 | 1 | 3.75KHz | 16 |
| 1 | 1 and 2 | NTN-TDLA100-1 | 1 | 15KHz | 16 |

Table 2.2.2-2 Ideal simulation results for NPUSCH format 2 demodulation requirements

| Test number | Subcarrier spacing [kHz] | Antenna configuration | SNR@1% BLER [dB] |
| --- | --- | --- | --- |
| 1 | 3.75 | 1x1 | -2.25 |
| 2 | 3.75 | 1x2 | -9.65 |
| 3 | 15 | 1x1 | -9.20 |
| 4 | 15 | 1x2 | -11.12 |

NPRACH

The ideal simulation results for NPUSCH format 2 demodulation requirements are shown as Table 2.2.3-2 based on the simulation assumption shown as Table 2.2.3-1.

Table 2.2.3-1 Simulation assumptions for NPRACH demodulation requirements

| Number of TX antennas | Number of RX antennas | Repetition number | Propagation channel and correlation | Frequency offset | PF0 | PF1 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | 8 | NTN-TDLA100-1 | 200Hz | YES | YES |
| 1 | 1 and 2 | 16 | NTN-TDLA100-1 | 200Hz | YES | YES |

Table 2.2.3-2 Ideal simulation results for NPRACH demodulation requirements

| Test number | Repetition number | Antenna configuration | SNR@1% BLER [dB] |  |
| --- | --- | --- | --- | --- |
|  |  |  | PF0 | PF1 |
| 1 | 8 | 1x1 | 8.25 | 8.01 |
| 2 | 8 | 1x2 | 3.66 | 3.72 |
| 3 | 32 | 1x1 | 1.34 | 1.49 |
| 4 | 32 | 1x2 | -2.63 | -2.88 |

Summary

In this contribution, we provide our updated simulation results about UE demodulation requirements for LTE NTN IOT.

Reference

R4-2305965, WF on IoT NTN SAN demodulation requirements, RAN4#106-bis-e, Ericsson