3GPP TSG-RAN WG4 Meeting # 107	< R4-2308917>

# Incheon, KR, May 22 – May 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3322 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on test cases for L1-SINR measurement for FR2 HST PC6 UE in TS 38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2 |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The core requirements of L1-SINR measurement accuracy for FR2 HST PC6 UE has been completed except for the side condition SNR upper bound. The test cases for L1-SINR measurement requirements are not defined, therefore it is necessary to introduce the test cases for verifying the requirements for L1-SINR measurement FR2 HST PC6 UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test cases of L1-SINR measurement accuracy requirement are introduced for FR2 PC6 UE dedicated for HST scenario. The test cases include A.7.6 Measurement procedure and A.7.7 Measurement Performance requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test cases to guarante the new requirement of L1-SINR measurement accuracy for FR2 PC6 UE dedicated for HST scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

A.7.6	Measurement procedure

A.7.6.6	L1-SINR measurement for beam reporting

A.7.7.6.X	L1-SINR measurement with SSB based CMR and dedicated IMR configured when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17

##### A.7.7.6.X.1	Test Purpose and Environment

The purpose of this test is to verify that the power class 6 UE makes correct reporting of L1-SINR measurement when highSpeedMeasFlagFR2-r17 is configured. This test will partly verify the L1-SINR measurement requirements for power class 6 UE configured with highSpeedMeasFlagFR2-r17 in clause 9.8.4.2, with the testing configurations for NR cells in Table A.7.7.6.X.1.

The AoA setup for this test is Setup 1 as defined in clause A.3.15

Table A.7.7.6.X.1-1: Applicable NR configurations for FR2 L1-SINR measurement test with SSB based CMR and CSI-IM based IMR L1-SINR test for power class 6 UE configured with highSpeedMeasFlag-r17

| Config | Description |
| --- | --- |
| 1 | LTE FDD,NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE FDD,NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

A.7.6.6.X.2	Test parameters

There is one cells in the test, the FR2 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.7.6.6.X.2-1 and Table A.7.6.6.X.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-SINR measurement on the SSBs and the associated CSI-IM resources, and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD measurements based on the SSBs, and UE is configured to perform L1-SINR measurement based on the SSBs as CMR and the CSI-IM resources as IMR.

Table A.7.6.6.X.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| highSpeedMeasFlagFR2-r17 | 1~2 |  | Set 1 |
| SSB GSCN | 1~2 |  | freq1 |
| Duplex mode | 1~2 |  | TDD |
| TDD Configuration | 1~2 |  | TDDConf.3.1 |
| BWchannel | 1~2 | MHz | 100: NRB,c = 66 |
| Data RBs allocated |  |  | 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.2 TDD |
|  | 2 |  | SR.3.3 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
|  | 2 |  | CR.3.2 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
|  | 2 |  | CCR.3.7 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
|  | 2 |  | SSB.2 FR2 |
| CSI-IM configuration | 1~2 |  | CSI-IM.3.3 TDD |
| OCNG Patterns | 1~2 |  | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1~2 |  | SMTC.1 |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 |
| DRX configuration | 1~2 |  | DRX.3 |
| reportConfigType | 1~2 |  | periodic |
| reportQuantity-r16 | 1~2 |  | ssb-Index-SINR-r16 |
| Number of reported RS | 1~2 |  | 2 |
| L1-SINR reporting period | 1~2 | slot | 640 |
| T1 | 1~2 | s | 5 |
| T2 | 1~2 | s | 2 |
| EPRE ratio of PSS to SSS |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~2 |  | AWGN 19444Hz |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.7.6.6.X.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  |  | Setup 1 according to A.3.15.1 |  |  |  |
| Beam assumptionNote 4 | 1~2 |  | Rough |  |  |  |
| Note2 | 1~2 | dBm/15kHz | -105 |  |  |  |
| Note2 | 1 | dBm/SSB SCS | -96 |  |  |  |
|  | 2 |  | -93 |  |  |  |
|  | 1~2 | dB | 0 | 0 | -Infinity | 9 |
| SSB RSRP Note3 | 1 | dBm/SSB SCS | -96 | -96 | -Infinity | -87 |
|  | 2 |  | -93 | -93 | -Infinity | -84 |
| Io Note3 | 1 | dBm/95.04MHz | -64 | -64 | -67 | -57.5 |
|  | 2 |  | -64 | -64 | -67 | -57.5 |
|  | 1~2 | dB | 0 | 0 | -Infinity | 9 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

A.7.6.6.X.3	Test Requirements

The UE shall send L1-SINR report every 640 slots. No later than 720 ms plus 640 slots from the beginning of time period T2, UE shall send L1-SINR report including the results for both SSB#0+CSI-IM#0 and SSB#1+CSI-IM#1 while meeting the accuracy requirements defined in clause 10.1.28.2

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<End of Change #1>

<Start of Change #2>

A.7.7	Measurement Performance requirements

A.7.7.6	L1-SINR measurement for beam reporting

A.7.7.6.X	L1-SINR measurement with SSB based CMR and dedicated IMR configured when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17

A.7.7.6.X.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clauses 9.8.4.2 and clause 10.1.28.2 for L1-SINR measurements with SSB based CMR and CSI-IM based IMR, with the testing configurations for NR cells in Table A.7.7.6.X.1-1.

The AoA setup for this test is Setup 1 as defined in clause A.3.15.

Table A.7.7.6.X.1-1: Applicable NR configurations for FR2 L1-SINR measurement test with SSB based CMR and CSI-IM based IMR L1-SINR test for power class 6 UE configured with highSpeedMeasFlag-r17

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE FDD, NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

A.7.7.6.X.2	Test parameters

In this set of test cases there are two cells in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.7.7.6.X.2-1 and Table A.7.7.6.X.2-2 below. The absolute and relative accuracy of L1-SINR measurements are tested by using the parameters in Table A.7.7.6.X.2-1 and Table A.7.7.6.X.2-2.

Here is no measurement gap configured in the test. Before the test, UE is configured one SSB resource set with two SSB resources and one CSI-IM resource set with two CSI-IM resource. UE is configured to perform RLM and BFD measurement based on the SSB resources 0 and 1. UE is configured to perform L1-SINR measurement based on the SSBs as CMR and the CSI-IM resources as IMR.

Table A.7.7.6.X.2-1: FR2 L1-SINR general test parameters with SSB based CMR and CSI-IM based IMR

| Parameter | Config | Unit | Test 1 |
| --- | --- | --- | --- |
| highSpeedMeasFlagFR2-r17 | 1~2 |  | Set 1 |
| SSB GSCN | 1~2 |  | freq1 |
| Duplex mode | 1~2 |  | TDD |
| TDD Configuration | 1~2 |  | TDDConf.3.1 |
| BWchannel | 1~2 | MHz | 100: NRB,c = 66 |
| Data RBs allocated | 1~2 |  | 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.2 TDD |
|  | 2 |  | SR.3.3 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
|  | 2 |  | CR.3.2 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
|  | 2 |  | CCR.3.7 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
|  | 2 |  | SSB.2 FR2 |
| CSI-IM configuration | 1~2 |  | CSI-IM 3.1 TDD |
| OCNG Patterns | 1~2 |  | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 |
| SMTC configuration | 1~2 |  | SMTC.1 |
| reportConfigType | 1~2 |  | periodic |
| reportQuantity-r16 | 1~2 |  | ssb-Index-SINR-r16 |
| Number of reported RS | 1~2 |  | 2 |
| L1-SINR reporting period | 1~2 |  | slot640 |
| Propagation condition | 1~2 |  | AWGN 19444Hz |
| Antenna configuration | 1~2 |  | 1x2 |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |

Table A.7.7.6.X.2-2: FR2 L1-SINR SSB specific test parameters

| Parameter | Config | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | SSB0 | SSB1 |
| Angle of arrival configuration |  |  | Setup 1 according to A.3.15.1 |  |
| Assumption for UE beamsNote 4 |  |  | Rough |  |
|  | 1~2 | dBm/15kHz | -100 |  |
|  | 1 | dBm/SSB SCS | -91 |  |
|  | 2 |  | -88 |  |
|  | 1~2 | dB | 8 | -2 |
| SS-RSRPNote1 | 1 | dBm/SCS | -81 | -93 |
|  | 2 |  | -78 | -90 |
| IoNote1 | 1~2 | dBm/95.04 MHz | -51.57 | -59.86 |
|  | 1~2 | dB | 10 | -2 |
| Note 1: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 3: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

A.7.7.6.X.3	Test Requirements

After 640ms from the beginning of the test, the L1-SINR measurement accuracy for SSB#0+CSI-IM#0 and SSB#1+CSI-IM#1 of Cell 1 shall fulfil the requirements in clauses 10.1.28.2. The following requirements are to be verified:

For Test 1:

Absolute accuracy of SSB#0+CSI-IM#0. The UE is deemed to meet the requirement if the reported L1-SINR is in the range shown in Table A.7.7.6.X.3-1.

Relative accuracy of SSB#0+CSI-IM#0 compared with SSB#1+CSI-IM#1. The UE is deemed to meet the requirement if the reported differential L1-SINR is in the range shown in Table A.7.7.6.X.3-2.

Table A.7.7.6.X.3-1: L1-SINR absolute accuracy test requirement

|  | Test requirement Notes1,2 |
| --- | --- |
| SSB#0+CSI-IM#0 | L1_SINR0 -δ ≤ Reported SINR(dB) ≤ L1_SINR0 +δ |
| Note 1: L1_SINR0 is the equivalent SINR received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the SSB#0+CSI-IM#0 under consideration Note 2: δ is the SINR absolute accuracy requirement from Table 10.1.28.2.1-2, selected according to the Io used in the test |  |

Table A.7.7.6.X.3-2: L1-SINR relative accuracy test requirement

|  | Test requirement Notes1,2 |
| --- | --- |
| SSB#0+CSI-IM#0 - SSB#1+CSI-IM#1 | L1-SINR0 - L1-SINR1 - σ≤ Reported DIFFSINR (dB) ≤ L1-SINR0 - L1-SINR1 + σ |
| Note 1: L1-SINRn is the equivalent SINR received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the SSB#n+CSI-IM#n under consideration Note 2: σ is the SINR relative requirement from Table 10.1.28.2.2-1, selected according to the Io used in the test |  |

<End of Change #2>