Me?3GPP TSG-RAN WG4 Meeting #107	R4-2308923

Incheon, Republic of Korea, 22-26 May, 2023

Source:	Ericsson

Title:	On timing requirements for ATG

Agenda item:	8.14.4.3

Document for:	Approval

# 1	Introduction

In WF from previous meeting [1]:

… 3 Timing and frequency adjustment 3.1 Timing requirements Issue 3-1-1: How to acquire reference location information (e.g. BS location information) Agreements: - Wait the feedback from RAN2 Issue 3-1-2: GNSS error assumption for initial transmit timing requirements Te_ATG Webinar offline Agreements: - 40m for GNSS accuracy assumption - Do not define 60KHz UL Te requirements Issue 3-1-3: Initial transmit timing requirements Te_ATG Agreements: - Exclude the 60 kHz SCS - Te, GNSS_error = 8x64xTc Issue 3-1-4: Gradual timing adjustment Agreements: - Tq_ATG and Tp_ATG should be 9.5*64*Tc in FR1 and all SCSs 3.2 Guard period as Large TDD cell impaction Issue 3-2-1: Whether co-existence and overhead issues due to large GP need to be addressed in R18 ATG - Option 1: - Large TDD cells are inefficient due to large GP, if we switch often. - The largest cell radius will define the GP for all TDD cells and UL/DL configuration has to be the same for all coexisting (overlapping) TDD networks - Option 2: How to configure a suitable GP is up to network. There is no need to discuss it. Issue 3-2-2: How to address the large guard period issue for ATG - Option 1: Utilize the LMF function for estimating the position of the aircraft and enable the calculation of the optimum gap period at the network side is preferred in relation to the uplink-TA-reporting. - Option 2: ATG UE reports the propagation delay information to ATG gNB, so that DL or UL scheduling restriction can be used by ATG gNB based on the information - Option 3: How to configure a suitable GP is up to network. There is no need to discuss it. …

And in LS in to RAN4 [2]:

… 1. Overall Description: RAN2 would like to thank RAN4 for the LS on the applicability of the SIB19 for ATG. RAN2 understands the intention is that ATG BS provides location information to assist UE-based time and/or frequency pre-compensation. RAN2 will address this and intends to find a solution for Rel-18 (via SIB19 or other SIB). Some companies in RAN2 think sharing exact base station’s location is sensitive. RAN2 kindly asks RAN4 to provide information on any required ATG ground station reference location accuracy (i.e., uncertainty in exact base station location). 2. Actions: To RAN4 group. ACTION: RAN2 kindly asks RAN4 to provide information on any required ATG ground station reference location accuracy. 3. Date of Next RAN2 Meetings: TSG-RAN WG2#122 May 22nd – 26th, 2023 Incheon, South Korea …

# 2	Discussion

## 2.1 	Issue 3-2-2: How to address the large guard period issue for ATG

Both options 1 and 2 are fine for us:

- Option 1: Utilize the LMF function for estimating the position of the aircraft and enable the calculation of the optimum gap period at the network side is preferred in relation to the uplink-TA-reporting.

- Option 2: ATG UE reports the propagation delay information to ATG gNB, so that DL or UL scheduling restriction can be used by ATG gNB based on the information.

Proposal 1: 	Estimating the position of the aircraft and enable the calculation of the optimum gap period or let  UE reports the propagation delay information to ATG gNB, so that DL or UL scheduling restriction can be used by ATG gNB based on the information.

An alternative TDD pattern would constitute very severe deployment constraints [3] and latency issues, so we need options 1 and 2, in places where an alternative TDD pattern is not possible to deploy.

Observation 1: 		An alternative TDD pattern would constitute very severe deployment constraints [ref Adrian] and latency issues, so we need options 1 and 2, in places where an alternative TDD pattern is not possible to deploy.

## 2.2 	LS reply

RAN2 asks RAN4:

RAN2 kindly asks RAN4 to provide information on any required ATG ground station reference location accuracy (i.e., uncertainty in exact base station location).

One way to derive an answer is to combine already made agreements in WF [1] with the existing NTN 3GPP Rel-17 NTN feature which Ericsson prefers as an ATG baseline for Release-18 ATG.

In RAN4#106bis it was agreed to use 40 meters for ATG UE GNSS error:

Issue 3-1-2: GNSS error assumption for initial transmit timing requirements Te_ATG

Webinar offline Agreements:

- 40m for GNSS accuracy assumption

- Do not define 60KHz UL Te requirements

In NTN Rel-17 we assumed 50 m for UE GNSS error and 30 m for satellite error. This makes the total error for UE to satellite link (NTN service link) equal to 50 + 30 m = 80 m.

Based on this the strictest bound for tolerable error is 80 – 40 m = 40 m, for a UE in dedicated state, with ongoing connection, after random access and call established.

Proposal 2: 	The strictest bound for tolerable error of BS position is 80 – 40 m = 40 m for an ongoing connection.

However, It is quite possible that the closed loop TA control can handle bigger position offsets, even if there could be some initial BS RX interference after first random access. If the BS position error is bigger than 40 meters, then two different ATG UL to BS links could interfere at BS RX, but if the link does not break then close loop TA commands can skew timing to optimum again, after initial connection is established. This could justify to allow a bigger a bigger BS position error, in case a very precise BS position requirement is not desirable for reasons of operator network confidentiality.

Observation 2: 	Final BS position error is a function of tolerable error that BS RX can correct with closed loop TA commands. This could justify a bigger possible BS position error compared to what strict steady state analysis gives for an ongoing connection.

# 3	Summary

Proposal 1: 	Estimating the position of the aircraft and enable the calculation of the optimum gap period or let  UE reports the propagation delay information to ATG gNB, so that DL or UL scheduling restriction can be used by ATG gNB based on the information.

Observation 1: 	An alternative TDD pattern would constitute very severe deployment constraints [ref Adrian] and latency issues, so we need options 1 and 2, in places where an alternative TDD pattern is not possible to deploy.

Proposal 2: 	Upper bound for tolerable error of BS position is 80 – 40 m = 40 m.

Observation 2: 	Final BS position error is a function of tolerable error that BS RX can correct with closed loop TA commands. This could justify a bigger possible BS position error compared to what strict steady state analysis gives for an ongoing connection.

# References

- R4-2306344, WF on NR ATG RRM requirements, CMCC

- R2-2304565, Reply LS on applicability of SIB19 for NR ATG, Qualcomm

- R4-2308533, Co-existence simulation for unsynchronized scenarios, Ericsson