3GPP TSG RAN WG4 Meeting #107		 R4-2308927

# Incheon, KR, May 22 – May 26, 2023

Agenda Item:	7.27.2

Source: 	MediaTek Inc.

Title: 	Discussion simultaneous Rx/Tx inter-band combinations for CA_n40-n41

Document for:	Discussion

1. Introduction

In RAN4 #106-bis-e, simultaneous Rx/Tx operation for CA_n40-n41 was extensively discussed and WF[1] had been agreed. The contribution provides continue discussion on the remaining requirements for simultaneous Rx/Tx operation for CA_n40-n41

2. Discussion

In the WF[1] agreed in RAN4#106-bis-e, the issue on simultaneous Rx-Tx and MIMO/SRS-AS for CA_n40-n41 were left for further discussion:

- Proposal 1: Discuss the following options and implications for simultaneous Rx-Tx and MIMO with CA_n40-n41:

    - Option 1b: A new optional signalling could be introduced to indicate that a UE supporting MIMO and simultaneous Rx/Tx cannot support both features at the same time for a specific combination.

    - Option 1c: Relax the 4Rx requirement of n41 if carrier aggregation is configured with band n40 and simultaneous Rx-Tx. This relaxation would only be applicable to handheld devices and allow improved performance of simultaneous Rx-Tx together with MIMO operation on band n40 and n41.

- Proposal 2 (MediaTek): For CA_n40A-n41A, UE is not expected to enable UL MIMO/TxD or SRS antenna switching at the same time when enabling simultaneous Rx/Tx operation

- Proposal 3: Extend the scope of LS [1] to ask clarifications on SRS-AS scheduling restrictions or even on disabling SRS-AS [3] to TDD-TDD band combinations (e.g., CA_n40-n41) with simultaneous RxTx operation.

It has been discussed UE implementation concerns on antenna configurations. Due to limited space for antenna design, there’s trade off that not all antennas can support full frequency range of all supported bands. In general, there’s less margin on transmit TRP requirement then receiving TIS requirements. UE may configure sharing antenna architecture on CA_n40-n41 if simultaneous RxTx is not required. When the simultaneous RxTx is required for the combo, it may require separate-antenna architecture to fulfil requirements that six high-band antennas would be needed.

Observation 1: UR RF front-end architecture would be different if supporting simultaneous RxTx capability on TDD-TDD CA where the TDD bands are in close-in frequency group that were usually implemented with sharing antenna architecture

Observation 2: The agreed TP in [2] adopts separate antenna architecture for MSD characterization.

Observation 3: It would require more quantity of antennas to support such TDD-TDD CA with simultaneous RxTx capability

Relax the 4Rx requirement to 2Rx requirement of n41 if carrier aggregation is configured with band n40 and simultaneous Rx-Tx is also helpful to address UE implementation concerns. In our view option 1c could be agreeable.

Proposal 1: Relax the 4Rx requirement to 2Rx requirement of n41 if carrier aggregation is configured with band n40 and simultaneous Rx-Tx. This relaxation would only be applicable to handheld devices

Proposal 2: Signalling indication on supporting simultaneous RxTx for the combo can be considered. The indication implicitly implies 2-Rx requirement is allowed.

There may also be some other features such as UL MIMO/TxD, SRS antenna switching and Tx switching need to be supported simultaneously. In our initial estimation, we found it is difficult to have all antennas to meet design requirements on both transmission and receiving of all the supporting bands and MRDC/NRDC combinations fulfil multiple UE capability at the same time. Relaxation on the quantity of antennas would be a feasible approach. Referring existing signalling on SRS antenna switching:

| srs-TxSwitch, srs-TxSwitch-v1610 Defines whether UE supports SRS for DL CSI acquisition as defined in clause 6.2.1.2 of TS 38.214 [12]. The capability signalling comprises of the following parameters: - supportedSRS-TxPortSwitch indicates SRS Tx port switching pattern supported by the UE, which is mandatory with capability signaling. The indicated UE antenna switching capability of ′xTyR′ corresponds to a UE, capable of SRS transmission on ′x′ antenna ports over total of ′y′ antennas, where ′y′ corresponds to all or subset of UE receive antennas, where 2T4R is two pairs of antennas. supportedSRS-TxPortSwitch-v1610, which is optional to report, indicates downgrading configuration of SRS Tx port switching pattern. If the UE indicates the support of downgrading configuration of SRS Tx port switching pattern using supportedSRS-TxPortSwitch-v1610, the UE shall report the values for this as below, based on what is reported in supportedSRS-TxPortSwitch. supportedSRS-TxPortSwitch; supportedSRS-TxPortSwitch-v1610. t1r2; t1r1-t1r2. t1r4; t1r1-t1r2-t1r4. t2r4; t1r1-t1r2-t2r2-t2r4. t2r2; t1r1-t2r2. t4r4; t1r1-t2r2-t4r4. t1r4-t2r4; t1r1-t1r2-t2r2-t1r4-t2r4. - txSwitchImpactToRx indicates the entry number of the first-listed band with UL (see NOTE) in the band combination that affects this DL, which is mandatory with capability signaling; - txSwitchWithAnotherBand indicates the entry number of the first-listed band with UL (see NOTE) in the band combination that switches together with this UL, which is mandatory with capability signaling. For txSwitchImpactToRx and txSwitchWithAnotherBand, value 1 means first entry, value 2 means second entry and so on. All DL and UL that switch together indicate the same entry number. The entry number is the band entry number in a band combination. The UE is restricted not to include fallback band combinations for the purpose of indicating different SRS antenna switching capabilities. NOTE: The first-listed band with UL includes a band associated with FeatureSetUplinkId set to 0 corresponding to the support of SRS-SwitchingTimeNR. | BC | FD | N/A | N/A |
| --- | --- | --- | --- | --- |

UE may still signal its supporting on SRS antenna switching configurations: t1r1-t1r2-t2r2 with the proposal 1 and 2.

Observation 4: UE may be able to support some configurations of SRS-AS while the number of Rx can be relaxed.

3. Conclusion

Observation 1: UR RF front-end architecture would be different if supporting simultaneous RxTx capability on TDD-TDD CA where the TDD bands are in close-in frequency group that were usually implemented with sharing antenna architecture

Observation 2: The agreed TP in [2] adopts separate antenna architecture for MSD characterization.

Observation 3: It would require more quantity of antennas to support such TDD-TDD CA with simultaneous RxTx capability

Proposal 1: Relax the 4Rx requirement to 2Rx requirement of n41 if carrier aggregation is configured with band n40 and simultaneous Rx-Tx. This relaxation would only be applicable to handheld devices

Proposal 2: Signalling indication on supporting simultaneous RxTx for the combo can be considered. The indication implicitly implies 2-Rx requirement is allowed.

Observation 4: UE may be able to support some configurations of SRS-AS while the number of Rx can be relaxed.

Reference:

- R4-2306547 WF on simultaneous Rx/Tx transmission, RAN4#106-bis-e

- R4-2306552 TP for TR 38.894: CA_n40A-n41A, RAN4#106-bis-e

- RP-223354 WID revision: Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18 RAN-P#98-e