3GPP TSG-RAN WG4 Meeting # 107	R4-2308937

Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.5.3.1.2

Source:	Ericsson

Title:	Remaining issues on PDSCH requirements for 8 Rx in FR1

Document for:	Discussion

# 1	Introduction

Last meeting #106bis-e, further progress has been achieved in this WI. In this meeting, we go through the remaining issues that still need to be addressed as stated in [1]. Below, we summarize the remaining open issues on the PDSCH demodulation requirements’ definition.

Issue 2-1: Maximum number of OFDM symbols (maxLength) for DL - Single-symbol for Rank not larger than 4, double-symbol for Rank larger than 4. Issue 2-2: Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx - Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 Issue 2-3: Propagation conditions for Rank 2 test - Option 1: TDLC300-100 ULA Medium B and TDLA30-10 ULA Medium B - Option 2: Only TDLC300-100 ULA Medium B Issue 2-4: MCS and antenna correlation for Rank 2 test - Proposals for TDLC300-100 ULA Medium B - Option 1: MCS 2, 7 (Table 2) - Option 2: MCS 19 (Table 1) - Proposals for TDLA30-10 ULA Medium B (if agreed) - Option 1: MCS 13, 26 (Table 2) - Option 2: MCS 19 (Table 1) Issue 2-5: MCS for Rank 4 test - Option 1: MCS 13 and MCS26 (Table 1) - Option 2: Only MCS17 (Table 1) Issue 2-6: N1 and N2 configuration for 8Tx case - Use (N1,N2) = (4,1), (O1, O2) = (4,1) Issue 2-7: Single MCS configuration for Rank 8 test - Option 1: MCS13 - Option 2: MCS17 Issue 2-8: Additional test for two MCS configuration for Rank 8 test Don’t introduce rank8 test with two different MCS

# 2	PDSCH remaining issues

The parameters specified in TS38.101-4 Table 5.2-1 [2] are valid for all PDSCH tests unless otherwise stated. Furthermore, some specific 8 Rx UE parameters have been agreed and summarized in the Appendix. The remaining issues for PDSCH reduce to the highest achievable MCS for Rank 2, 4 and 8.

Observation 1: Referring to our simulation results in R4-2308938 [3], we note that 8 Rx UE can support 8 DL MIMO Layers in FR1 using MCS17 (Table 1) under TDLA30-10 propagation environment, where an SNR of 19.3 dB is required to achieve 70% of the peat throughput. This requirement remains 5 dB below the maximum SNR levels for conducted requirements.

For 8 Rx UE, Proposal 1 is provided for PDSCH mapping Type A test cases with Rank 8.

Proposal 1: Consider PDSCH mapping Type A for 8 Rx UE and Rank 8 with the following test cases and parameters

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB)/ | TDD UL-DL pattern | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 (106) | FR1.30-1 | QPSK, 0.30 MCS 4 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-2 | 40 / 30 (106) | FR1.30-1 | 16QAM, 0.48 MCS 13 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-3 | 40 / 30 (106) | FR1.30-1 | 64QAM, 0.43 MCS 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM |  |  |  |  |  |  |  |  |

Observation 2: Referring to our simulation results in R4-2308938 [3], we note that 8 Rx UE can support 4 DL MIMO Layers in FR1 using MCS26 (Table 1) under TDLA30-10 propagation environment, where an SNR of 18.5 dB is required to achieve 70% of the peat throughput. This requirement remains 6 dB below the maximum SNR levels for conducted requirements.

For 8 Rx UE, Proposal 2 is provided for PDSCH mapping Type A test cases with Rank 4.

Proposal 2: Consider PDSCH mapping Type A for 8 Rx UE and Rank 4 with the following test cases and parameters

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | TDD UL-DL pattern | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | 40 / 30 (106) | FR1.30-1 | QPSK, 0.30 MCS 4 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |
| 2-2 | 40 / 30 (106) | FR1.30-1 | 16QAM, 0.48 MCS 13 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |
| 2-3 | 40 / 30 (106) | FR1.30-1 | 64QAM, 0.85 MCS 26 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM |  |  |  |  |  |  |  |  |

For Rank 2, the discussions have not been limited to the highest achievable MCS, but also which channel model to consider, whether to consider both TDLC and TDLA or TDLA only.

Our view is to consider both channel models: TDLC for up to 16QAM and TDLA for higher modulation order. Furthermore, 256QAM has been used in Table 5.2.3.2.1-3 [2] for 4 Rx UE with Rank 1. We do believe, and based on our simulation results for 8 Rx UE, 256QAM can also be considered for Rank 2 in 2x8 ULA Medium B channels.

Observation 3: It is worth reminding that 256QAM has been used in Table 5.2.3.2.1-3 (TS 38.101-4) for 4 Rx UE with Rank 1. Consequently, and based on our simulation results in R4-2308938 [3], 256QAM can be considered for 8 RX UE with Rank 2 in 2x8 ULA Medium B channels.

Observation 4: Considering 256QAM for 8 Rx UE with Rank 2, our simulation results in R4-2308938 [3] show that MCS26 (Table 2) can be achieved in TDLA30-10 ULA Medium B channel, where an SNR of 23.5 dB is required to achieve 70% of the peak throughput. This requirement remains 6.5 dB below the maximum SNR levels for conducted requirements.

For 8 Rx UE, Proposal 3 is provided as our strong preference for PDSCH mapping Type A test cases with Rank 2.

Proposal 3: Consider PDSCH mapping Type A for 8 Rx UE and Rank 2 with the following test cases and parameters

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | TDD UL-DL pattern | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | 40 / 30 (106) | FR1.30-1 | QPSK, 0.30 MCS 2 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| 3-2 | 40 / 30 (106) | FR1.30-1 | 16QAM, 0.48 MCS 7 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| 3-3 | 40 / 30 (106) | FR1.30-1 | 64QAM, 0.56 MCS 13 | 2 | TDLA30-10 | 2x8, ULA Medium B | 70 | TBD |
| 3-4 | 40 / 30 (106) | FR1.30-1 | 256QAM, 0.89 MCS 26 | 2 | TDLA30-10 | 2x8, ULA Medium B | 70 | TBD |
| MCS were defined based on TS 38.214 Table 5.1.3.1-2: MCS index table 2 for PDSCH (256QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM Use Tx EVM 3% for 256QAM. |  |  |  |  |  |  |  |  |

In case of only TDLC channel mode will be considered for 8 Rx UE with Rank 2, we have carried out simulations and based on the results in R4-2308938, we provide an alternative in Proposal 4.

Observation 5: Considering 256QAM for 8 Rx UE with Rank 2, our simulation results in R4-2308938 [3] show that MCS20 (Table 2) can be achieved in TDLC300-100 ULA Medium B channel, where an SNR of 18.5 dB is required to achieve 70% of the peak throughput. This requirement remains 11.5 dB below the maximum SNR levels for conducted requirements.

Proposal 4: If only TDLC channel model will be considered for 8 Rx UE with Rank 2, consider the following test cases and parameters

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | TDD UL-DL pattern | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | 40 / 30 (106) | FR1.30-1 | QPSK, 0.30 MCS 2 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| 3-2 | 40 / 30 (106) | FR1.30-1 | 16QAM, 0.48 MCS 7 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| 3-3 | 40 / 30 (106) | FR1.30-1 | 64QAM, 0.56 MCS 13 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| 3-4 | 40 / 30 (106) | FR1.30-1 | 256QAM, 0.67 MCS 20 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| MCS were defined based on TS 38.214 Table 5.1.3.1-2: MCS index table 2 for PDSCH (256QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM Use Tx EVM 3% for 256QAM. |  |  |  |  |  |  |  |  |

# 3	Summary

In this paper we tried to contribute with the following proposals for further discussions:

Observation 1: Referring to our simulation results in R4-2308938 [3], we note that 8 Rx UE can support 8 DL MIMO Layers in FR1 using MCS17 (Table 1) under TDLA30-10 propagation environment, where an SNR of 19.3 dB is required to achieve 70% of the peat throughput. This requirement remains 5 dB below the maximum SNR levels for conducted requirements.

Proposal 1: Consider PDSCH mapping Type A for 8 Rx UE and Rank 8 with the following test cases and parameters

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB)/ | TDD UL-DL pattern | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 (106) | FR1.30-1 | QPSK, 0.30 MCS 4 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-2 | 40 / 30 (106) | FR1.30-1 | 16QAM, 0.48 MCS 13 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-3 | 40 / 30 (106) | FR1.30-1 | 64QAM, 0.43 MCS 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM |  |  |  |  |  |  |  |  |

Observation 2: Referring to our simulation results in R4-2308938 [3], we note that 8 Rx UE can support 4 DL MIMO Layers in FR1 using MCS26 (Table 1) under TDLA30-10 propagation environment, where an SNR of 18.5 dB is required to achieve 70% of the peat throughput. This requirement remains 6 dB below the maximum SNR levels for conducted requirements.

Proposal 2: Consider PDSCH mapping Type A for 8 Rx UE and Rank 4 with the following test cases and parameters

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | TDD UL-DL pattern | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | 40 / 30 (106) | FR1.30-1 | QPSK, 0.30 MCS 4 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |
| 2-2 | 40 / 30 (106) | FR1.30-1 | 16QAM, 0.48 MCS 13 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |
| 2-3 | 40 / 30 (106) | FR1.30-1 | 64QAM, 0.85 MCS 26 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM |  |  |  |  |  |  |  |  |

Observation 3: It is worth reminding that 256QAM has been used in Table 5.2.3.2.1-3 (TS 38.101-4) for 4 Rx UE with Rank 1. Consequently, and based on our simulation results in R4-2308938 [3], 256QAM can be considered for 8 RX UE with Rank 2 in 2x8 ULA Medium B channels.

Observation 4: Considering 256QAM for 8 Rx UE with Rank 2, our simulation results in R4-2308938 [3] show that MCS26 (Table 2) can be achieved in TDLA30-10 ULA Medium B channel, where an SNR of 23.5 dB is required to achieve 70% of the peak throughput. This requirement remains 6.5 dB below the maximum SNR levels for conducted requirements.

Proposal 3: Consider PDSCH mapping Type A for 8 Rx UE and Rank 2 with the following test cases and parameters

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | TDD UL-DL pattern | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | 40 / 30 (106) | FR1.30-1 | QPSK, 0.30 MCS 2 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| 3-2 | 40 / 30 (106) | FR1.30-1 | 16QAM, 0.48 MCS 7 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| 3-3 | 40 / 30 (106) | FR1.30-1 | 64QAM, 0.56 MCS 13 | 2 | TDLA30-10 | 2x8, ULA Medium B | 70 | TBD |
| 3-4 | 40 / 30 (106) | FR1.30-1 | 256QAM, 0.89 MCS 26 | 2 | TDLA30-10 | 2x8, ULA Medium B | 70 | TBD |
| MCS were defined based on TS 38.214 Table 5.1.3.1-2: MCS index table 2 for PDSCH (256QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM Use Tx EVM 3% for 256QAM. |  |  |  |  |  |  |  |  |

Observation 5: Considering 256QAM for 8 Rx UE with Rank 2, our simulation results in R4-2308938 [3] show that MCS20 (Table 2) can be achieved in TDLC300-100 ULA Medium B channel, where an SNR of 18.5 dB is required to achieve 70% of the peak throughput. This requirement remains 11.5 dB below the maximum SNR levels for conducted requirements.

Proposal 4: If only TDLC channel model will be considered for 8 Rx UE with Rank 2, consider the following test cases and parameters

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | TDD UL-DL pattern | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | 40 / 30 (106) | FR1.30-1 | QPSK, 0.30 MCS 2 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| 3-2 | 40 / 30 (106) | FR1.30-1 | 16QAM, 0.48 MCS 7 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| 3-3 | 40 / 30 (106) | FR1.30-1 | 64QAM, 0.56 MCS 13 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| 3-4 | 40 / 30 (106) | FR1.30-1 | 256QAM, 0.67 MCS 20 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| MCS were defined based on TS 38.214 Table 5.1.3.1-2: MCS index table 2 for PDSCH (256QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM Use Tx EVM 3% for 256QAM. |  |  |  |  |  |  |  |  |

# References

- R4-2305888, “WF for 8Rx UE performance requirements”, Huawei.

- 3GPP TS 38.101-4, “NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”, v.17.8.0.

- R4-2308938, “Simulation results for 8 Rx PDSCH requirements in FR1”, Ericsson.

# Appendix	Simulation assumption

## A.1	PDSCH Simulation Parameters

| Assumptions | Value |
| --- | --- |
| Duplex mode | TDD |
| TDD UL-DL pattern | 7D1S2U with S=6D:4G:4U |
| Carrier Frequency [GHz] | 3.5 GHz |
| Subcarrier Spacing [KHz] | 30 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (30 ns delay spread) and TDLC (300 ns delay spread) |
| Antenna configuration | 2x8 ULA Medium B, 4x8 and 8x8 ULA Low |
| Velocity | 3 km/h (Doppler spread = 10 Hz), 30 km/h (Doppler spread = 100 Hz) |
| PA Model | None |
| Pre-loaded Tx EVM | 6% (QPSK/16QAM/64QAM) and 3% (256QAM) |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 2, Rank 4 and Rank 8 |
| DMRS Configuration | 2+2 Type 1 DMRS symbols at (2,3,10,11) symbol indexes for Rank 8 1+1 Type 1 DMRS symbols at (2, 10) symbol indexes for Rank 2 and Rank 4 |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 20 (64QAM). From MCS Table 2 (TS38.214): Up to MCS 20 (256QAM) |
| NOH | 0 |
| Number of HARQ Processes | 8 |
| Maximum HARQ transmissions | 4 |

| Parameter |  | Value |
| --- | --- | --- |
| SCS (kHz)/Bandwidth (MHz) |  | 30/40 |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 for Rank ≤ 4 2 for Rank > 4 |