3GPP TSG-RAN WG4 Meeting # 107										 R4-2309003

Incheon, KR, May 22 – May 26, 2023

Title: 	Discussion on UL Tx switching

Source: 	Xiaomi

Agenda item:	8.24.2.1

Document for:	Approval

- Introduction

In RAN4#106bis-e meeting, some agreements are achieved on UL Tx switching with single TAG [1] and an LS [2] was sent to RAN1/2. This contribution will provide our view on the remaining issues.

- Discussion

### Sub-topic 1-1: Exact location of switching period Way forward after round 2 discussion: - For Rel-18 Tx switching, further discuss the following options taking into account RAN1 response: - Option 1: the switching period immediately precedes the time T0 at which a transmission starts on a carrier following a Tx switch from a preceding transmission - Option 1a: the switching period ends at time T0 - Option 2: define the time relation of the switching period location and T0 for two cases with the switching period located on the switch-from carrier and switch-to carrier respectively. - The switching period location is determined based on the following RAN1 agreement: The gNB configures priority for each band. The UE determines the switching period location on either switching-from band(s) or switching-to band(s) that is involved in the UL Tx switching and is not with the highest priority band. - Option 3: No need to define the time relationship between the location of the switching period and T_0 in RAN4 specification

RAN4 has discussed the switching period location for several meetings. In the last meeting, companies have different understanding on T0 defined by RAN1 and an LS was sent to RAN1 to ask for clarification. In our understanding, T0 is the starting time for an uplink transmission on the switched-to carrier from network scheduling perspective. So from network perspective, sufficient UE preparation time for UL can be guaranteed. T0 may or may not be the same as the starting time of actual uplink transmission, which depends on the following two aspects:

- The gap between the last OFDM symbol of UL transmission on switched-from carrier and the first OFDM symbol of UL transmission on switch-to carrier.

- Located carrier of switching period according to RRC signalling uplinkTxSwitchingPeriodLocation.

Therefore, our understanding is that when gNB scheduling does not provide sufficient time between the last OFDM symbol of UL transmission on switch-from carrier and the first OFDM symbol of UL transmission on switch-to carrier, and the switching period is located at the switch-to carrier according to the RRC signalling uplinkTxSwitchingPeriodLocation, T0 is possible to be located preceding the switching period. There is no fixed location relationship between T0 and switching period.

Considering the different cases involved, it is expected to not restrict the location relationship between switching period and T0 in time mask requirements. The time location of switching period in time domain can be up to UE implementation. The flexibility of UE implementation should not be impacted.

Proposal 1: No need to define the time relationship between the location of the switching period and T0 in RAN4 specification

#### Issue 1-3-2: Advanced optional UE ability - Further discuss the following options in the next meeting - Option 1: For the switching scenario without ambiguity, i.e., the two Tx chains are on the same band before or after switching (switch between {1T, 1T, 0T} and {0T, 0T, 2T}), introduce advanced optional UE ability to allow the Tx chain #1 to be used for transmission during the time duration of (Tswitch_2 - Tswitch_1). - Tswitch_1 and Tswitch_2 are the switching periods of Tx chain #1 and Tx chain #2 respectively, and Tswitch_1 < Tswitch_2. - Option 1a: apply the same UE capability for this scenario and for the following scenario agreed in the previous meetings. (MTK, China Telecom, ZTE) RAN4 LS approved in R4-2303507: When one of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), the other Tx chain is maintained on a different band (named “band C” or “band D” in the case of 4-band) and the number of Tx chain on band C or band D is unchanged due to the switching, RAN4 agreed the granularity of the optional UE capability to allow UL transmission on the band with the number of Tx chain unchanged during UL switching as follows: - Per band (only for the band(s) in the band combination but not included in the pair of bands before and after switching) for each pair of bands before and after switching in each band combination. - Option 2: Not introduce advanced optional UE ability

Based on the previous agreement on three band case, i.e., A->B and C->C, it is feasible to introduce advance optional UE capability for the switching case between {1T, 1T, 0T} and {0T, 0T, 2T}.

#### Issue 1-4-1: Switching case across four bands, i.e., {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} Way forward after round 2 discussion: - Further discuss in the next meeting: - Option A: As optional UE behaviour, total switching period can be extended if UE is not capable for concurrent TX switching on the two TX chains. - Option A1: add new values {70, 175} usec in addition to the agreed set of {35 us, 140 us, 210 us} - Option A2: Sum of two switching periods - When UE is scheduled to switch two TX chains in such way that switching periods may overlap, the switching period is extended for both band pairs and total switching time is the sum of possible switching periods for the band pairs involved. - When UE is scheduled for transmissions so that the switching is from two bands with one TX each to another two bands one TX each, denoted for example A+B to C+D, and it cannot be determined if UE switches TX chains from A to C or D or from B to C or D, the switching time is sum of max{Tswitch_A-C, Tswitch_A-D,} and max{Tswitch_B-C, Tswitch_B-D}. - Include clearly the aspect that when two TX chains are switched with different lengths of the switching periods, none of the TX chains are expected to be used for transmissions. - Option B: Keep the previous agreements #### Issue 1-4-2: Switching case across three bands, i.e., {1T, 1T, 0T} to {0T, 0T, 2T} Way forward after round 2 discussion: - Further discuss in the next meeting: - Option A: As optional UE behaviour, total switching period can be extended if UE is not capable for concurrent TX switching on the two TX chains. - Option A1: add new values {70, 175} usec in addition to the agreed set of {35 us, 140 us, 210 us} - Option A2: Sum of two switching periods - When UE is scheduled to switch two TX chains in such way that switching periods may overlap, the switching period is extended for both band pairs and total switching time is the sum of possible switching periods for the band pairs involved. - Option B: Keep the previous agreements

RAN1 has provide response on RAN4’s question in [3] as below:

Agreement: In the reply LS to RAN4, RAN1 confirms following three example scenarios related to RAN4 question are supported from RAN1 perspective. - Example #1: In the case of 3-band Tx switching, the switching is performed from 1T+1T on band A and B to 2T on band C. - Example #2: In the case of 4-band Tx switching, the switching is performed from 1T+1T on band A and B to 1T+1T on band C and D. - Example #3: In the case of 3-band Tx switching, the switching is performed from 2T on band A to 1T+1T on band B and C. Agreement: In the reply LS to RAN4, the answer to RAN4 question is as follows: - RAN1 confirms that it is possible that the two Tx chains are switched concurrently between two different band pairs for one Tx switching instance during a single switching gap for the following three examples. - Example #1: In the case of 3-band Tx switching, the switching is performed from 1T+1T on band A and B to 2T on band C. - Example #2: In the case of 4-band Tx switching, the switching is performed from 1T+1T on band A and B to 1T+1T on band C and D. - Example #3: In the case of 3-band Tx switching, the switching is performed from 2T on band A to 1T+1T on band B and C. - It is RAN1 understanding that the switching of two Tx chains between two different band pairs can be performed for one Tx switching instance during a single determined switching gap derived by switching periods for different band pairs reported by UE according to RAN4 LS [R1-2300029/R4-2220548]. - Whether two Tx chains are switched simultaneously or sequentially for one Tx switching instance during the single determined switching gap is up to RAN4. RAN1 is still discussing some details on example scenarios, including under which conditions one Tx switching instance is used for Tx switching of two Tx chains.

It is observed from RAN1 response, a single determined switching gap derived by switching period of different band pairs by UE can be used for one Tx switching instance. RAN1 thinks whether two Tx chains are switched simultaneously or sequentially for one Tx switching instance during the single determined switching gap is up to RAN4. So sequential switching is possible between two Tx chains.

Figure 1 and Figure 2 illustrates simultaneous or sequential switching cases that can be considered as one Tx switching instance. The controversial issue in RAN4 is how to define a single determined switching gap for such new cases.

Figure 1: One switching period contained by another

Figure 2: Switching period overlapped but not starting at the same time

Regarding option A1, two new values {70, 175} usec are added in addition to the agreed set of {35 us, 140 us, 210 us}. It is noted that both two new value is not larger than the largest agreed value, i.e., 210us. If one of the switching periods of sequential switching on two Tx chain is 210us, a larger value than 210us is expected as the total switching period. So adding new values that are shorter than 210us seem not appropriate for some cases.

Regarding option A2, it is feasible to use the sum of possible switching periods for sequential switching case. This can be considered as an optional capability for sequential switching case and does not violate the previous agreement.

Proposal 2: Option A2 can be considered for sequential switching case.

### Sub-topic 2-1: RF specification impact Further discuss the following options in the next meeting: - Option 1: time masks are also included also for the dual-TAG case to make clear the following - Option 2: Do not modify the time mask for Tx switching for multiple TAGs. The impact of Tx switching with multiple TAGs can be considered as scheduling restriction. - Option 3: The duration for UE omitting uplink transmission is switching period+1 symbol.

Firstly, the switching period location in time domain for two-TAG case can rely on UE implementation similar as single-TAG case. It is not required to restrict the switch period location based on T0.

Secondly, there is no need to capture TA difference into time mask requirements. As we mentioned previously, TA difference is not a constant value depending on the distance between UE and BS. We think capturing TA difference into time mask will potentially cause more challenge and burden for RAN5 to do the test with a lot of uncertainty. Whether to test all possible cases with different TA differences or select several or a proper TA difference, and how to decide a proper TA difference is not clear or needs to be decided. Based on above, the same principle as Rel-17 Sidelink is preferred for UL Tx switching with multiple TAG.

Proposal 3: Do not modify the time mask for Tx switching for multiple TAGs.

- Conclusion

This contribution gives our views on UL Tx switching for single-TAG and dual-TAG case. The following proposals are derived:

Proposal 1: No need to define the time relationship between the location of the switching period and T0 in RAN4 specification

Proposal 2: Option A2 can be considered for sequential switching case.

Proposal 3: Do not modify the time mask for Tx switching for multiple TAGs.

- Reference

- R4-2306621, WF on Rel-18 Tx switching, China Telecom, RAN4#106bis-e

- R4-2306623, LS on Rel-18 Tx switching, China Telecom, RAN4#106bis-e

- R1-2304219, Reply LS on Rel-18 Multi-carrier enhancement for NR, China Telecom, RAN1#112bis-e