3GPP RAN4 Meeting #107	R4-2309020

# Incheon, Korea, May 22nd – 26th, 2023

Title: 	Sidelink general aspects

Agenda Item:	8.31.1

Source: 	Qualcomm Incorporated

Document for:	Approval

# Introduction

In this document we discuss the specification structure for sidelink in the MPS.

# Specification structure for sidelink

As we know, the 38.101-1 specification uses suffix E for V2X . We also know the term V2X means ‘Vehicle to everything’. It is in fact sidelink and operates PC5. Sidelink and V2X are the same from a core UE requirements standpoint. Given that our view is sidelink requirements that we develop should reside under suffix E.

We show a general outline of the general/transmit requirements section below. Many of the requirements can be re-used from V2X and fit well in the structure. This is not necessarily a full list, but shows the re-use in a general manner.

| Aspect | Clause | Comments |
| --- | --- | --- |
| Specification suffix information | 4.3 | Table 4.3.1 Replace ‘V2X’ in suffix E with ‘sidelink’ |
| Operating bands | 5.2E | Add new sidelink bands |
| Transmitter power for sidelink | 6.2E | General: Add SL-U bands n46/96/102 Max power: Add SL-U power classes MPR and A-MPR: Add as needed. Possibly re-use some of the existing requirements. Configured transmit power: FFS what needs to be done |
| Transmitter power for sidelink: CA Max power, MPR, A-MPR | 6.2E.xA | CA requirements added under suffix A re-using the same structure as shared spectrum |
| Output power dynamics | 6.3E | Re-use as much as possible per RAN4 agreements. FFS if anything needs to change |
| Output power dynamics: CA | 6.3E.xA | FFS is anything needs to change |
| Transmit signal quality | 6.4E | Re-use most of the requirements for SL-U per RAN4 agreements, FFS whether all can be re-used |
| Output RF spectrum emissions | 6.5E | Re-use as much as possible A-MPR NS cases need to be added |
| Time alignment error | 6.6E | FFS whether this can be re-used |

Proposal : Capture the sidelink requirements under suffix E in 38.101.

Proposal: V2X is specific to vehicles, where sidelink is a more general category. Replace the ‘V2X’ term in the specification with ‘sidelink’

# Conclusions

Proposal : Capture the sidelink requirements under suffix E in 38.101.

Proposal: V2X is specific to vehicles, where sidelink is a more general category. Replace the ‘V2X’ term in the specification with ‘sidelink’

# References

- RP-230077, WID revision: NR sidelink evolution, 3GPP TSG RAN Meeting #99

- R4-2305817,Sidelink general aspects