3GPP TSG-RAN WG4 Meeting #107 	R4-2309043

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	BS EMC enhancements

Agenda item:	8.18.2 - BS EMC enhancements

Document for:	Approval

Introduction

This paper includes further study of issues defined in R4-2305912 WF document.

· A clear and detailed explanation of “common HW” with existing terminology, as the condition of EMC enhancement of immunity test. · Continue bringing evidence whether to test only up to two CSs with the largest number of RATs, which include all declared supported RATs, and if all subsets of supported CSs need to be tested. · Investigate whether existing CSs/TCs in TS 37.145-1/-2 can cover the outcome of test simplification for TS 37.114.

Discussion

This paper addresses the three open issues raised in the WF [1] document.

The first issue proposed by WF [1] for further study was related to "common HW". A clear and detailed explanation of "common HW" was requested, using existing terminology, as a condition for EMC immunity test enhancement.

MSR BS with same baseband components for all radio access technologies without any RAT-specific hardware blocks, is subject to EMC immunity test enhancement.

The second issue in WF [1] was to continue bringing evidence whether to test only up to two CSs with the largest number of RATs, which include all declared supported RATs, and if all subsets of supported CSs need to be tested. Overlapping tests could be avoided by testing CS versions including a wide range of RATs. As it is stated in tdoc [2], experience has shown that testing of CS with multi-RAT enables to identify HW-based challenges and therefore covers testing CS with a limited number of RATs or single RATs. 5 different RATs can be covered with 2 CSs and therefore:

It is sufficient to test up to two CSs with the largest number of RATs, which include all declared supported RATs, and all subsets of supported CSs are not required to be tested.

The third issue of WF was to investigate whether existing CSs/TCs in TS 37.145-1/-2 can cover the outcome of test simplification for TS 37.114 [5].

Discussion on previous meetings #106 and #106bis-e have been focused to simplification of MSR BS EMC capability sets. Noticeable related to TS 37.145-1/2 and TS 37.114 specifications is that these documents do not establish minimum RF characteristics or minimum performance requirements for Narrow-Band Internet of Things (NB-IoT) in band, NB-IoT guard band, or standalone NB-IoT operation related to AAS BS [3,4]. Table 1 summarizes capability sets included [3-4,6]

Table 1 Capability sets supported by the MSR BS and AAS BS [3-4, 6]

| Capability set | RATs supported by MSR BS | RATs supported by AAS BS |
| --- | --- | --- |
| CS1 | UTRA (MC) | MSR operation of UTRA only in the band |
| CS2 | E-UTRA (MC) | MSR operation of E-UTRA only in the band |
| CS3 | UTRA, E-UTRA | UTRA and E-UTRA MSR in the band |
| CSA3A | - | NR and E-UTRA MSR in the band |
| CSA3B | - | UTRA, E-UTRA, NR MSR in the band |
| CS4 | GSM, UTRA | single-RAT UTRA in the band |
| CS5 | GSM, E-UTRA | single-RAT UTRA in the band |
| CS6 | GSM, UTRA, E-UTRA | - |
| CS7 | GSM, UTRA, E-UTRA | - |
| CS8 | NB-IoT standalone | - |
| CS9 | GSM, NB-IoT standalone | - |
| CS10 | UTRA, NB-IoT standalone | - |
| CS11 | E-UTRA, NB-IoT standalone | - |
| CS12 | GSM, UTRA, NB-IoT standalone | - |
| CS13 | GSM, E-UTRA, NB-IoT standalone | - |
| CS14 | UTRA, E-UTRA, NB-IoT standalone | - |
| CS15 | GSM, UTRA, E-UTRA, NB-IoT standalone | - |
| CS16 | NR, E-UTRA | - |
| CS17 | NR, E-UTRA, NB-IoT standalone | - |
| CS18 | GSM, E-UTRA, NR | - |
| CS19 | UTRA, E-UTRA, NR | - |

MSR BS specification [6] includes broader variety of capability set than AAS BS specifications [3,4]. Test requirements and Test Tolerances for NB-IoT in-band, NB-IoT guard band, or standalone NB-IoT operation are not supported by AAS BS and not covered by [3-5] specifications.

CSs/TCs in TS 37.145-1/-2 can cover the outcome of test simplification for TS 37.114 related to broad band RATs.

Conclusion

In the paper, the following Observations and Proposals were made:

Proposal 1: MSR BS with same baseband components for all radio access technologies without any RAT-specific hardware blocks, is subject to EMC immunity test enhancement.

Proposal 2: It is sufficient to test up to two CSs with the largest number of RATs, which include all declared supported RATs, and all subsets of supported CSs are not required to be tested.

Observation 1: MSR BS specification [6] includes broader variety of capability set than AAS BS specifications [3,4]. Test requirements and Test Tolerances for NB-IoT in-band, NB-IoT guard band, or standalone NB-IoT operation are not supported by AAS BS and not covered by [3-5] specifications.

Proposal 3: CSs/TCs in TS 37.145-1/-2 can cover the outcome of test simplification for TS 37.114 related to broad band RATs.

- References

- R4- 230591, WF on BS EMC enhancement, Ericsson, RAN4 #106, Electronic Meeting, April 17 – 26, 2023.

- R4-2300443, Analysis of EMC enhancement solutions for BS, Nokia, Nokia Shanghai Bell, RAN4 #106, Athens, Greece, February 27 – March 3, 2023.

- 3GPP TS 37.145-1; Active Antenna System (AAS) Base Station (BS) conformance testing; Part 1: Conducted conformance testing (Release 18)

- 3GPP TS 37.145-2; Active Antenna System (AAS) Base Station (BS) conformance testing; Part 2: Radiated conformance testing (Release 18)

- 3GPP TS 37.114; Active Antenna System (AAS) Base Station (BS) Electromagnetic Compatibility (EMC) (Release 17)

- 3GPP TS 37.141; NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing (Release 18)