3GPP RAN WG4 Meeting #107		R4-2309055

Incheon, South Korea, May 22 - 26, 2023

Agenda item:	8.16.2.1

Source:	Apple

Title:	UL MIMO radiated output power metric and test methodology

Work Item:	NR_FR1_TRP_TRS_enh-Core

Document for:	Approval

# 1	Introduction

The Rel-18 work item on the enhancement of TRP/TRS methodologies and requirements for FR1 includes the following objectives related to TRP of 2Tx UEs [1]:

- Enhancements of TRP TRS test methodology - Specify necessary enhancement of the anechoic-chamber based test methodology (i.e. reference test methodology) to support (test methodology defined in TR 38.834 is the baseline): - UE with NR 2Tx configuration - Case 1: TxD (i.e., TxD capability supported) - Case 2: single layer UL-MIMO (i.e., codebook-based capability supported) - Study proper configuration from UE implementation and test system feasibility perspective - Define test case applicability for case 1 and case 2

RAN4 has devoted a considerable amount of time to discuss the radiated output power test methodology for UL MIMO capable devices.  RAN4 #105 made preliminary agreements [2].  During RAN4 #106 the contributions in [3], [4], [5], [6], and [7] were discussed and resulted in the agreement in [8]:

Issue 1-1-1: Proper TPMI-index for UL-MIMO TRP test Agreement: - FFS whether dynamic TPMI approach can be considered for RAN4 TRP requirements introduction, further discuss the details on dynamic TPMI approach. Issue 1-1-4: Test method for TxD Agreements: - Stick to previous agreement as following: - Enable 2Tx antenna active simultaneously for 2Tx testing as 1st priority. - Sequential 1Tx test and then sum up with FFS data processing approach can be further studied as 2nd priority. Issue 1-1-7: General performance metric for UL-MIMO radiated output power test (new item based on offline feedback) Agreement: - For the UL MIMO radiated output power requirement, RAN4 to further discuss the following metrics: - Option 1: Surface integral of measured EIRP, given fixed TPMI = 2 (NOTE: this metric is TRP-like if normalized by the radiated power of an ideal isotropic radiator) - Option 2: Surface integral of measured EIRP, given TPMI is swept over all applicable TPMI according to the UE capability, and EIRP is selected as the maximum - Option 3: Surface integral of measured EIRP for each TPMI swept over all applicable TPMI according to the UE capability to obtain TRP-like metric for each TPMI and then average the TRP-like metrics - Option 4: Spherical coverage CDF of measured EIRP, given TPMI is swept over all applicable TPMI according to the UE capability, and EIRP is selected as the maximum - Other options are not precluded … # Annex for information: illustration of the swept TPMI approach and possible procedure (Not agreement) The figure below provides an illustration of the swept TPMI approach, which is applicable to Options 2 through 4 in Issue 1-1-7, (NOTE: the figure uses coherent UL MIMO TPMIs as an example): Possible TPMI sweep Test Procedure For the UL MIMO radiated output power test procedure for Options 2 through 4 in Issue 1-1-7 above, current test procedure from TR 38.834 can be re-used with few changes (in red). ### 8.2.3 Test procedure For TRP measurement, the evaluations shall be performed at maximum transmit power. The measurement procedure includes the following steps: 1) Place the DUT inside the QZ following the positioning guideline defined in Clause 6. 2) Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2.1.4.2 of TS 38.521-1 [5] [comment: this reference to TS 38.521-1 needs to be revised] and ensure the DUT transmits with its maximum power. 3) Set the SS to transmit TPMIi=2 . 4) Measure the power, and calculate EIRPTPMIi=2,θ,ϕ by adding the composite loss of the entire transmission path. 5) Repeat steps 3) and 4) for the remaining TPMIi , with i = {3, 4, 5}. 6) Repeat steps 3) to 5) for each measurement point. Option 2 The TRP value is calculated using the TRP integration approaches outlined in Clause 5.1, by taking maxEIRPTPMIi=2,θ,ϕ, EIRPTPMIi=3,θ,ϕ,EIRPTPMIi=4,θ,ϕ,EIRPTPMIi=5,θ,ϕ at each measurement point. Option 3 TRPTPMIi value is calculated for each TPMIi , with i = {2, 3, 4, 5}, using the TRP integration approaches outlined in Clause 5.1 taking EIRPTPMIi,θ,ϕ at each measurement point. Final TRP value is calculated as TRP= 14i=25TRPTPMIi. Option 4 The EIRPtarget-CDF is then obtained from the Cumulative Distribution Function (CDF) computed using maxEIRPTPMIi=2,θ,ϕ, EIRPTPMIi=3,θ,ϕ,EIRPTPMIi=4,θ,ϕ,EIRPTPMIi=5,θ,ϕ for all grid points.

During the RAN4 #106bis meeting contributions [9], [10], [11], [12], [13], and [14] were discussed, with the following agreement captured in the WF [15]:

Issue 2-1: Over-The-Air TRP and TRS definition Agreements: - RAN4 should conclude this issue no later than RAN4#108. Issue 2-2-1: TPMI-index configuration for singe-layer UL-MIMO TRP OTA test Proposals: - Option 1: Surface integral of measured EIRP, given fixed TPMI = 2 (NOTE: this metric is TRP-like if normalized by the radiated power of an ideal isotropic radiator) - Option 2: Surface integral of measured EIRP, given TPMI is swept over all applicable TPMI according to the UE capability, and EIRP is selected as the maximum. - Option 3: Surface integral of measured EIRP for each TPMI swept over all applicable TPMI according to the UE capability to obtain TRP-like metric for each TPMI and then average the TRP-like metrics. - Option 3a: Define TRP for one-layer UL MIMO with TPMI 2-5 as the average of two TRP values with TPMI 2 and 3, or 4 and 5. - Option 4: Spherical coverage CDF of measured EIRP, given TPMI is swept over all applicable TPMI according to the UE capability, and EIRP is selected as the maximum. Agreements: - Down-selection of above option is needed. FFS how to down select - Encourage proponent companies to clarify the performance metric definition for the corresponding option next meeting.

This contribution provides further analysis of the options for the test methodology for radiated power of UL MIMO capable devices and a proposal to make a decision.

# 2	Discussion

In an effort to further explore the feasibility and efficacy of the UL MIMO radiated output metric, we have prepared simulation results with ideal half-wave dipole patterns.  The simulation assumptions are summarized in Table 1 below.

Table 1: Simulation assumptions

| Parameter | Value |
| --- | --- |
| Tx1, Tx2 antenna patterns | Ideal half-wave dipole G = 1.7 dBi |
| Mutual coupling | Not considered |
| Antenna separation | 15 cm |
| Frequency | {1 GHz, …, 6 GHz} |
| MIMO codebooks | Case 1: Coherent MIMO Case 2: Non-coherent MIMO Case 3: Fixed TPMI=2 |
| UL MIMO layers | 1 |
| PA and Tx distortion | Not modeled |
| Phase difference between Tx1 and Tx2 | Not modeled |

The simulation methodology is based on the single azimuth cut approach taken in [3] and extended to the full sphere analysis of coverage [9].  The simulations are normalized to the total power applied to the 2-Tx system:  3+1.7=4.7 dB corresponds to the maximum gain possible for an idealized 0 dBi antenna, where there is constructive superposition in the transmitted signals, and -3+1.7=-1.3 dB corresponds to the single Tx case, where only one antenna transmits using half of the available 2-Tx total power.  Additionally, we would anticipate a result of 0+1.7=1.7 dB to account just for the power combining gain (without any superposition of the patterns).

The total radiated power (TRP) metric is calculated for Case 1 and Case 2 from the envelope of the gain patterns of all applicable TPMI indeces (see the swept TPMI procedure included in Annex of [8]).  For Case 3 TRP is calculated from the spatial response pattern corresponding to TPMI=2.

The results of surface integrals (TRP) for the three cases (coherent MIMO, non-coherent MIMO, and fixed TPMI=2) for 1 GHz and 2 GHz center frequency configurations are shown in Figures 1 and 2 below.

a)  b)  c)

Figure 1: Coverage pattern and TRP at f=1 GHz for a) coherent MIMO codebook; b) non-coherent MIMO codebook; c) fixed TPMI=2 configuration

a) b) c)

Figure 2: Coverage pattern and TRP at f=2 GHz for a) coherent MIMO codebook; b) non-coherent MIMO codebook; c) fixed TPMI=2 configuration

Table 2 below summarizes the results in terms of the TRP calculation for each case:  note that the table includes results for frequencies of 1.0 GHz through 6.0 GHz.

Table 2: Summary of simulation results (TRP metric calculations of coherent/non-coherent/fixed TPMI cases)

| Frequency (GHz) | TRP calculation (dBm) |  |  |
| --- | --- | --- | --- |
|  | Case 1: coherent MIMO | Case 2: non-coherent MIMO | Case 3: fixed TPMI=2 |
| 1.0 | 2.79 | -0.03 | -0.71 |
| 2.0 | 2.79 | 0.57 | 0.16 |
| 3.0 | 2.79 | 0.40 | -0.07 |
| 4.0 | 2.79 | 0.48 | 0.04 |
| 5.0 | 2.79 | 0.43 | -0.03 |
| 6.0 | 2.79 | 0.47 | 0.02 |

Observation 1:	The swept TPMI approach of calculating TRP for the coherent MIMO and non-coherent MIMO codebooks (Cases 1 and 2) demonstrates additional gains due to constructive superposition of signals.

Observation 2:	The case of fixed TPMI=2 exceeds the single antenna radiated output power baseline but falls short of the simple power gain target without pattern superposition.

Observation 3:	The difference between Case 1 and Case 3 of ~[3.5 to 2.6] dB represents the potential underestimation of the UE’s ability to deliver power to the gNB, if a UE capable of coherent MIMO were verified using the fixed TPMI approach.

Observation 4:	A frequency-dependent fluctuation in TRP results is observed for both Case 2 and Case 3.  Since both schemes are suboptimal ways to combine transmitted signals, these results point to the performance gap between coherent MIMO and other schemes.

Considering the outcome of the RAN4 #106bis discussions related to the options, and in particular the proposed metric of spherical coverage (Option 4 in [15]), we have focused this analysis on Option 2.  In this analysis the surface integral of the combined 2Tx radiation pattern was used to propose a potential TRP definition for UL MIMO devices and to illustrate the differences between the three simulation cases of coherent MIMO, non-coherent MIMO, and fixed TPMI=2.  These simulations give strong indications that the fixed TPMI verification approach for radiated output power has the potential to significantly underestimate the UE’s ability to deliver power to the gNB receiver.

Proposal 1:	RAN4 should de-prioritize the fixed TPMI option (Option 1) from further consideration of the radiated output power test method for UL MIMO devices.

Proposal 2:	RAN4 should define the UL MIMO TRP metric as the surface integral of measured EIRP, given TPMI is swept over all applicable TPMI according to the UE capability, and EIRP is selected as the maximum (Option 2).

Another scheme, which was under consideration during RAN4 #106bis is the scheme captured in Option 3a.  It is well  described in [10]:

We have applied the same simulation methodology as above to this scheme, which means that the TRP metric calculated is the maximum of the envelope of the pairs of TPMI indeces (2,3) and (4,5), rather than the average, as described in the paper.  This is done just to allow for a basis of comparison between Option 2 and Opton 3 metrics, since average or max operation on the TRP results just applies a scaling on the final result.

The results of surface integrals (TRP) for the three cases (Case A: coherent MIMO with full codebook; Case B: coherent MIMO with TPMI={2,3}; Case C: coherent MIMO with TPMI={4,5} for 1 GHz and 2 GHz center frequency configurations are shown in Figures 3 and 4 below.

a) b) c)

Figure 3: Coverage pattern and TRP at f=1 GHz for a) coherent MIMO codebook; b) coherent MIMO with TPMI={2,3}; c) coherent MIMO with TPMI={4,5}

a) b) c)

Figure 4: Coverage pattern and TRP at f=2 GHz for a) coherent MIMO codebook; b) coherent MIMO with TPMI={2,3}; c) coherent MIMO with TPMI={4,5}

Table 2 below summarizes the results in terms of the TRP calculation for each case:  note that the table includes results for frequencies of 1.0 GHz through 6.0 GHz.

Table 3: Summary of simulation results (TRP metric calculations of coherent/coherent subset TPMI cases)

| Frequency (GHz) | TRP calculation (dBm) |  |  |
| --- | --- | --- | --- |
|  | Case A: coherent MIMO | Case B: coherent MIMO with TPMI={2,3} | Case 3: coherent MIMO with TPMI={4,5} |
| 1.0 | 2.79 | 2.18 | 2.09 |
| 2.0 | 2.79 | 2.15 | 2.13 |
| 3.0 | 2.79 | 2.15 | 2.13 |
| 4.0 | 2.79 | 2.14 | 2.13 |
| 5.0 | 2.79 | 2.14 | 2.14 |
| 6.0 | 2.79 | 2.14 | 2.14 |

Observation 5:	Considering only a subset of the TPMIs (either the 2,3 pair or the 4,5 pair) results in a lower TRP metric when compared to the full coherent MIMO codebook, since these restricted modes fail to optimize the UE’s radiated response in all directions.

Considering the highly idealistic simulation assumptions taken in Table 1 (ideal dipole antennas, no mutual coupling, no phase difference/fluctuation between the Tx paths) and in the analysis in [10], it is prudent to highlight that these implementation impairments can dramatically alter the radiation patterns of each antenna as well as the 2-Tx combined radiation pattern of the device.  The antenna patterns of modern smartphones exhibit variations in gain and phase over the sphere, and even UEs which supports coherent UL MIMO are allowed a tolerance on the max phase difference between the conducted Tx1 and Tx2 paths.  All of these effects accumulate to distort the radiated response of each TPMI mode, such that only the max envelope of all TPMI modes represents the power delivered in all directions around the UE in an optimized UL MIMO system.

Observation 6:	In a practical smartphone implementation, non-ideal antenna patterns, mutual coupling effects, and phase difference/flucturation between the Tx paths yield unique radiated patterns for each TPMI and invalidate any claims of equivalence or substitution among the TPMI pairs of {2,3} and {4,5}.

Proposal 3:	The proposed metric to define TRP for one-layer UL MIMO with TPMI 2-5 as the average of two TRP values with TPMI 2 and 3, or 4 and 5 is applicable only to the hypothetical scenario of 2Tx with ideal dipoles and is not applicable to practical UE implementations.

# 3	Conclusions

This contribution provides further analysis of the options for the test methodology for radiated power of UL MIMO capable devices and a proposal to make a decision.  The following observations and proposals are made:

Observation 1:	The swept TPMI approach of calculating TRP for the coherent MIMO and non-coherent MIMO codebooks (Cases 1 and 2) demonstrates additional gains due to constructive superposition of signals.

Observation 2:	The case of fixed TPMI=2 exceeds the single antenna radiated output power baseline but falls short of the simple power gain target without pattern superposition.

Observation 3:	The difference between Case 1 and Case 3 of ~[3.5 to 2.6] dB represents the potential underestimation of the UE’s ability to deliver power to the gNB, if a UE capable of coherent MIMO were verified using the fixed TPMI approach.

Observation 4:	A frequency-dependent fluctuation in TRP results is observed for both Case 2 and Case 3.  Since both schemes are suboptimal ways to combine transmitted signals, these results point to the performance gap between coherent MIMO and other schemes.

Observation 5:	Considering only a subset of the TPMIs (either the 2,3 pair or the 4,5 pair) results in a lower TRP metric when compared to the full coherent MIMO codebook, since these restricted modes fail to optimize the UE’s radiated response in all directions.

Observation 6:	In a practical smartphone implementation, non-ideal antenna patterns, mutual coupling effects, and phase difference/flucturation between the Tx paths yield unique radiated patterns for each TPMI and invalidate any claims of equivalence or substitution among the TPMI pairs of {2,3} and {4,5}.

Proposal 1:	RAN4 should de-prioritize the fixed TPMI option (Option 1) from further consideration of the radiated output power test method for UL MIMO devices.

Proposal 2:	RAN4 should define the UL MIMO TRP metric as the surface integral of measured EIRP, given TPMI is swept over all applicable TPMI according to the UE capability, and EIRP is selected as the maximum (Option 2).

Proposal 3:	The proposed metric to define TRP for one-layer UL MIMO with TPMI 2-5 as the average of two TRP values with TPMI 2 and 3, or 4 and 5 is applicable only to the hypothetical scenario of 2Tx with ideal dipoles and is not applicable to practical UE implementations.

# References

RP-223112, “Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC),” vivo, 3GPP RAN #98, December 2022

R4-2220610, “WF for Rel-18 TRP TRS WI,” vivo, 3GPP RAN4 #105, November 2022

R4-2300349, “Test methodology for radiated power of UL MIMO capable devices,” Apple, 3GPP RAN4 #106, Februarry 2023

R4-2300141, “Proposals for OTA tests under UL MIMO and CA,” Huawei, 3GPP RAN4 #106, Februarry 2023

R4-2300996, “Discussion on 2Tx test method in Rel-18 TRP TRS WI,” Samsung, 3GPP RAN4 #106, February 2023

R4-2301459, “Consideration on proper TPMI for UL-MIMO TRP test,” OPPO, 3GPP RAN4 #106, February 2023

R4-2302754, “Testability aspects for 2Tx TRP,” Rohde & Schwarz, 3GPP RAN4 #106, February 2023

R4-2302917, “WF for Rel-18 TRP/TRS requirements,” vivo, 3GPP RAN4 #106, February 2023

R4-2304343, “On TRP TRS test methodology,” Apple, 3GPP RAN4 #106bis, April 2023

R4-2305701, “Further discussion on FR1 1 Layer UL-MIMO 2Tx SISO OTA TRP,” MediaTek, vivo, CAICT, 3GPP RAN4 #106bis, April 2023

R4-2304028, “on TRP TRS test aspects,” Huawei, 3GPP RAN4 #106bis, April 2023

R4-2304828, “Discussion on TRP concept and test method for multi-antenna UE,” Samsung, 3GPP RAN4 #106bis, April 2023

R4-2305607, “Discussion on enhancement of test method for TRP and TRS,” Qualcomm, 3GPP RAN4 #106bis, April 2023

R4-2305610, “Discussion on concept and definition of TRP,” OPPO, 3GPP RAN4 #106bis, April 2023

R4-2305904, “WF for Rel-18 TRP/TRS requirements,” vivo, 3GPP RAN4 #106bis, April 2023