3GPP RAN WG4 Meeting #107		R4-2309089

Incheon, South Korea, May 22nd – 26th, 2023

Agenda item:	7.26.2.1

Source:	Apple

Title:	n71 REFSENS for 25MHz/30MHz/35MHz symmetric CBW

WI/SI:	NR_bands_R18_BWs-Core

Release:	Rel-18

Document for:	Approval

# 1	Introduction

n71 DL channel bandwidth (CBW) up to 35 MHz has already been introduced, while UL CBW has been restricted to 20 MHz owing to the concern of the potential large sensitivity degradation caused by reduced frequency separation between the UL and DL resource allocations and the elevated MPR due to relative bandwidth ratio exceeding 3% for UL CBW wider than 20 MHz. Nevertheless, in RAN4 #106 meeting, a new request to add 25MHz and 30MHz UL CBWs to n71 was proposed [1] and the associated revised WID on “adding new channel bandwidth(s) support to existing NR bands” [2] was approved in RAN #99 meeting. In last RAN4 meeting, we provided our preliminary analysis on REFSENS degradation for 30MHz symmetric UL/DL CBW as compared to the existing REFSENS requirement for 30MHz DL CBW with 20MHz UL CBW and proposed to define new REFSENS requirements for 25MHz, 30MHz, and 35MHz DL CBW based on UL CBW at either 25MHz or 30MHz [3]. As 35MHz symmetric UL/DL CBW would not be available without the introduction of 35MHz UL CBW, the n71 proponent company commented that they would propose to add 35MHz UL CBW to the basket WID in next plenary meeting. On the other hand, according to the agreements in the approved WF on “adding new channel bandwidths support to existing NR bands” [4], the symmetric UL/DL 25MHz and 30MHz REFSENS values will be captured as optional requirements in addition to the existing REFSENS requirements with UL CBW at 20MHz. In addition, it was agreed that PC2 FDD should not be considered complete for n71 until PC2 FDD 1Tx and 2Tx RSD (REFSENS Degradation) are specified for symmetric UL/DL 25MHz and 30MHz CBWs. In this contribution, we provide our analyses on both PC3 REFSENS and PC2 RSD relative to PC3 REFSENS for n71 25MHz/30MHz/35MHz symmetric UL/DL CBWs and propose 0.5dB ΔMPR for 30MHz UL CBW.

# Discussion

Unlike in LTE, NR FR1 UE maximum output power reduction (MPR) has been specified without UL CBW dependency except when the relative CBW ratio exceeds 4% for TDD band and 3% for FDD band, where additional MPR relaxation (ΔMPR) may be applied. The relative channel bandwidth ratio is defined as the CBW divided by the carrier frequency. In current RAN4 specifications [5], ΔMPR had been specified for Band n28 at 30MHz UL CBW and Band n40 for 100MHz UL CBW, as shown in Table 2-1.

| NR Band | Duplex | Power Class | UL CBW (MHz) | UL CBW Ratio | ΔMPR (dB) |
| --- | --- | --- | --- | --- | --- |
| n28 | FDD | PC3 | 30 | 4.2% | 0.5 |
| n40 | TDD | PC3 and PC2 | 100 | 4.3% | 1.0 |
| n71 | FDD | PC3 | 25 | 3.7% | FFS |
| n71 | FDD | PC3 | 30 | 4.4% | FFS |
| n71 | FDD | PC3 | 35 | 5.1% | FFS |

Table 2-1 MPR relaxations for UL relative CBW ratio exceeding 3% for FDD and 4% for TDD band

To our understanding, the ∆MPR is to accommodate the more pronounced PA memory effect when the relative CBW ratio exceeds certain percentage which could result in unbalanced and degraded ACLR at either side of the spectrum. As n71 is in the similar frequency range with n28, it would be reasonable to apply the same ∆MPR requirement at least for 30MHz UL CBW.

Proposal 1: ΔMPR is specified as 0.5 dB for n71 30MHz UL CBW, FFS for 25MHz and 35MHz UL CBWs.

For the potential further REFSENS degradation due to UL CBW wider than 20MHz, we have performed numerical simulation for symmetric UL/DL CBW at 25MHz/30MHz/35MHz and compared the results to the current n71 REFSENS requirements with UL CBW at 20MHz. For the UL CBW at 25MHz/30MHz/35MHz, we also assumed the UL configuration is restricted to 20 RBs which are located closest to the DL band (RBstart = 0). Both PC3 REFSENS and PC2 RSD relative to PC3 REFSENS with 1Tx and 2Tx were evaluated.

Table 2-2 summarizes our simulation assumptions for the REFSENS analysis. The simulated results for PC3 REFSENS are presented in Table 2-3.

| Parameter | Value | Comment |
| --- | --- | --- |
| Tx and PA model | Same as used for MPR/A-MPR simulations | CIM3, CIM5, IRR, etc. |
| Tx power | 23 dBm | At antenna port |
| FE insertion loss | 4 dB |  |
| Duplexer isolation | 50 dB | Tx to Rx |
|  | 42 dB | Tx to ANT |
| Antenna isolation | 10 dB | Main to diversity |
| MRC |  | Uncorrelated |

Table 2-2 Simulation assumptions for n71 REFSENS analysis

| NR Band | DL CBW (MHz) | UL CBW (MHz) | UL LCRB | UL RBstart | REFSENS (dBm) |
| --- | --- | --- | --- | --- | --- |
| n71 | 25 | 20 | 20 | 0 | -84.1 |
|  | 25 | 25 | 20 | 0 | -75.1 |
|  | 30 | 20 | 20 | 0 | -82.5 |
|  | 30 | 30 | 20 | 0 | -66.9 |
|  | 35 | 20 | 20 | 0 | -80.7 |
|  | 35 | 35 | 20 | 0 | -65.5 |

Table 2-3 Simulated PC3 REFSENS for n71 25MHz/30MHz/35MHz symmetric UL/DL CBWs

The simulated results for PC2 1Tx and 2Tx RSD relative to PC3 REFSENS are summarized in Table 2-4.

| NR Band | DL CBW (MHz) | UL CBW (MHz) | No. of Tx | RSD (dB) |
| --- | --- | --- | --- | --- |
| n71 | 25 | 25 | 1 | 1.7 |
|  | 25 | 25 | 2 | 7.4 |
|  | 30 | 30 | 1 | 1.5 |
|  | 30 | 30 | 2 | 7.7 |
|  | 35 | 35 | 1 | 1.5 |
|  | 35 | 35 | 2 | 7.8 |

Table 2-4 Simulated PC2 RSD relative to PC3 REFSENS

Proposal 2: RAN4 to take the PC3 REFSENS values in Table 2-3 for n71 25MHz/30MHz/35MHz symmetric UL/DL CBWs into consideration.

Proposal 3: RAN4 to take the 1Tx and 2Tx PC2 RSD values relative to PC3 REFSENS in Table 2-4 for n71 25MHz/30MHz/35MHz symmetric UL/DL CBWs into consideration.

# 3	Conclusion

In this contribution, we provide our analyses on both PC3 REFSENS and PC2 RSD relative to PC3 REFSENS for n71 25MHz/30MHz/35MHz symmetric UL/DL CBWs and propose 0.5dB ΔMPR for 30MHz UL CBW.

Proposal 1: ΔMPR is specified as 0.5 dB for n71 30MHz UL CBW, FFS for 25MHz and 35MHz UL CBWs.

Proposal 2: RAN4 to take the PC3 REFSENS values in the Table below for n71 25MHz/30MHz/35MHz symmetric UL/DL CBWs into consideration.

| NR Band | DL CBW (MHz) | UL CBW (MHz) | UL LCRB | UL RBstart | REFSENS (dBm) |
| --- | --- | --- | --- | --- | --- |
| n71 | 25 | 25 | 20 | 0 | -75.1 |
|  | 30 | 30 | 20 | 0 | -66.9 |
|  | 35 | 35 | 20 | 0 | -65.5 |

Proposal 3: RAN4 to take the 1Tx and 2Tx PC2 RSD values relative to PC3 REFSENS in the table below for n71 25MHz/30MHz/35MHz symmetric UL/DL CBWs into consideration.

| NR Band | DL CBW (MHz) | UL CBW (MHz) | No. of Tx | RSD (dB) |
| --- | --- | --- | --- | --- |
| n71 | 25 | 25 | 1 | 1.7 |
|  | 25 | 25 | 2 | 7.4 |
|  | 30 | 30 | 1 | 1.5 |
|  | 30 | 30 | 2 | 7.7 |
|  | 35 | 35 | 1 | 1.5 |
|  | 35 | 35 | 2 | 7.8 |

# 4	References

R4-2303584 “Revised Basket WID on adding channel bandwidth support to existing NR bands”, Ericsson, 3GPP TSG RAN WG4 Meeting #106, Athens, Greece, February 27th – March 3rd, 2023

RP-230756 “Basket WID: adding new channel bandwidth(s) support to existing NR bands”, Ericsson, 3GPP TSG RAN Meeting #99, March 20th – 23rd, 2023

R4-2304349 “On RF requirements for 25MHz and 30MHz CBW for n71”, Apple, 3GPP TSG RAN WG4 Meeting #106bis-e, online, April 17th – 26th, 2023

R4-2306545 “WF on adding new channel bandwidths support to existing NR bands”, Ericsson, 3GPP TSG RAN WG4 Meeting #106bis-e, online, April 17th – 26th, 2023

3GPP TS 38.101-1 V18.1.0 (2023-03)