3GPP TSG-RAN WG4 Meeting #107	R4-2309099

# Incheon, Korea,  22nd May – 2nd June, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | 0501 | rev | 0 | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.141-2: 71 GHz Extension BS conformance test Test Model clarification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Perf |  |  |  |  |  | Date: |  |  | 2023-5-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Further clarification is necessary for FR2-2 test model definition to clear ambiguity left from previously endorsed and implemented text. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Clause 4.9.2.3 - Added clarificaiton on repetition is about payload data. - Not to repeat other physical layer related sequence such as DM-RS and PT-RS which raised as question requires clarification during discussion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification left with ambigity on FR2-2 test model definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 4.9.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 4.9.2.3.1	PDCCH

-	NsymbCORESET= 2

-	PDCCH modulation to be QPSK as described in TS 38.211 [20], clause 5.1.3.

-	For each slot the required amount of bits for all PDCCHs is as follows: 1(# of PDCCH) * 1(# of CCE per PDCCH) * 6(REG per CCE) * 9(data RE per REG) * 2(bits per RE) with these parameters according to the NR-FR2-TM definitions in clause 4.4.9.2.2.

-	Generate this amount of bits from the output of the PN23 sequence generator [28]. For FR2-1, the PN sequence generator is initialized with a starting seed of "all ones" in the first allocated slot of each frame. The PN sequence is continuous over the slot boundaries. For FR2-2, the PN sequence generator is initialized with a starting seed of "all ones" in the first allocated slot of each 80 slots time period of test model length, identical payload data sequence repeated in following 80 slot. The PN sequence is continuous over the slot boundaries.

-	1 CCE shall be according to TS 38.211 [20], clause 7.3.2. PDCCH using non-interleaved CCE-to-REG mapping. PDCCH occupies the first two symbols for 6 resource-element groups, where a resource element group equals one resource block during one OFDM symbol.

-	Perform PDCCH scrambling according to TS 38.211 [20], clause 7.3.2.3.

-	NID=NIDcell in DM-RS sequence generation in TS 38.211 [20], clause 7.4.1.3.

-	nRNTI=0 in scrambling sequence generation in TS 38.211 [20], clause 7.3.2.3.

-	Perform mapping to REs according to TS 38.211 [20], clause 7.3.2.5.

##### 4.9.2.3.2	PDSCH

-	Generate the required amount of bits from the output of the PN23 sequence generator [28]. For FR2-1, the PN sequence generator is initialized with a starting seed of "all ones" in the first allocated slot of each frame. For FR2-2, the PN sequence generator is initialized with a starting seed of "all ones" in the first allocated slot of each 80 slots time period of test model length, identical payload data sequence repeated in following 80 slot. The PN sequence is continuous over the slot boundaries. For TDD TMs, the PN sequence can be generated for all symbols (in the DL, UL or special slots) or only DL symbols (in the DL or special slots). For TMs with multi-users, the PN sequence can be generated per user (nRNTI).

-	NR-FR2-TMs utilize 1 or 2 user PDSCH transmissions distinguished by nRNTI. For each NR-FR2-TM, PRBs are mapped to user (nRNTI) as follows:

Table 4.9.2.3.2-1: Mapping of PRBs to  for NR-FR2-TM

| Test model |  | Number of users |
| --- | --- | --- |
| NR-FR2-TM1.1 | 2 for PRBs located in PRB#0-2 0 for remaining PRBs | 2 |
| NR-FR2-TM2 | 2 for all PRBs | 1 |
| NR-FR2-TM2a | 2 for all PRBs | 1 |
| NR-FR2-TM3.1 | 2 for PRBs located in PRB#0-2 0 for remaining PRBs | 2 |
| NR-FR2-TM3.1a | 2 for PRBs located in PRB#0-2 0 for remaining PRBs | 2 |

-	Perform user specific scrambling according to TS 38.211 [20], clause 7.3.1.1.

-	nID=NIDcell

-	Perform modulation of the scrambled bits with the modulation scheme defined for each user according to TS 38.211 [20], clause7.3.1.2.

-	Perform mapping of the complex-valued symbols to layer according to TS 38.211 [20], clause 7.3.1.3.   Complex-valued modulation symbols  for codeword q shall be mapped onto the layers ,  where υ is equal to 1.

-	Perform PDSCH mapping type A according to TS 38.211 [20].

-	PDSCH resource allocation according to TS 38.214 [21] as following;

-	NR-FR2-TM1.1, NR-FR2-TM3.1, NR-FR2-TM3.1a: type 1 for PDSCH with nRNTI = 0, nRNTI = 2

-	NR-FR2-TM2, NR-FR2-TM2a: type 1 for PDSCH with nRNTI = 2

-	DM-RS sequence generation according to TS 38.211 [20], clause 7.4.1.1.1 where l is the OFDM symbol number within the slot with symbols indicated by table 4.9.2.2-3.

-	NIDnSCID=NIDcell

-	nSCID=0

-	DM-RS mapping according to TS 38.211 [20], clause 7.4.1.1.2 with parameters listed in table 4.9.2.2-3.

-	For NR-FR2-TM PT-RS sequence generation according to TS 38.211 [20], clause 7.4.1.2.1, with parameters listed in table 4.9.2.2-3.

-	For NR-FR2-TM PT-RS mapping according to TS 38.211 [20], clause 7.4.1.2.2, with parameters listed in table 4.9.2.2-3.