3GPP TSG-RAN WG4 Meeting #107	R4-2309114

Incheon, KR, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for non-collocated FR1 inter-band EN-DC with overlapping bands

Agenda item:	8.12.3

Document for:	Discussion

Introduction

In RAN4#106bis-e meeting, MRTD/MTTD CR for FR1 non-collocated intra-band NR-CA were endorsed. The following was agreed in the WF [1] that the impact on legacy MRTD/MTTD requirements due to UE capability of interBandMRDC-WithOverlapDL-Bands-r16 should be discussed in RRM maintenance part, but it does not indicate how to handle the impact on R18 MRTD/MTTD and other RRM requirements for inter-band EN-DC with overlapping DL bands.

Issue 1-1-1: MRTD/MTTD requirements for FR1 inter-band EN-DC with overlapping DL band in R16 - Agreements - Impact on legacy MRTD/MTTD requirements due to UE capability of interBandMRDC-WithOverlapDL-Bands-r16 should be discussed in RRM maintenance part instead of in this WI.

In this paper, we will provide our view on the impact on legacy MRTD/MTTD requirements due to UE capability of interBandMRDC-WithOverlapDL-Bands-r16.

Discussion

In RAN4#106bis-e meeting, it was agreed that the impact on legacy MRTD/MTTD requirements due to UE capability of interBandMRDC-WithOverlapDL-Bands-r16 should be discussed in RRM maintenance part instead of the R18 WI NonCol_intraB_ENDC_NR_CA, the agreement was captured in the agreed WF[1] as below:

In our understanding, the UE capability interBandMRDC-WithOverlapDL-Bands-r16 was introduced in Rel-16, and the RF requirements for 2-layer inter-band EN-DC with overlapping DL bands were specified in R16 and R17. RRM requirements including MRTD/MTTD were not in the scope of the legacy WIs where RF requirements were introduced.

Observation #1: RRM requirements including MRTD/MTTD were not in the scope of the legacy WIs where RF requirements for 2-layer inter-band EN-DC with overlapping DL bands were introduced.

In addition, according to the R18 non-collocated intra-band NRCA/EN-DC WID [2], the following are listed in the objectives. This clearly includes identifying the MRTD/MTTD and other RRM requirements impacts in non-collocated deployment.

- Phase II:

  - Phase II work will get started after the feasibility in phase I is confirmed

  - Specify MRTD and MTTD requirements and other identified RRM requirement impact(s) (if any) in non-collocated deployment

  - Discuss and decide if the different requirements will be specified based on UE capability of interBandMRDC-WIthOverlapDL-Bands-r16.

Observation #2: Identifying the MRTD/MTTD and other RRM requirements impacts in non-collocated deployment is clearly indicated in R18 non-collocated intra-band NRCA/EN-DC WID.

Therefore, the discussion on the MRTD/MTTD and other RRM requirements impact in R18 shall be within the R18 non-collocated intra-band NRCA/EN-DC WID. This would not impact the discussion on the impact to legacy MRTD/MTTD requirements in Rel-16 and Rel-17, which will be handled in RRM maintenance part in parallel considering the outcome from R18 discussion.

Proposal 1: The discussion on the MRTD/MTTD and other RRM requirements impact in R18 shall be within the R18 non-collocated intra-band NRCA/EN-DC WID.

Based on the proposal, we prepare the CRs for R18 MRTD/MTTD and interruption requirements for non-collocated inter-band EN-DC with overlapping in [5] and [6].

- Conclusion

This contribution discusses the MRTD/MTTD requirements for non-collocated FR1 inter-band EN-DC with overlapping bands. The observations and proposals are summarized as below:

Observation #1: RRM requirements including MRTD/MTTD were not in the scope of the legacy WIs where RF requirements for 2-layer inter-band EN-DC with overlapping DL bands were introduced.

Observation #2: Identifying the MRTD/MTTD and other RRM requirements impacts in non-collocated deployment is clearly indicated in R18 non-collocated intra-band NRCA/EN-DC WID.

Proposal 1: The discussion on the MRTD/MTTD and other RRM requirements impact in R18 shall be within the R18 non-collocated intra-band NRCA/EN-DC WID.

References

- R4-2306335	WF on RRM requirements for intra-band non-collocated EN-DC/NR-CA, Huawei, RAN4#106bis-e

- RP-230742 WID_RAN4-R18_Intra-band non-collocated EN-DC/NR-CA, KDDI, RAN#99

- R4-2309115, Rel-18 MRTD/MTTD requirement for non-collocated inter-band EN-DC with overlapping bands, Nokia, Nokia Shanghai Bell, Ericsson

- R4-2309116, Rel-18 interruption requirement for non-collocated inter-band EN-DC with overlapping bands, Nokia, Nokia Shanghai Bell