3GPP TSG-RAN WG4 Meeting #107	R4-2309135

Incheon KR, May 22 – 26, 2023

Agenda item:	8.23.3.6

Source: 	Qualcomm Incorporated

Title: 	On requirements for carrier phase positioning

Document for:	Discussion

# Introduction

RAN4 continued discussing requirements for carrier phase positioning (CPP) in RAN4#106bis-e. New agreements and open issues were captured in a WF [1].

In this paper we provide our views and proposals on open issues based on progress in RAN1.

# Discussion

Let us begin the discussion by reviewing agreements regarding the definition of measurements for CPP.

#### Issue 2-1-1: Measurement types to be defined for DL carrier phase measurement requirements

Agreements:

- Wait for more progress and outcomes on the definition of measurement from RAN1/RAN2 to define the measurement period requirements for CPP.

According to agreement in RAN1#112bis-e, RAN1 will define two types of carrier phase measurements [2]. DL RSCP is a measurement of absolute carrier phase, while DL RSCPD is a differential phase measurement. Thus far, CPP measurements can be reported as secondary/supplementary positioning measurements. DL RSCP can be reported with UE Rx-Tx for multi-RTT positioning, and DL RSCPD can be reported with RSTD for DL-TDOA. It has not been agreed to support reporting of stand-alone CPP measurements.

Agreement

Introduce DL reference carrier phase (DL RSCP) and NR DL reference carrier phase difference (DL RSCPD) as DL carrier phase measurements.

- Note: It is up to RAN4 to decide whether and how to define the requirements for DL RSCP and/or DL RSCPD. No LS needed to RAN4 for this note.

- DL RSCP can be reported together with UE Rx – Tx time difference measurement

- DL RSCPD can be reported together with RSTD measurement

- FFS: details on how to eliminate unknown initial Rx phase with RSCP/RSCPD reporting can be further discussed

- Note: Whether to support standalone DL RSCP and/or DL RSCPD reporting, or DL RSCP/DL RSCPD reporting with other new types of measurements (if agreed), can be further discussed.

Agreement

Support the reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both UE-based and UE-assisted NR DL carrier phase positioning, including

- UE in RRC_CONNECTED state with measurement gap.

- FFS: UE in RRC_CONNECTED state without measurement gap

- UE in RRC_ INACTIVE state

Proposal 1: Do not define requirements for DL RSCP reported with RSTD.

Proposal 2: Do not define requirements for DL RSCPD reported with UE Rx-Tx.

Proposal 3: RAN4 will not discuss requirements for stand-alone CPP measurements unless RAN1 agrees first to support such measurements.

Since RSCP is an absolute carrier phase measurement, subject to unknown, variable carrier phase offset resulting from multiple sources at the UE and TRP, it is not practical to define absolute accuracy requirements for such measurements.

Proposal 4: Do not define absolute measurement accuracy requirements for DL RSCP.

Regarding the applicable RF frequencies for CPP measurements, RAN1 has agreed that for both DL/UL measurements the default assumption is the center frequency of the PFL [2]. There are no RAN1 agreements to date enabling CPP measurements to be performed over multiple sub-bands/frequencies within a PFL or carrier. For now, RAN4 should follow the current RAN1 agreement of CPP measurements performed over the whole PFL/carrier bandwidth.

#### Issue 2-2-2: Applicable RF frequencies for DL RSCP measurement period requirements:

< Way forward >

- Option 1:

  - The basic measurement requirement for DL RSCP is defined for each RF frequency. And RAN4 to discuss whether to define measurement requirements for multiple RF frequencies.

Agreement

The specific RF frequency associated with a DL carrier phase measurement is defined as the center frequency of the DL PFL by default.

- Note: It is open to further discussion whether a frequency other than the center frequency of the DL PFL can also be the specific RF frequency for non-default case(s), if RAN1 agrees to introduce them.

Agreement

The specific RF frequency associated with a UL carrier phase measurement is defined, by default, as the center frequency of the transmission bandwidth of the SRS for positioning purpose.

- Note: It is open to further discussion whether a frequency other than the center frequency of the UL carrier can also be the specific RF frequency for a non-default case(s), if RAN1 agrees to introduce them.

Proposal 5: RAN4 to define requirements for CPP measurements performed over the whole PFL/carrier bandwidth and associated with the center frequency of the PFL/carrier by default, consistent with RAN1 agreements.

Proposal 6: RAN4 will not discuss requirements for CPP measurements performed for multiple frequencies within a PFL/carrier unless RAN1 agrees first to support such measurements.

Due to the need to eliminate/mitigate various sources of error impacting CPP measurements, RAN1 considers that it is necessary to coordinate measurements/transmissions between target UE and PRU and between TRPs involved in positioning methods relying on CPP measurements. That is the motivation for introducing a time window indication in the request for CPP measurements in the below RAN1 agreement [2]. In our view, this enhancement is essential (not merely a nice to have) to enable practical use of CPP measurements. Measurement requirements for CPP should be formulated accounting for the time window indication from the LMF.

In addition, it is clear from the agreement below that RAN1 is discussing how the LMF will indicate to the UE applicable PRS resources for CPP measurements. RAN4 should wait for RAN1 to reach a conclusion on this issue.

Agreement

To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements:

- Enabling LMF to request the UEs, including target UE and PRU(s), to perform measurements on [indicated] DL PRS resources occurring within indicated time window(s).

- FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time.

Observation 1: RAN1 is discussing whether/how the LMF will indicate to the UE the PRS resources used to measure carrier phase. RAN4 to wait for RAN1 conclusions on this issue.

Proposal 7: Measurement period requirements for CPP shall take into account the time window indication by the LMF.

Proposal 8: RAN4 to wait for further progress in RAN1 before defining the measurement period requirements for CPP measurements.

In addition to the time window constraints specified by RAN1, we understand RAN4 should prioritize accuracy requirements for CPP measurements based on single sample. We do not consider it practical to specify accuracy requirements assuming joint processing of multiple samples. While mult-sample processing may be enabled with additional assumptions or information, it is unlikely that the entity (UE/PRU or TRP) performing the measurements would have additional information available when the measurements are performed. E.g. whether carrier phase continuity can be assumed across multiple transmissions of a RS resource. Such additional information may be provided to the location server (or positioning engine) after measurements are reported. It can be left up to location server implementation whether and how to leverage side information to perform joint processing of multiple measurements reported by the UE, PRUs and TRPs.

#### Issue 2-1-5: Applicable number of measurement instances

< Way forward >

- Option 1:

  - RAN4 to investigate and specify the measurement period, reporting, and accuracy requirements for DL CPP measurements, for the cases of a single and multiple measurement instances aligned to requirements for legacy positioning techniques in Rel-16 and Rel-17.

Proposal 9: Prioritize defining CPP measurement accuracy requirements for single sample measurements. Note: This agreement does not preclude specifying measurement period requirements for number of samples greater than one.

# Conclusions

Proposal 1: Do not define requirements for DL RSCP reported with RSTD.

Proposal 2: Do not define requirements for DL RSCPD reported with UE Rx-Tx.

Proposal 3: RAN4 will not discuss requirements for stand-alone CPP measurements unless RAN1 agrees first to support such measurements.

Proposal 4: Do not define absolute measurement accuracy requirements for DL RSCP.

Proposal 5: RAN4 to define requirements for CPP measurements performed over the whole PFL/carrier bandwidth and associated with the center frequency of the PFL/carrier by default, consistent with RAN1 agreements.

Proposal 6: RAN4 will not discuss requirements for CPP measurements performed for multiple frequencies within a PFL/carrier unless RAN1 agrees first to support such measurements.

Observation 1: RAN1 is discussing whether/how the LMF will indicate to the UE the PRS resources used to measure carrier phase. RAN4 to wait for RAN1 conclusions on this issue.

Proposal 7: Measurement period requirements for CPP shall take into account the time window indication by the LMF.

Proposal 8: RAN4 to wait for further progress in RAN1 before defining the measurement period requirements for CPP measurements.

Proposal 9: Prioritize defining CPP measurement accuracy requirements for single sample measurements. Note: This agreement does not preclude specifying measurement period requirements for number of samples greater than one.

# References

[1] R4-2306351, WF on RRM requirements for NR sidelink positioning and carrier phase positioning, RAN4#106bis-e

[2] Session notes for 9.5 Expanded and Improved NR Positioning, RAN1#112bis-e