3GPP TSG-RAN WG4 Meeting #107	R4-2309136

Incheon KR, May 22 – 26, 2023

Agenda item:	5.2.10.3

Source: 	Qualcomm Incorporated

Title: 	On Rel-17 MG_enh RRM maintenance issues

Document for:	Approval

# Introduction

RAN4 discussed RRM maintenance issues for Rel-17 measurement gaps (MG) enhancements in RAN4#106. Open issues were captured in a WF [1].

In this brief paper we provide our views and proposals to address the remaining issues.

# Discussion

## NCSG

Issue 1-2: Requirements for inter-frequency measurement without MG

- Agreement

  - For inter-frequency without MG (cl. 9.3.9), define separate NW sync assumption and number of samples for

    - Scenario 1, where inter-frequency SSB is within UE active BWP, and

    - Scenario 2, where UE indicates ‘nogap-noncsg’ for the inter-frequency measurement.

Proposal 1: Number of samples for scenario 2 is changed to 8 from 5 for scenario 2.

Observation 1: About NW sync assumption, the current NW sync assumption is applied for scenario 1. For scenario 2, if NW can keep the sync assumption for sceanrio2, there will be no spec impact. Otherwise, deriveSSB-indexFromCellinter-r17 flag can be used for NW sync assumption.

For inter-frequency SSB based measurements without measurement gaps in active BWP, it is assumed that when UE performs inter-frequency measurements without measurement gaps in a TDD bands on FR1 and FR2, the following condition is met: - SFN and frame boundary across serving cell and inter-frequency neighbor cells is aligned

## Concurrent MG

The first two issues for concurrent MG are clarifications of how the measurement requirements depend on the overlap between a MO and MGs.

The first issue aims to clarify whether a MO associated with a measurement gap is always measured within gap.

Issue 2-1: MO associated with a MG but measured without MG

- Way forward

  - For an MO that can be measured outside MG and associated with an MG, FFS

    - Option 1: the MO is always measured within the associated MG

    - Option 2: the MO is measured within the associated MG only when it needs to be measured with MG

Our understanding is that if a MO can be measured without gap, the it is measured without gap even if association with a MG is indicated by the network. For example, if some SMTC occasions of a SSB do not overlap with any gaps, then the SSB is measured without gaps even if the network associated the MO with a concurrent MG.

Figure 1: Example of SSB measured without MG.

Proposal 2: A MO that is associated with a concurrent MG is measured within the associated MG only when it cannot be measured without MG.

The second issue aims to clarify the condition of full overlap between a MO and concurrent MGs.

Issue 2-2: MO fully overlapping with the union of the MG1 and MG2

- Wayforward

  - the MO should be measured within the associated MG in the following cases,

    - When a MO is fully overlapping with the associated MG, or

    - FFS: When a MO is partially overlapping with the associated MG and fully overlapping with the union of the ConMGs.

  - Whether to clarify this in spec is FFS

Clearly a MO associated with a MG can only be measured within MG if it overlaps at least partially (at least some occasions overlap) with the associated MG. Otherwise, it can only be measured without MG, if applicable, or no requirements apply.

Figure 2: Example of full overlap between a MO and concurrent MGs. The MO is measured within the associated MG.

Proposal 3: A MO is measured within MG when it fully overlaps with (the union of) concurrent MGs and it overlaps with the associated MG.

The last issue considers whether to restrict the association of LTE MOs to concurrent MGs.

Issue 2-3: E-UTRAN measurement with con-MGs

- Way forward

  - FFS whether all E-UTRAN measurement objects are expected to be associated with a single concurrent gap pattern or not when both E-UTRAN measurement objects and other type of measurements are configured

    - Option 1: Yes.

    - Option 2: No.

    - Option 3: Up to UE capability (e.g. concurrentMeasGapEUTRA-r17)

    - For UE supporting the capability, different LTE MOs can be associated with multiple MGs.

    - For UE not supporting the capability, all LTE MOs can be associated with only a single MG.

In our view, there is no strong reason to limit the association of all MOs to a single measurement gap. If other companies have a strong preference for a new UE capability, we would support it.

Proposal 4: Support associating E-UTRAN measurement objects to different concurrent MGs. A new UE capability can be considered.

# Conclusions

NCSG

Proposal 1: Number of samples for scenario 2 is changed to 8 from 5 for scenario 2.

Observation 1: About NW sync assumption, the current NW sync assumption is applied for scenario 1. For scenario 2, if NW can keep the sync assumption for sceanrio2, there will be no spec impact. Otherwise, deriveSSB-indexFromCellinter-r17 flag can be used for NW sync assumption.

Concurrent MG

Proposal 2: A MO that is associated with a concurrent MG is measured within the associated MG only when it cannot be measured without MG.

Proposal 3: A MO is measured within MG when it fully overlaps with (the union of) concurrent MGs and it overlaps with the associated MG.

Proposal 4: Support associating E-UTRAN measurement objects to different concurrent MGs. A new UE capability can be considered.

# References

[1] R4-2303206, WF on R17 NR MG enhancements RRM maintenance, RAN4#106.