3GPP TSG-RAN4 Meeting #107	R4-2309161

# Incheon, Korea (Republic Of), 22nd – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1600 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of FR1 DMRS bundling measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh-Core |  |  |  |  |  | Date: |  |  | 2023-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In R4-230xxxx RAN5 has provided inputs on the open items regarding the phase offset measurement for DMRS bundling. The current description in the specification needs to be adapted according to the RAN5 feedback to ensure the testability of the requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add timing correction to section F.9.3. Update the description in section F.9.4 to account for the mean phase error in each slot. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement is not testable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.9.3, F.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.521-1 CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged sections omitted >

# F.9	Phase offset measurement for DMRS bundling

## F.9.1	Measurement point

The measurement point for phase offset measurement is defined in Figure F.9.1-1.

Figure F.9.1-1: Measurement point for phase offset for DMRS bundling

## F.9.2	Symbols used

Phase offset is determined based on DMRS REs (3 DMRS symbols per slot) with the option to use data symbols.

## F.9.3	Modified test signal

Same as described in Annex F.4.

In addition, the timing difference ∆t derived according to Annex F.4 shall be corrected before further processing the signal.

## F.9.4	Phase offset measurement

The phase offset measurement is based on the phase response of the Tx chain  as derived based on Annex F.4.

In addition, the phase response in each slot shall be corrected by the mean phase averaged over all subcarriers in the same slot:

φ't,f= φt,f-1ni=1nφt,fi

The phase difference ∆φf for each subcarrier between a reference timeslot tref and the measurement timeslot tm is then calculated as defined below, based on the corrected phase measurement above:

∆φ'f= φ'tm,f-φ'(tref,f)

The phase offset between the reference and measurement timeslots are then calculated as the maximum over the results for all subcarriers as shown below:

PhaseOffset=maxf(∆φ'f∆φf)

#### < End of changes >