3GPP TSG-RAN WG4 Meeting#107                             R4-2309167

Incheon , KR, May 22 – May 26,2023

Source:	ZTE Corporation

Title:	TP for TS 38.876: Clause 6.1~6.3

Agenda Item:	8.14.1.1

Document for:	Approval

- Introduction

- This contribution provides a text proposal to TS38.876 [1] on sub-clause 6.1~6.3.

- Reference

- R4-230xxxx, TS 38.876 Study on Air-to-ground network for NR, CMCC, approved.

- Text Proposal

6	Co-existence study

6.1 Co-existence simulation scenario

Table 6.1-1 summarizes the initial simulation scenarios for ATG coexistence study considering non co-location scenario as the baseline. Assume non-co-located for simulation cases 1, 4, 5, 6, 7, 8, 9, 12, 13, 14. For simulation cases 2, 3, 10, 11, if evidence is brought forward that the ACLR/ACS requirements to cover co-location are substantially different to the requirements for the non-co-location, then discuss further how to cover both cases.

Table 6.1-1: Simulation scenarios for ATG coexistence study

| No. | Combination | Aggressor |  | Victim |  | Simulation frequency | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | deployment scenario UL/DL | CBW duplex mode | deployment scenario UL/DL | CBW duplex mode |  |  |  |
| 1 | TN with ATG | ATG DL | 100MHz TDD | TN rural DL | 100MHz /TDD | 3.5 GHz |  | Phase 1 |
| 2 | TN with ATG | ATG UL | 100MHz TDD | TN rural UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 3 | TN with ATG | TN rural DL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 4 | TN with ATG | TN rural UL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 5 | TN with ATG | ATG DL | 100MHz TDD | TN rural UL | 100MHz /TDD | 3.5GHz |  | Phase 2 |
| 6 | TN with ATG | ATG UL | 100MHz TDD | TN rural DL | 100MHz TDD | 3.5GHz |  | Phase 2 |
| 7 | TN with ATG | TN rural DL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz |  | Phase 2 |
| 8 | TN with ATG | TN rural UL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | Phase 2 |
| 9 | TN with ATG | ATG DL | 20MHz FDD | TN rural DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 10 | TN with ATG | ATG UL | 20MHz FDD | TN rural UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 11 | TN with ATG | TN rural DL | 20MHz FDD | ATG DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 12 | TN with ATG | TN rural UL | 20MHz FDD | ATG UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 13 | TN with ATG | ATG UL | 20MHz FDD | TN rural DL | 20MHz TDD | 2 GHz | n1/n39 | Phase 2 |
| 14 | TN with ATG | TN rural DL | 20MHz TDD | ATG UL | 20MHz FDD | 2 GHz | n39/n1 | Phase 2 |
| NOTE 1: ACLR/ACS requirement for ATG BS and ATG CPE are derived based on the synchronized scenario in Phase 1. NOTE 2: the non-synchronized coexistence scenarios listed in phase 2 are mainly used to identify the isolation distance between ATG BS and the the legacy TN BSs. And it’s agreed to reuse 45dB ACLR and 46dB ACS for both TN and ATG BS for non-synchronized scenarios to derive isolated distance as starting point. NOTE 3: For scenario 11, if simulator doesn’t support additional ring of TN network, i.e. additional 18 site and 54 cells, it’s allowed to double the aggregate interference power from the 57 TN cells to simplify the platform modification, i.e. interference +3dB. |  |  |  |  |  |  |  |  |

6.2 Co-existence simulation assumption

6.2.1 Network layout model

6.2.1.1 Co-existence between ATG and NR terrestrial network

Co-existence modelling is based on positioning a single ATG BS/sector and a TN cluster. Two options exist for the positioning of the TN cluster relative to the ATG BS.

For simulation cases 1, 4, 5, 7, 8, 9, 12, 14 the worst-case network layout for simulation is one in which the TN cluster is placed at the same location as the ATG BS. The ATG BS are offset from the TN BS with 33ISDTN as depicted in figure 6.2.1.1-2.

Figure 6.2.1.1-1 Network layout with TN network located close to ATG BS

Figure 6.2.1.1-2 Offset of ATG BS compared to TN BS grid when the TN cluster is located close to the ATG BS

For the remaining simulation cases, if the ATG UE has an omnidirectional radiation pattern then the worst-case network layout for simulation is one in which the TN is placed directly below the aircraft. If the UE has a directional radiation pattern, then in these cases companies should assess which of the network layout options (TN placed close to ATG BS or TN placed underneath the aircraft) is worst case and apply the worst-case option.

Figure 6.2.1.1-3 Network layout with TN network located directly underneath the aircraft

#### 6.2.1.2 TN Network Layout

A rural hexagonal grid layout is assumed for TN network clusters with the parameters of table 6.2.1.1.

Table 6.2.1.2-1: Simulation scenarios for ATG coexistence study

| Parameters |  | Values |
| --- | --- | --- |
| Network layout |  | For scenario 11: hexagonal grid, 37 macro sites, 3 sectors per site with wrap around For other scenarios: hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance |  | 7.5 km (4GHz) 3.5 km (2GHz) |
| BS antenna height |  | 30 m |
| UE location | Outdoor/indoor | Outdoor only |
|  | Indoor UE ratio | 0% |
|  | LOS/NLOS | LOS and NLOS, see subclause 6.2.5 |
|  | UE antenna height | 1.5m |
| UE distribution (horizontal) |  | Uniform |
| Minimum BS - UE distance (2D) |  | 35 m |

#### 6.2.1.3 ATG Network Layout

A single ATG BS with a single cell is assumed for the ATG network. The aircraft are assumed to fly in a straight line from the minimum distance to the maximum horizontal distance from the ATG BS in the horizontal boresight of the ATG sector. The minimum and maximum horizontal distance of the ATG UE from the ATG BS are as depicted in table 6.2.1.3-1 and depend on the assumption of sub-arrays or not for the antenna model

In the vertical domain, ATG UEs are distributed in height between 3000m and 10000m

Table 6.2.1.2-1: Simulation scenarios for ATG coexistence study

| Parameters | No sub-arrays | Sub-arrays |
| --- | --- | --- |
| Minimum ATG BS-UE horizontal distance | 20km | 50km |
| Maximum ATG BS-UE horizontal distance | 100km | 100km |

Figure 6.2.1.3-1 ATG BS layout (In horizontal boresight direction of ATG antenna)

For Scenarios 2, 3, 10, 11, in addition to simulations in the above assumptions some extra simulations were carried out in which the ATG UE is assumed to be around 300km from the ATG BS. The TN network is also located at 300km, and the ATG UE is assumed to be positioned within a straight line from the ATG BS and within the TN ground network area.

- For Scenario 2,3,10,11, it is agreed to conclude simulation results based on that ATG UEs flying over a TN cluster while ATG BS is 100km away as shown below.

  - Optional: ATG BS is 300km away.

Figure 6.2.1.3-2 ATG BS layout (Case 2,3, 10 and 11)

Issue 1-3-2: location relationship between ATG UE and TN network for non-synchronized scenario to conclude isolation distance

FFS and conclude in May meeting to simulate the worst case.

- ATG UE and TN cluster are on the same side of ATG BS, i.e. ATG BS actually point at TN cluster to consider the worst case.

- Minimum distance between ATG BS and ATG UE is 20/50km

- There is no additional restriction between ATG BS and ATG CPE. Companies are welcome to provide their results for worst case with detailed deployment illustration.

- Following show one example for simulation

  - if isolation distance between TN cluster and ATG BS is larger than 20/50km, ATG UE is always on top of TN cluster

  - if isolation distance between TN cluster and ATG BS is less than 20/50km, ATG UE is dropped 20/50km distant from ATG BS

- The isolation distance is the distance from the ATG BS to the nearest TN BS. The ATG BS sector and nearest TN BS sector should be pointing towards one another in azimuth. The diagram below is illustrative and does not capture this.

### 6.2.2 System parameters

#### 6.2.2.1 ATG parameters

The system parameters for ATG BS for co-existence study are assumed as below.

Table 6.2.2.1-1: system parameters for ATG BS

| ATG BS altitude | 30m |
| --- | --- |
| Carrier frequency | 2GHz, 4GHz |
| Frequency reuse factor | 1 |
| Duplex mode | FDD@2GHz, TDD@4GHz |
| Channel bandwidth | 20MHz@2GHz, 100MHz@4GHz |
| Subcarrier spacing (SCS) | 15k@2GHz, 30k@4GHz |
| Number of cells | one cell |
| UE distribution | Single ATG UE per ATG cell Horizontal: Random between minimum and maximum distance in the straight line within range described in section 6.2.1.3 Vertical: Based on ATG UE uniform distribution among 3 to 10km |
| Indoor UE percentage | 0% |
| Number of DL active UEs per cell (NOTE 2) | one UE |
| Number of UL active UEs per cell (NOTE 2) | one UE |
| DL scheduled bandwidth per UE | Full bandwidth |
| UL scheduled bandwidth per UE | Full bandwidth |
| UL target SNR (NOTE 3) | 15dB |
| Traffic model | Full buffer |
| ATG BS maximum output power | 46dBm sum of two polarizations for 2GHz 53dBm sum of two polarizations for 4GHz |
| ATG BS noise figure | 5dB |
| Handover margin | Not needed |
| NOTE 1: ATG BS is assumed to serve UEs in the rural environment. NOTE 2: Same as the number of BS beam(s). NOTE 3: Target SNR for simulation is based on CL values and only compensates pathloss in the simulation assumptions. |  |

#### 6.2.2.2 ATG UE parameters

The system parameters for ATG UE are assumed as below.

Table 6.2.2.2-1: system parameters for ATG UE

| ATG UE altitude | Vertical: Distributed between 3km and 10km |
| --- | --- |
| Carrier frequency | 2GHz, 4GHz |
| ATG UE max TX power in dBm | 40dBm TRP for 2GHz 43dBm EIRP for 4GHz |
| ATG UE min TX power in dBm | - [-33dBm] for 100MHz - [-40dBm] for 20MHz |
| ATG UE noise figure | 9dB |

#### 6.2.2.3 TN BS and UE parameters

The system parameters for TN BS and TN UE are assumed as below.

Table 6.2.2.3-1: system parameters for TN BS and UE

| Parameters | Rural | Rural |
| --- | --- | --- |
| Carrier frequency | 2GHz | 4GHz |
| Channel bandwidth | 20MHz | 100MHz |
| Scheduled channel bandwidth per UE (DL) | 1 | 1 |
| Scheduled channel bandwidth per UE (UL) | 1 | 1 |
| The number of active UE (DL) (NOTE 1) | 1 | 1 |
| The number of active UE (UL) (NOTE 1) | 1 | 1 |
| Traffic model | full buffer | full buffer |
| DL power control | No | No |
| UL power control | Yes | Yes |
| UL target SNR (NOTE 3) | 15dB | 15dB |
| TN BS-UE min distance in meters | 35m | 35m |
| TN BS max TX power in dBm (NOTE 2) | 46dBm | 53dBm |
| TN UE max TX power in dBm | 23dBm | 23dBm |
| TN UE min TX power in dBm | -40dBm | -40dBm |
| TN BS Noise figure in dB | 5dB | 5dB |
| TN UE Noise figure in dB | 9dB | 9dB |
| Handover margin | 3dB | 3dB |
| NOTE 1: Same as the number of BS beam(s). NOTE 2: TN BS max TX power is defined as the sum over both polarizations. NOTE 3: Target SNR for simulation is based on CL values and only compensates pathloss in the simulation assumptions. |  |  |

6.2.3 Antenna and beamforming pattern modelling

6.2.3.1 ATG BS antenna model

For ATG BS antenna modelling, the following two options for antenna modelling could be used for ATG coexistence study.

Option 1: non sub-array model

Table 6.2.3.1-1: AAS antenna parameters for non sub-array model

|  | ATG |
| --- | --- |
| Base Station Antenna Characteristics |  |
| Antenna pattern | TR 38.921 |
| Element gain (dBi) (Note 2) | 7.1 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 54º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) (Note 4) | 8 × 8 elements |
| Number of supported polarizations, P | 2 |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.9 of wavelength for V |
| Array Ohmic loss (dB) (Note 2) | 2 |
| Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 25 |
| Base station maximum coverage angle in the horizontal plane (degrees) | 120 |
| Base station vertical coverage range (degrees) (Note 1) | 25 |
| Mechanical uptilt (degrees) | 14 |

Option 2: sub-array model

Table 6.2.3.1-1: AAS antenna parameters for sub-array model

| Parameter | Macro urban |
| --- | --- |
| Element gain (dBi) (Note 2) | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna sub-array configuration (Row × Column) (Note 4) | 4 × 8 elements |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 2.1 of wavelength for V |
| Number of element rows in sub-array | 3 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V |
| Array Ohmic loss (dB) (Note 2) | 2 |
| Conducted power (before Ohmic loss) per sub-array (dBm) (Note 3) | 28 |
| Base station horizontal coverage range (degrees) | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 10 |
| Mechanical up-tilt (degrees) | 6.5 |

6.2.3.2 ATG UE antenna model

For 2GHz, assume omni-directional antenna, assume [40dBm] UE output power for calibration (as worst case for simulation purposes).

For 4GHz, assume that UE is equipped with directional antenna, assume a UE EIRP of [43dBm] for calibration (as worst case for simulation purposes)

  - Use following as the starting point for calibration.

Table 6.2.3.2-1:  antenna parameters for phase antenna array

|  | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 90º for V |
| --- | --- | --- |
|  | Element gain (dBi) | 5 dBi |
|  | Horizontal/vertical front-to-back ratio (dB) | 30dBc |
|  | Antenna polarization | Linear ±90º |
|  | Antenna array configuration (Row × Column x Polarization) (Note 4) | (8x2x2) or (16x1x2) |
|  | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.5 of wavelength for V |
|  | UE antenna orientation | Single UE panel deployed on the abdomen of the airplane facing downwards and with the longest dimension of the array aligned with the direction of the flight route. The flight route is pointed at the BS. |

6.2.3.3 TN BS antenna model

For TN BS antenna modelling, the following two options for antenna modelling could be used for ATG coexistence study.

Option 1: non sub-array model

Table 6.2.3.3-1: Antenna parameters for non sub-array model

|  | TN |
| --- | --- |
| Base Station Antenna Characteristics |  |
| Antenna pattern | TR 38.921 |
| Element gain (dBi) (Note 2) | 7.1 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 54º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) (Note 4) | 8 × 8 elements AAS [8 x 1 elements non AAS] |
| Number of supported polarizations, P | 2 |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.9 of wavelength for V |
| Array Ohmic loss (dB) (Note 2) | 2 |
| Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 25 |
| Base station maximum coverage angle in the horizontal plane (degrees) | 120 |
| Base station vertical coverage range (degrees) (Note 1) | 25 |
| Mechanical down (degrees) | 3 |

Option 2: sub-array model

Table 6.2.3.3-1: Antenna parameters for sub-array model

| Parameter | Macro urban |
| --- | --- |
| Element gain (dBi) (Note 2) | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna sub-array configuration (Row × Column) (Note 4) | 4 × 8 elements AAS [4x1 elements non-AAS] |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 2.1 of wavelength for V |
| Number of element rows in sub-array | 3 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V |
| Array Ohmic loss (dB) (Note 2) | 2 |
| Conducted power (before Ohmic loss) per sub-array (dBm) (Note 3) | 28 |
| Base station horizontal coverage range (degrees) | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 10 |
| Mechanical downtilt (degrees) | 3 |

6.2.3.4 TN UE antenna model

The following assumption for TN UE antenna is shown as below.

Table 6.2.3.4-1: TN handheld UE antenna charateristic

| Characteristics | Handheld |
| --- | --- |
| Antenna type and configuration | (1, 1, 2) with omni-directional antenna element |
| Polarisation | Linear: +/-45°X-pol |
| Tx/Rx Antenna gain | 0 dBi per element |
| the number of Tx and Rx | 1T2R |

6.2.4 ACLR and ACS modelling

For DL it seems reasonable from the perspective of simulating worst case scenarios that we assume BS ACLR is modelled as flat in space, and the UE ACS can be modelled flat in space.

If this assumption is for DL, then the similar assumption could be made for the UL.

Therefore, it is assumed that both ACLR ( or the adjacent channel interference) and ACS are flat in both space and frequency. The ACIR model can be express as:

(Assuming ACLR, ACS and ACIR to be linear).

The ACLR/ACS requirements for TN are defined as below.

Table 6.2.4-1: ACLR/ACS requirements for TN

| NR TN system |  |  |
| --- | --- | --- |
| BS | ACLR | 45 dB |
|  | ACS | 46 dB |
| UE | ACLR | 30dB (ACLR1) 43dB (ACLR2) |
|  | ACS | 33 |

6.2.5 Propagation model

6.2.5.1 Propagation model between TN UE and ATG UE

Referring to section 6.6 in TR 38.811, the propagation model between TN UE and ATG UE can be simplified and summarized as below.

LOS probability

Line-Of-Sight (LOS) probability depends on UE environment and elevation angle, and is obtained from Table 6.2.5.1-1. Reference elevation angles are considered from 10° to 90° with a 10° step. For an ATG UE-to-ATG BS, the LOS probability is taken from the nearest reference elevation angle.

Table 6.2.5.1-1 LOS probability

| Elevation | Suburban and Rural scenarios |
| --- | --- |
| 10° | 78.2% |
| 20° | 86.9% |
| 30° | 91.9% |
| 40° | 92.9% |
| 50° | 93.5% |
| 60° | 94.0% |
| 70° | 94.9% |
| 80° | 95.2% |
| 90° | 99.8% |

Path loss and Shadow fading

The signal path between ATG UE and ATG BS undergoes several stages of propagation and attenuation. The path loss (PL) is composed of components as follows:

PL=PLb=FSPLd, fc+SF+CLα, fc,	(6.2.5.1-1)

where	is the total path loss in dB,

is the basic path loss in dB,

This section specifies the basic path loss model () which accounts for the signal's free space propagation, clutter loss, and shadow fading.

The free space path loss (FSPL) in dB for a separation distance d (between ATG UE and ATG BS) in meter and frequency fc in GHz is given by

(6.6-2)

is clutter loss, and  is shadow fading loss represented by a random number generated by the normal distribution, i.e.,  ~. When the UE is in LOS condition, clutter loss is negligible and should be set to 0 dB in the basic path loss model.

Table 6.2.5.1-2: Shadow fading and clutter loss for suburban and rural scenarios

| Elevation | 2GHz 3.5GHz |  |  |
| --- | --- | --- | --- |
|  | LOS | NLOS |  |
|  | (dB) | (dB) | (dB) |
| 10° | 1.79 | 8.93 | 19.52 |
| 20° | 1.14 | 9.08 | 18.17 |
| 30° | 1.14 | 8.78 | 18.42 |
| 40° | 0.92 | 10.25 | 18.28 |
| 50° | 1.42 | 10.56 | 18.63 |
| 60° | 1.56 | 10.74 | 17.68 |
| 70° | 0.85 | 10.17 | 16.50 |
| 80° | 0.72 | 11.52 | 16.30 |
| 90° | 0.72 | 11.52 | 16.30 |

6.2.5.2 Propagation model between TN BS and TN UE

Referring to section 7.4 in TR 38.901, the propagation model between TN BS and TN UE can be summarized as below, which is same as RMa scenario.

Pathloss:

The pathloss models are summarized in Table 6.2.5.2-1 and the distance definitions are indicated in Figure 6.2.5.2-1 and Figure 6.2.5.2-2. Note that the distribution of the shadow fading is log-normal, and its standard deviation for each scenario is given in Table 6.2.5.2-1.

| Figure 6.2.5.2-1: Definition of d2D and d3D for outdoor UTs | Figure 6.2.5.2-2: Definition of d2D-out, d2D-in and d3D-out, d3D-in for indoor UTs. |
| --- | --- |

Note that

(7.4-1)

Table 6.2.5.2-1: Pathloss models

| Scenario | LOS/NLOS | Pathloss [dB], fc is in GHz and d is in meters, see note 6 | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- | --- |
| RMa | LOS | , see note 5 |  | h = avg. building height W = avg. street width The applicability ranges: |
|  | NLOS | for |  |  |
| Note 1: Breakpoint distance d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. The effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and hE is the effective environment height. For UMi hE = 1.0m. For UMa hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. With C(d2D, hUT) given by , where . Note that hE depends on d2D and hUT and thus needs to be independently determined for every link between BS sites and UTs. A BS site may be a single BS or multiple co-located BSs. Note 2: The applicable frequency range of the PL formula in this table is 0.5 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS pathloss is from TR36.873 with simplified format and PLUMa-LOS = Pathloss of UMa LOS outdoor scenario. Note 4: PLUMi-LOS = Pathloss of UMi-Street Canyon LOS outdoor scenario. Note 5: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0 108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. Note 6: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |  |

LOS probability

The Line-Of-Sight (LOS) probabilities are given in Table 6.2.5.2-2.

Table 6.2.5.2-2 LOS probability

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| RMa |  |

O2I building penetration loss

The pathloss incorporating O2I building penetration loss is modelled as in the following:

(6.2.5.2-2)

where  is the basic outdoor path loss given in Clause 6.2.5.2, where  is replaced by  .  is the building penetration loss through the external wall,  is the inside loss dependent on the depth into the building, and σP is the standard deviation for the penetration loss.

is characterized as:

(6.2.5.2-3)

is an additional loss is added to the external wall loss to account for non-perpendicular incidence; , is the penetration loss of material i, example values of which can be found in Table 7.4.3-1;  is proportion of i-th materials, where ; and N is the number of materials.

Table 6.2.5.2-3: Material penetration losses

| Material | Penetration loss [dB] |
| --- | --- |
| Standard multi-pane glass |  |
| IRR glass |  |
| Concrete |  |
| Wood |  |
| Note: f is in GHz |  |

Table 6.2.5.2-4 gives ,  and σP for two O2I penetration loss models. The O2I penetration is UT-specifically generated, and is added to the SF realization in the log domain.

Table 6.2.5.2-4: O2I building penetration loss model

|  | Path loss through external wall: in [dB] | Indoor loss: in [dB] | Standard deviation: σP in [dB] |
| --- | --- | --- | --- |
| Low-loss model |  | 0.5 | 4.4 |
| High-loss model |  | 0.5 | 6.5 |

is minimum of two independently generated uniformly distributed variables between 0 and 25 m for UMa and UMi-Street Canyon, and between 0 and 10 m for RMa.  shall be UT-specifically generated.

Both low-loss and high-loss models are applicable to UMa and UMi-Street Canyon.

Only the low-loss model is applicable to RMa.

6.2.5.3 Propagation model between ATG BS and TN BS

The propagation model between ATG BS and TN BS is same as the LOS propagation model between TN UE and ATG UE in clause 6.2.5.1 (shadow fading and clutter loss are assumed to be zero, since the ATG BS and TN BS are at 30m above the clutter).

6.2.5.4 Propagation model between ATG BS and TN UE

The propagation model between ATG BS and TN UE is same as the propagation model between TN BS and TN UE in clause 6.2.5.2

6.2.5.5 Propagation model between TN BS and ATG UE

The propagation model between TN BS and ATG UE is same as the LOS propagation model between TN UE and ATG UE in clause 6.2.5.1 (shadow fading and clutter loss are assumed to be zero, since the ATG BS and TN BS are at 30m and above the clutter).

6.2.5.6 Propagation model between ATG BS and ATG UE

The propagation model between ATG BS and ATG UE is same as the LOS propagation model between TN UE and ATG UE in clause 6.2.5.1 (shadow fading and clutter loss are assumed to be zero, since the ATG BS and TN BS are at 30m and above the clutter).

6.2.6 Transmission power control model

6.2.6.1 TN UL TPC

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 is applied for TN with following parameters.

where:

-	Pmax = 23dBm,

-	Rmin = -40dBm,

-	CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y, where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1 For uplink scenario.

6.2.6.2 TN DL TPC

For downlink scenario, no power control scheme is applied.

6.2.6.3 ATG UL TPC

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 is applied for TN with following parameters.

where:

-	Pmax is ATG UE maximum output power (TRP) dBm,

-	Rmin is ATG UE minimum output power (TRP) dBm,

-	CLx-ile and γ are set as following:

-	CLx-ile = 10*log10(Pmax) – (SNRtarget + (-174+F+10*log(B)) )

-	SNRtarget is the targeted UL SNR (dB).

-    F is BS noise figure (dB).

-    B is UL transmission BW (Hz)

-	γ = 1 for uplink scenario.

The specific parameters are assumed as below in table 6.2.6.3.

Table 6.2.6.3-1 LOS probability

| UE UL power control parameters | 2GHz | 4GHz |
| --- | --- | --- |
| Target SNR | 15dB | 15dB |
| Pmax (TRP) | 40dBm | 23dBm for per polarization |
| BW | 20MHz | 100MHz |
| Rmin | -27dBm | -20dBm |
| NF for ATG BS | 5dB | 5dB |
| CLx_ile | 121dB | 97 |

6.2.6.4 ATG DL TPC

For downlink scenario, no power control scheme is applied.

6.2.7 Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

where:

-	RX_PWR is the received power

-	TX_PWR is the transmitted power

-	G_TX is the transmitter antenna gain (directional array gain)

-	G_RX is the receiver antenna gain (directional array gain).

6.2.8 Performance metric

For NR, the average throughput loss and 5%-ile throughput loss should be less than 5%.

For ATG, the average throughput loss and 5%-ile throughput loss should be less than 5%.

For the TN network, the average throughput loss should be calculated in each of the TN cells. Results should be presented for the average throughput loss in the worst case TN cell, and an average of the average throughput losses in all of the TN cells in the cluster may also be presented. The details could be found as following:

- 5% and 50% in the whole network (When TN as victim, 5% and 50% are 5% ad 50% of the whole TN. When ATG as victim, 5% and 50% are the 5% and 50% among all drops.)

- 5% and 50% of users within the cell with largest throughput loss for the case of TN DL victim

6.2.9 Link level performance for NR ATG coexistence

The throughput of a modem with link adaptation can be approximated by an attenuated and truncated form of the Shannon bound. (The Shannon bound represents the maximum theoretical throughput than can be achieved over an AWGN channel for a given SNIR). The following equations approximate the throughput over a channel with a given SNIR, when using link adaptation:

Throughput SNIR, bps/Hz =0                                  for SNIR< SNIRMIN                                        ∝∙SSNIR                     for SNIRMIN≤SNIR<SNIRMAX ∝∙SSNIRMAX               for SNIR ≥ SNIRMAX

Where:

-	S(SNIR)		Shannon bound, S(SNIR) =log2(1+SNIR) bps/Hz

-					Attenuation factor, representing implementation losses

-	SNIRMIN		Minimum SNIR of the code set, dB

-	SNIRMAX		Maximum SNIR of the code set, dB

The parameters α, SNIRMIN and SNIRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in Table 4.2.7-1 represent a baseline case, which assumes:

-	1:1 antenna configuration

-	AWGN channel model

-	Link Adaptation (see Table 4.2.7-1 for details of the highest and lowest rate codes)

-	No HARQ

Table 6.2.9-1: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

Note that the parameters proposed in Table 4.2.7-1 are targeted for eMBB coexistence scenario.

6.3 Co-existence simulation methodology

Adopt following simulation steps.

-	Step 1: Generate aggressor and victim networks.

-		One ATG site with one sector is dropped referring to clause 6.2.1.3

- 	Deployment of terrestrial network (19 cells with wraparound) refers to Table 6.2.1.2-1

- 	The relationship between TN and ATG can refer to clause 6.2.1.1

-	Step2: UE associations

-	TN UE are generated randomly inside the TN network, make sure enough TN UEs are associated to each TN sectors based on coupling loss.

-	Deployment of ATG UE refers to clause 6.2.1.3.

-	Step 3: Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS/ATG BS-UE. This is done for both victim and aggressor networks.

-	Step 4: Throughput is computed in the victim systems without considering ACI as below:

ThputNO ACIbpshz=fSINRICI=fSN+IICI,

where:			IICI is the inter-cell interference.

-	Step 5: Throughput is computed considering ACI as below:

ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI,

where:			IACI is the adjacent channel interference.

-	Step 6: RF parameters are determined based on the degradation cause by ACI as below:

LossACI=1-ThputACIThputSINGLE

----- End of TP -----