# 3GPP TSG-RAN WG4 Meeting # 107						R4-2309207

# Incheon, Korea, 22 May - 26 May , 2023

Source:		Ericsson

Title:			LS reply for the switching time for RedCap UE positioning

Agenda item:		8.23.3.4

Document for:		Approval

# Introduction

In RAN4#106-bis-e LS[5] and WF[6], the switching time for SRS/PRS hopping is agreed to be a set of limited number and in this paper, we continue to present our view on the LS question from RAN1 for RedCap positioning[1].

# Discussion

In LS[1], the question below is asked from RAN1:

- RAN1 kindly asks RAN4 for feedback on the feasible values for the switching time between hops, at least when numerology and bandwidth for each hop can be the same, and the Tx/Rx antennas used in all hops can be the same.

According to RAN1 agreement, the short RF retuning time between adjacent hops is preferred for both UL SRS TX and DL PRS Rx hopping.

Agreement

For Positioning enhancements for redcap UEs for UL SRS Tx and DL PRS Rx frequency hopping, from the RAN1 perspective, short switching time to allow RF retuning between adjacent hops may be beneficial in terms of accuracy and latency performance.

There is no quantitative magnitude in RAN 1 agreement on the total duration of the frequency hopping. Though not mentioned by LS, such aggregation assumes the channel does not change within the whole duration of frequency hopping. As the channel status depends on the coherence time of the channel and such time relates to the doppler spread of UE caused by mobility, it can be easily understood that if RF tuning time would be long, the total number of frequency hops will be reduced to accommodate the coherence time of the channel and thus the accuracy of positioning will be affected.

The total hopping time is bounded by coherence time of the channel and shorter RF tuning time benefit the positioning accuracy by increased aggregated frequency range, hence increased # of hop.

Figure 1: UL SRS frequency hopping with partial overlap

In WF[6] the switching time for SRS is {70us, 140us} for FR1 and {35us, 70us, 140us} for FR2 but whether multiple or single value for switching time is remaining to be agreed.  In 36.211, the RF tuning time for Tx-to-Tx is either 1 or 2 symbols, as NR symbol time with normal CP for SCS 15kHz is 71.35us, at least the 70us and 140us is in par with LTE-M RF retuning time. However, it also should be discussed what would be expected for positioning performance when 2 symbols RF retuning would be allowed. The needed RF tuning times for different SCS is listed in Table 1 below:

Table 1: RF retuning time mapping to # of OFDM symbols

| Parameter / Numerlogy (u) | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| SCS | 15kHz | 30kHz | 60kHz | 120kHz | 240kHz |
| OFDM Symbol Duration (us) | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 |
| Cyclic Prefix Duration (us) | 4.69 | 2.34 | 1.17 | 0.57 | 0.29 |
| OFDM Symbol including CP (us) | 71.35 | 35.68 | 17.84 | 8.92 | 4.46 |
| RF tuning time (70us, FR1) | 1 | 2 | 4 |  |  |
| RF tuning time (140us, FR1) | 2 | 4 | 8 |  |  |
| RF tuning time (35us, FR2) |  |  | 2 | 4 | 8 |
| RF tuning time (70us, FR2) |  |  | 4 | 8 | 16 |
| RF tuning time (140us, FR2) |  |  | 8 | 16 | 32 |

Because of the different # of symbols required for RF tuning, this will create complex scheduling to multiplex the different UE with different RF retuning time. As illustrated in Figure 1, when a network has different RF tuning time capability UE, network can multiplex the RedCap UE with different RF tuning time for SRS hopping orthogonally in time domain, so the same RF retuning time UE, they could be scheduled at same symbol in time domain but different RB in frequency domain. To do so, as illustrated in Figure 1, there could be a case where there are more symbols between two SRS hops and therefore UE may be scheduled with a larger time gap than the required RF retuning time. This creates a case where the UE have bigger timing gap than necessary between SRS hops. In Figure 2, it means that for a UE with 4 symbols Re-tuning time, the possible SRS position will be limited to 2 when multiplexing together with other 2 UE with 2 symbols retuning time.  This will orverall limit the possibility of the multiplexing the UEs in time domain and increase the SRS hopping duration for some UE For FR2, the doppler frequency is much higher for higher frequency and thus coherence time even shorter, too long RF retuning basically means losing the coherent time for the channel. In Table 2, the coherence time is compared for different UE speed and frequency. It can be observed that for FR1, if UE speed is higher than 30m/s, only 1 subframe of SCS of 15kHz would be usable for SRS hopping. For FR2, 1 slot for speed of below 10m/s.

Table 2: coherence time comparison

| Frequency /UE speed |  | 3m/s | 10 m/s (36km/h) | 30m/s | 300m/s |
| --- | --- | --- | --- | --- | --- |
| 2GHz | Doppler (Hz) | 40 | 133 | 400 | 4000 |
|  | Coherence time (ms) | 10.6 | 3.18 | 1.06 | 0.106 |
| 28GHz | Doppler(Hz) | 560 | 1866 | 5600 | 56000 |
|  | Coherence time | 0.76 | 0.22 | 0.076 | 0.0076 |
|  | # of Slots for SCS 120kHz | 6 | 1.76 | 0.6 | 0.06 |

Multiplexing UEs with  different RF tuning time may reduce configuration flexibility on the number of SRS symbols within a slot and also increase the SRS hopping duration for some UE

The number of the slots to be configured for SRS hopping depend on UE speed and frequency, which generally demand less SRS hopping duration

Figure 1: Multiplexing 2 UE with different RF retuning for FDD band for SRS hopping

Figure 2: Multiplexing 3 UE with different RF retuning for TDD band for SRS hopping

- If UE reports higher RF tuning time, it will reduce the appliance of the SRS/PRS hopping as only it can be usable for low UE speed. Considering the possible longer SRS hopping duration when mulplexing the different RF retuning time UE and also the need of shorten the SRS hopping duration to increase the usage of this feature, we believe only faster RF retuning will be necessary for this feature.  From above analysis, we suggest to agree the FR1 retuning with 70us only and for FR2, agree with 35 us only.

For UL SRS Tx frequency hopping, use the switching time of 70us for FR1.

For DL PRS frequency hopping, use the switching time of 70us for FR1.

For UL SRS Tx frequency hopping, use the switching time of 35 us for FR2.

For DL SRS Tx frequency hopping, use the switching time of 35us for FR2.

# Conclusions

In this contribution, we present our view on the switching time for RedCap positioning with LS reply in annex and with below proposal and observations:

Observation 1 The total hopping time is bounded by coherence time of the channel and shorter RF tuning time benefit the positioning accuracy by increased aggregated frequency range, hence increased # of hop.

Observation 2 Multiplexing different RF tuning time UE may reduce configuration flexibility on the number of SRS symbols within a slot and also increase the SRS hopping duration for some UE

Observation 3The number of the slots to be configured for SRS hopping depend on UE speed and frequency, which generally demand less SRS hopping duration

Proposal-1: For UL SRS Tx frequency hopping, use the switching time of 70us for FR1.

Proposal-2: For DL PRS frequency hopping, use the switching time of 70us for FR1.

Proposal-3: For UL SRS Tx frequency hopping, use the switching time of 35 us for FR2.

Proposal-4: For DL SRS Tx frequency hopping, use the switching time of 35us for FR2.

# References

R1-2302127, LS on switching time for DL PRS or UL SRS frequency hopping for RedCap UEs, Ericsson

R4-1702029, Reply LS on UE RF Bandwidth Adaptation in NR

R4-1700645, RF aspects of bandwidth adaptation, Ericsson

R4-2103290, Reply LS on Beam switching gaps for Multi-TRP UL transmission

R4-2306659, LS reply for the switching time for RedCap UE positioning, Ericsson

R4-2306620, WF on UE RF aspects for Rel-18 NR positioning

# Annex

3GPP TSG-RAN WG4 Meeting # 107							R4-230xyz

Incheon, Korea, 22 May - 26 May , 2023

Title:	LS reply on switching time for DL PRS or UL SRS frequency hopping for RedCap UEs

Response to:	R1-2302127

Release:	Rel-18

Work Item:	NR_pos_enh2

Source:	RAN WG4

To:	RAN WG1

Cc:

Contact Person:

Name:	Chunhui Zhang

E-mail Address:	Chunhui (dot) Zhang (at) ericsson (dot) com

Attachments:

1. Overall Description:

RAN4 thanks RAN1 for LS R1-2302127. RAN4 has discussed the question in the LS and reached the following agreements:

- For RedCap UE UL SRS Tx frequency hopping, RAN4 agrees the switching time of 70us for FR1

- For UL SRS Tx frequency hopping, RAN4 agrees the switching time of 35us for FR2

- For RedCap UE DL PRS Rx frequency hopping, RAN4 agrees the switching time of  70us for FR1

- For DL PRS Rx frequency hopping, RAN4 agrees the switching time of 35us for FR2

2. Actions:

To RAN WG1 group.

ACTION: 	no action needed.

3. References:

4. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #108		August 21 – 25, 2023				Toulouse

TSG RAN WG4 Meeting #108-bis		Oct 9 – 13, 2023					Xiamen