3GPP TSG-RAN4 Meeting #107	R4-2309255

# Incheon, South Korea,  22 - 26 May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1606 | rev | - | Current version: | 16.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-1 Rel-16 Cat F, MSD correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_R16_2BDL_xBUL-Core |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Harmonic mixing MSD is missing from CA_n28-n77. It is specified for corresponding EN-DC combination. CA_n5-n77 entries are offset by one cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Harmonic mixing MSD is added for CA_n28-n77 by using the same MSD value as specified for corresponsing EN-DC combination. CA_n5-n77 entries are offset by one cell and correct SCS is included. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MSD requirement is missing and NR single-RAT specification is misaligned from EN-DC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 7.3A.4	Reference sensitivity exceptions due to UL harmonic interference for CA

Sensitivity degradation is allowed for a band in frequency range 1 if it is impacted by UL harmonic interference from another band in frequency range 1 of the same CA configuration. Reference sensitivity exceptions are specified in Table 7.3A.4-1 with uplink configuration specified in Table 7.3A.4-2.

Table 7.3A.4-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

|  |  | MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB |  | dB | dB | dB |
| n1 | n771,2 |  |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n773 |  |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n2 | n481, 2 |  | 27.1 | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.912 | 16.112 |  | 14.812 | 14.312 | 13.812 |
|  | n483 |  | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n2 | n771, 2 |  |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.5 | 14.8 | 14.3 | 13.8 |
|  | n773 |  |  | 1.1 | 0.8 | 0.3 | 0.1 |  |  |  |  |  |  |  |  |
| 2 | n781,2 |  |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n3 | n771,2 |  |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.9 | 16.1 |  | 14.8 | 14.3 | 13.8 |
|  | n773 |  |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
|  | n781,2 |  |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.9 | 16.1 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n5 | n774, 5,13 |  |  | 10.5 | 8.9 | 7.8 | 7.2 | 6.5 | 5.1 | 4.2 | 3.5 | 2.8 | 2.3 | 2.1 | 1.4 |
| n5 | n776,7,13 |  |  | 10.4 | 8.9 | 6.7 | 6.0 | 6.5 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| n5 | n784,5 |  |  | 10.5 | 8.9 | 7.8 |  |  | 5.4 | 4.2 | 3.5 |  | 2.3 | 2.1 | 1.4 |
| n8 | n311 |  | N/A | N/A | N/A | N/A | N/A | N/A |  |  |  |  |  |  |  |
|  | n418,9 |  |  | 13.0 | 11.3 | 10.1 |  |  | 7.0 | 6.1 | 5.5 |  | 4.3 | 3.9 | 3.5 |
|  | n784,5 |  |  | 10.8 | 9.1 | 8.0 |  |  | 5.1 | 4.2 | 3.5 |  | 2.3 | 2.1 | 1.4 |
|  | n796,7 |  |  |  |  |  |  |  | 6.8 | 6.2 | 5.6 |  | 4.9 |  | 4.4 |
| n20 | n784,5 |  |  | 10.8 | 9.1 | 8 |  |  | 6 | 4.0 | 3.2 |  | 2.0 | 1.5 | 1.0 |
| 25 | n781,2 |  |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n28 | n18,9 |  | 10.2 | 7.6 | 6.2 | 5.3 |  |  |  |  |  |  |  |  |  |
|  | n501,2 |  |  | 19.8 | 18.0 | 16.8 |  |  | 13.8 | 12.8 | 12.0 |  | 10.8 |  |  |
|  | n751,2 |  | 28.1 | 25.3 | 24.0 | 22.8 | 21.8 | 21.0 | 19.7 | 18.7 |  |  |  |  |  |
|  | n776,7 |  |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 |  | 1.7 | 1.2 | 0.7 |
|  | n786,7 |  |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 |  | 1.7 | 1.2 | 0.7 |
| n66 | n481, 2 |  | 27.1 | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.912 | 16.112 |  | 14.812 | 14.312 | 13.812 |
|  | n483 |  | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n66 | n771, 2 |  |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.3 | 14.8 | 14.3 | 13.8 |
|  | n773 |  |  | 1.1 | 0.8 | 0.3 | 0.1 |  |  |  |  |  |  |  |  |
| n66 | n781,2 |  |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n71 | n2510 |  | 10 | 7.5 | 6 | 5.1 |  |  |  |  |  |  |  |  |  |
|  | n414,5 |  |  | 10.8 | 9.1 | 8.0 |  |  | 5.1 | 4.2 | 3.5 |  | 2.3 | 2.1 | 1.4 |
|  | n708,9 |  | 9.9 | 7.1 | 6.7 | 4.9 | 4.1 |  |  |  |  |  |  |  |  |
| n92 | n784,5 |  |  | 10.8 | 9.1 | 8 |  |  | 6 | 4.0 | 3.2 |  | 2.0 | 1.5 | 1.0 |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the band combination: ∆FHD = 10 MHz for CA_n1-n77, CA_n2-n78, CA_n3-n77, CA_n3-n78, CA_n2-n48, CA_n25-n78, CA_n48-n66, CA_n66-n78. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 5: The requirements should be verified for UL NR-ARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 7: The requirements should be verified for UL NR-ARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 9: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 10: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz. NOTE 11: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.2 apply unless otherwise specified). NOTE 12: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.4-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA, FR1

|  | NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band |  | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1 |  | n77 |  | 25 | 36 | 50 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n2 |  | n48 | 25 | 50 | 50 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n2 |  | n77 |  | 25 | 36 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| n2 |  | n78 |  | 25 | 36 | 50 | 50 | 50 | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n3 |  | n77 |  | 25 | 36 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n3 |  | n78 |  | 25 | 36 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n5 |  | n77 |  | 16 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| n5 |  | n78 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n8 |  | n41 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n8 |  | n78 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n8 |  | n79 |  |  |  |  |  |  | 25 | 25 | 25 |  | 25 |  | 25 |
| n20 |  | n78 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n25 |  | n78 |  | 25 | 36 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n28 |  | n1 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |  |
| n28 |  | n50 |  | 25 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 |  |  |
| n28 |  | n75 | 12 | 25 | 36 | 50 | 50 | 50 | 50 | 50 |  |  |  |  |  |
| n28 |  | n77 |  | 10 | 15 | 20 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n28 |  | n78 |  | 10 | 15 | 20 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n66 |  | n48 | 12 | 25 | 36 | 50 |  |  | 100 | 128 | 160 |  | 200 | 200 | 200 |
| n66 |  | n77 |  | 25 | 36 | 50 | 64 | 80 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n66 |  | n78 |  | 25 | 36 | 50 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n71 |  | n25 | 84 | 84 | 84 | 84 |  |  |  |  |  |  |  |  |  |
| n71 |  | n41 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n71 |  | n70 | 8 | 16 | 20 | 20 | 20 |  |  |  |  |  |  |  |  |
| n92 |  | n78 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
|  | NOTE 1: 15 kHz SCS is assumed for UL band. NOTE 2: The UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. NOTE 3: Unless stated otherwise, UL resource blocks shall be centred within the transmission bandwidth configuration for the channel bandwidth. NOTE 4: These requirements apply when the lower edge frequency of the uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1990 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.4-3: Void

Table 7.3A.4-3a: Void

Sensitivity degradation is allowed for a band if it is impacted by receiver harmonic mixing due to another band part of the same CA configuration. Reference sensitivity exceptions are specified in Table 7.3A.4-4 with uplink configuration specified in Table 7.3A.4-4a.

Table 7.3A.4-4: Reference sensitivity exceptions due to harmonic mixing for CA in NR FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz(dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz(dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n25 | n713,4 | 26.5 | 23.3 | 20.9 | 15.3 |  |  |  |  |  |  |  |  |  |
| n40 | n284 | 37.8 | 34.8 | 33 | 30.3 |  |  |  |  |  |  |  |  |  |
| n40 | n781 |  | 8.3 | 8.0 | 6.9 |  |  | 3.9 | 3 | 2.3 |  | 1.2 |  | 0.4 |
| n41 | n781 |  | 8.3 | 8.0 | 6.9 |  |  | 3.9 | 3 | 2.3 |  | 1.2 |  | 0.4 |
| n77 | n2 | 6.7 | 5.0 | 4.0 | 3.7 |  |  |  |  |  |  |  |  |  |
| n77 | n5 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |  |  |
| n77 | n285 | 28 | 25 | 23.2 | 22 |  |  |  |  |  |  |  |  |  |
| n78 | n402 | 10.4 | 10.4 | 10.4 | 10.4 |  |  | 7.2 | 6.2 | 5.5 |  | 4.5 |  |  |
| n78 | n412 |  | 10.4 | 10.4 | 10.4 |  |  | 8.2 | 7.6 | 7.3 |  | 6.6 | 6.4 | 6.3 |
| n79 | n85 | 25 | 21.8 | 19.4 | 13.8 |  |  |  |  |  |  |  |  |  |
| NOTE 1: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.4-4a: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for CA in NR FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n25 | n71 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n40 | n28 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n40 | n78 | 30 |  | 24 | 24 | 24 |  |  | 24 | 24 | 24 |  | 24 |  | 24 |
| n41 | n78 | 30 |  | 24 | 24 | 24 |  |  | 24 | 24 | 24 |  | 24 |  | 24 |
| n77 | n2 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n77 | n5 | 15 | 25 | 25 | 20 | 20 |  |  |  |  |  |  |  |  |  |
| n77 | n28 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n78 | n40 | 30 | 50 | 50 | 50 | 50 |  |  | 50 | 50 | 50 |  | 50 |  |  |
| n78 | n41 | 30 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n79 | n8 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.3A.5	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA