3GPP TSG-RAN WG4 Meeting #107-e	R4-2309280

Incheon, KR, May 2023

Title:	On configured Tx power for STxMP in FR2

Source:	Qualcomm Incorporated

Agenda item:	8.30.2.1

Document for:	Approval

# 1.	Introduction

The LS from RAN1 [1] sought to determine from RAN4 what constraints govern UL power for an FR2 UE under conditions of simultaneous UL transmission across multiple UE panels (STxMP). The RAN1 enquiry is thought to be relevant their power-control framework which can be simplified if it is possible to specify a per-TCI state configured power requirement. We previously concluded [3,4] it was feasible to simultaneously enable per-panel EIRP that individually approached the regulatory limit while still ensuring that the UE did not exceed regulation.

In this contribution, we confirm our earlier assertion that MPR and AMPR can indeed by defined per TCI-state power.

# 2.	Discussion

We first refine the version of the previously proposed requirement [1] for simultaneous transmission (STxMP) by incorporating useful elements from other comments and proposals:

### 6.2x.4 Configured transmitted power for [STxMP] The UE can configure its maximum output power for each UL TCI-state indicated for [STxMP]. The configured UE maximum output power PCMAX,f,c,k for TCI state k of carrier f and serving cell c defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement for TCI state k as specified in TS 38.215 [11]. The configured UE maximum output power PCMAX,f,c,k shall be set such that the corresponding measured peak EIRP PUMAX,f,c,k for each of the active TCI states k indicated for [STxMP] is within the following bounds PPowerclass + DPIBE – MAX(MAX(MPRf,c,k, A- MPRf,c,k) + ΔMBP,n, P-MPRf,c,k) – MAX{T(MAX(MPRf,c,k, A- MPRf,c,k)), T(P-MPRf,c,k)} -[∆TSTxMP] ≤ PUMAX,f,c,k ≤ EIRPmax and where the corresponding measured peak EIRP for carrier f of a serving cell c, over all active TCI states indicated for [STxMP], PUMAX,f,c satisfies PUMAX,f,c ≤ EIRPmax while the corresponding measured total radiated power PTMAX,f,c is bounded by PTMAX,f,c ≤ TRPmax [where ∆TSTxMP is a relaxation specific to STxMP operation]

The formulation above assumes the existence of per-TCI state MPR and A-MPR. The assumption however would benefit from discussion on its feasibility. Confirmation of a per-TCI state MPR and A-MPR would help definitively address the question of whether per TCI-state configured power can be adopted as the working assumption for STxMP.

At this point it is useful to establish that the UE has at least as many logical panels for Tx as there are layers over all TCI states scheduled for STxMP. This observation follows from the panel definition [5,6] which establishes that a panel can only make one independently steerable spatial filter (‘beam’) at a time and is defined per polarization (‘Confirm that a physical panel with dual polarization is assumed as two “panels”.’ [6]). In a logical sense, each panel is assigned just one TCI-state with independent beam steering control. If therefore follows that multiple panels cannot share any hardware resources between the phase shifters (beam selector) and the radiation interface boundary. This further means that no PA in the UE is required to transmit UL from both TCI states at the same time. This separation of UL signals among different PAs is used as foundation for the per-TCI state MPR formulation. The implication of this separation is that even when the 2 TCI states use mutually exclusive RBs for transmission, there are no emissions products generated by the PA by virtue of having to process both UL signals (no beat products).

Observation 1: Each active PA is dedicated to one TCI state or another, no PA will be required to concurrently amplify UL signals belonging to different TCI states.

As is well understood, MPR and A-MPR are back off allowances that a UE can optionally avail to ensure transmit signal quality and emissions compliance. In RAN4, STxMP is currently scope-limited to mDCI [5], which means each UL enjoys independent link adaptation. For example, one link of the two could be scheduled for 64 QAM while the other is scheduled for QPSK based on propagation conditions. In such a case, applying the same back-off to both links is unlikely to reflect an optimal solution, given the very different EVM expectations for each.

Observation 2: For waveforms that are transmit signal quality limited, MPR must be TCI-state specific for mDCI STxMP and must be equal (or greater) than the single CC MPR.

Emissions limits outside the channel and pertaining to regulation on the other hand are defined in a TRP sense, and the proposed TRP formulation requires that the UE TRP for STxMP is not increased compared to single CC case:

### 6.2x.4 Configured transmitted power for [STxMP] … while the corresponding measured total radiated power PTMAX,f,c is bounded by PTMAX,f,c ≤ TRPmax …

Observation 3: For waveforms that are emissions limited in single CC operation, STxMP operation would not create more emissions compared to single CC operation because STxMP shares the upper TRP limit with single CC operation (TRPmax of UE power class).

With the previously agreed mDCI assumption, the per TCI-state UL powers can be assumed to be independently controlled. The network may request different EIRPs for the STxMP TCI-state pair which in turn would result in different TRP splits at the UE. The MPR tables need to be evaluated for both extremities of TRP split:

- TRP split with big differential: An example of this type of TRP split across TCI-states is 22.9+6.9 dBm for a UE limited to 23 dBm TRP. If the network reduces the EIRP associated with one TCI-state sufficiently, the other TCI-state TRP can be increased to take advantage of the TRPmax allowance.  This ‘high power’ TCI state would need sufficient back off, but since its TRP can only approach that of the single CC TRP limit, the existing single CC MPRs suffice.

- Equal TRP split: The other extremity of power sharing for the example UE is an equal TRP split (20+20), when the network commands both EIRPs towards their respective maximums. Since the per UE TRP is still limited to TRPmax, there is no extra risk of emissions failure, but there would be a reduction in per TCI-state EIRP. In this case, each TCI_State would need an ‘alternative minimum MPR’ of 3dB. For waveform types that need more MPR due to signal quality considerations, both, the signal quality challenge, and the emissions problem are simultaneously resolved, and no further consideration is necessary.

A simplified general MPR strategy would therefore be to retain the single CC MPRs as the baseline and allow an alternative minimum MPR to accommodate equal TRP split across TCI states. The argument applies equally to A-MPR.

Observation 4: The per TCI-state MPR ‘MPRf,c,k’ formulated as MAX(MPRsingleCC, 3 dB) would allow the UE to comply with emissions as well as transmit signal quality requirements for STxMP while keeping per UE regulation compliant. A similar formulation is applicable to A-MPR.

Proposal 1: Define per TCI-state MPR ‘MPRf,c,k’ as MAX(MPRsingleCC, 3 dB). A-MPR to be treated similarly.

Additional UE implementation limitations pertaining to STxMP operation may be incorporated into the relaxation factor ‘∆TSTxMP’ as identified previously [2]. A detailed discussion on value of ∆TSTxMP can be postponed until there is a known need to specify a full suite of requirements. An example of an implementation mechanism accommodated here could be array splitting in FWA devices which can be reasonably assumed to have a single (large) array.

Proposal 2: RAN4 confirms that the per-TCI state configured power included in the beginning of this section is the baseline assumption for a future UE RF requirement set for STxMP.

New signaling like ‘power sharing’ intends to help the network refine its UL power expectations from each UE. Other mechanisms like per-TCI state PHR already can carry similar information. Moreover, this information can be baked into the MPR assumption at the outset, see observation 3. We therefore considered such discussion as part of  ‘further optimization’ rather than one to establish a more basic conceptual understanding.

Observation 5: Additional signaling like ‘power sharing’ represents optimization and is not central to the basic concept of per-TCI state transmitted power configurations.

# 3. 	Conclusions

Observation 1: Each active PA is dedicated to one TCI state or another, no PA will be required to concurrently amplify UL signals belonging to different TCI states.

Observation 2: For waveforms that are transmit signal quality limited, MPR must be TCI-state specific for mDCI STxMP and must be equal (or greater) than the single CC MPR.

Observation 3: For waveforms that are emissions limited in single CC operation, STxMP operation would not create more emissions compared to single CC operation because STxMP shares the upper TRP limit with single CC operation (TRPmax of UE power class).

Observation 4: The per TCI-state MPR ‘MPRf,c,k’ formulated as MAX(MPRsingleCC, 3 dB) would allow the UE to comply with emissions as well as transmit signal quality requirements for STxMP while keeping per UE regulation compliant. A similar formulation is applicable to A-MPR.

Proposal 1: Define per TCI-state MPR ‘MPRf,c,k’ as MAX(MPRsingleCC, 3 dB). A-MPR to be treated similarly.

Proposal 2: RAN4 confirms that the per-TCI state configured power below will be pursued as a baseline for a future UE RF requirement set for STxMP.

### 6.2x.4 Configured transmitted power for [STxMP] The UE can configure its maximum output power for each UL TCI-state indicated for [STxMP]. The configured UE maximum output power PCMAX,f,c,k for TCI state k of carrier f and serving cell c defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement for TCI state k as specified in TS 38.215 [11]. The configured UE maximum output power PCMAX,f,c,k shall be set such that the corresponding measured peak EIRP PUMAX,f,c,k for each of the active TCI states k indicated for [STxMP] is within the following bounds PPowerclass + DPIBE – MAX(MAX(MPRf,c,k, A- MPRf,c,k) + ΔMBP,n, P-MPRf,c,k) – MAX{T(MAX(MPRf,c,k, A- MPRf,c,k)), T(P-MPRf,c,k)} -[∆TSTxMP] ≤ PUMAX,f,c,k ≤ EIRPmax and where the corresponding measured peak EIRP for carrier f of a serving cell c, over all active TCI states indicated for [STxMP], PUMAX,f,c satisfies PUMAX,f,c ≤ EIRPmax while the corresponding measured total radiated power PTMAX,f,c is bounded by PTMAX,f,c ≤ TRPmax [where ∆TSTxMP is a relaxation specific to STxMP operation]

Observation 5: Additional signaling like ‘power sharing’ represents optimization and is not central to the basic concept of per-TCI state transmitted power configurations.

# 4. 	References

- R4-2211516, ‘LS on UE power limitation for STxMP in FR2’, RAN1, RAN4#104-e, Aug. 2022

- R4-2304600, ‘On configured Tx power for STxMP in FR2’, Qualcomm, RAN4#106-Bis-e, Apr. 2023

- R4-2303494, ‘Reply LS on UE power limitation for STxMP in FR2’, vivo, RAN4#106, Athens, GR., Feb-Mar. 2023

- R4-2306657, ‘Reply LS on configured Tx power for STxMP in FR2’, Qualcomm, RAN4#106-Bis-e, Apr. 2023

- R4-2303495, ‘WF on NR MIMO evolution for downlink and uplink’, Samsung, RAN4#106, Athens, GR., Feb-Mar. 2023

- R4-2220533, ‘WF on FR2 multi-Rx’, Apple, RAN4#105, Toulouse, FR., Nov. 2023