3GPP TSG-RAN WG4 Meeting # 107	R4-2309298

# Incheon, KR, May 22 – May 26, 2023

Source: 	Skyworks Solutions, Inc.

Title: 	Input on CA_n5-n71 cross-band MSD and DeltaT/R

Agenda Item:	7.1.1.2	Others	[WI code]

Document for:	Approval

# Introduction

In RAN4#106b-e, the support of 1UL configurations for CA_n5-n71 was captured in [1] but was missing cross-band MSD issues. In this contribution, we provide our inputs regarding architecture options and Delta T, Delta R and 1UL cross-band MSD evaluation.

# Discussion

## Architecture

It should first be acknowledged that Band n71 can be supported with an n105 filter implementation, which is the most challenging low band duplexer in terms of bandwidth, duplex gap and the need to protect the DL from DTV transmitters; and thus, any additional multiplexing requirement cannot be fulfilled with improved performance without compromising the band performance itself. Furthermore, n105 filter can be used to support Band n71, and there are multiple LBLB combinations that will need to be supported within the same UE that include bands n5, n71 or n105, and including cases where the same operator requests the support of multiple LBLB combinations:

- CA_n5-n105

- CA_n5-n71

- CA_n28-n105

- CA_n71-n85

- CA_n71-n29

- CA_n5-n28

- Note that LBLBLB cases are also requested.

It is therefore impossible to optimize the band-to-band Tx/Rx isolations and Tx rejections for all bands and to enable multiple LBLB support in a UE, conservative numbers should be used.

Still, given the distance between the two bands it should be feasible with the proper Delta T and Delta R, to target a two-antenna architecture with two triplexers supporting n71 with an n105 implementation and should be used for the study:

- n105 DL+ n105 UL + n5 DL triplexer

- n105 DL+ n5 UL + n5 DL triplexer.

Proposal on architecture: CA_n5-n71 requirement is based on a two-antenna implementation with:

- Band n71 and n105 co-banding

- n5DL+n105UL+n105DL triplexer on antenna 1

- n105DL+n5UL+n5DL triplexer on antenna 2

- Support of multiple LBLB combinations.

## Delta T and Delta R

Although Delta T and Delta R were already proposed in [1], we believe the issue of multiple LBLB support including 2UL was not considered. We thus examined this aspect further.

For CA_n5-n71 the 2 antenna implementation should drive the TIB,c and RIB,c values. Based on other LB-LB cases and especially DC_5_n71, Delta T of 0.5dB for both bands can be used. For Delta Rib, CA_n5-n12 values can be reused.

Proposal on Delta T and Delta R: the values in Table 1 and 2 are used for CA_n5-n71.

Table 1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n5-n71 | 0.5 | 0.5 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n5-n71 | 0.5 | 0.3 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

## 1UL cross-band MSD test points

For CA_n5-n71, the 1UL cross band interference is as follows for the IMD orders of the allocated UL RBs and its image:

- For the n71 20MHz maximum UL CBW, the IMD interference order to n5 is >9

- For the n5 20MHz maximum UL CBW, the IMD interference order to n105 is >9

Given this, there is no MSD due to IMDs but still the transmitter noise floor must still be considered.

With the architecture aspects discussed above and the need to support multiple LBLB cases, conservative values should be considered:

- 45dB cross band Tx-Rx isolation for the triplexers

- 40dB Tx rejection in the other DL band

- 10dB antenna coupling

- 4dB post PA losses

- Transmitter noise floor of -125dBm/Hz.

Based on these assumptions, the MSD with MRC combining is calculated in Table 3.

Table 3: MRC calculations for band n5 and n71 cross-band MSD

|  | band | n71 |  | n5 |  |
| --- | --- | --- | --- | --- | --- |
| CBW/NRB | [MHz]/# | 5 | 25 | 5 | 25 |
| transmitter noise floor | [dBm/CBW] | -58.5 |  | -58.5 |  |
| 5MHz CBW REFSENS | [dBm/CBW] | -97.2 |  | -98.0 |  |
| RX noise floor wo Tx noise: Main/Div | [dBm/CBW] | -93.2 | -93.2 | -94.0 | -94.0 |
| Tx-Rx / Tx-Ant duplexer isolation/rejection | [dB] | 45 | 40 | 45 | 40 |
| LNA to antenna insertion loss | [dB] | 4 |  | 4 |  |
| Antenna isolation | [dB] | 0 | 10 | 0 | 10 |
| PC3 1Tx interference levels: Main/Div | [dBm/CBW] | -99.5 | -104.5 | -99.5 | -104.5 |
| noise degradation due to Tx noise: Main/Div | [dB] | 0.9 | 0.3 | 1.1 | 0.4 |
| # noise degradation due to Tx noise: Main/Div | # [lin] | # 1.2 | # 1.1 | # 1.3 | # 1.1 |
| REFSENS degradation after MRC | uncor [dB] | 0.6 |  | 0.7 |  |
| REFSENS degradation after MRC | cor [dB] | 1.1 |  | 1.3 |  |

Proposal for CA_n5-n71 1UL cross-band MSD: see Table 4 below.

Table 4: CA_n5-n71 1UL cross-band MSD

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n5 | 688 | 20 | 15 | 20 (RBstart=86) | 871.5 | 5 | 1.3 | >ACLR2 |
| n5 | n71 | 834 | 20 | 15 | 20 (RBstart=0) | 649.5 | 5 | 1.1 | >ACLR2 |

# Conclusions

In this contribution, we have discussed the requirements for CA_n5-n105 study and make the following proposals.

Proposal on architecture: CA_n5-n71 requirement is based on a two-antenna implementation with:

- Band n71 and n105 co-banding

- n5DL+n105UL+n105DL triplexer on antenna 1

- n105DL+n5UL+n5DL triplexer on antenna 2

- Support of multiple LBLB combinations.

Proposal on Delta T and Delta R: the values in Table 1 and 2 are used for CA_n5-n71.

Table 1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n5-n71 | 0.5 | 0.5 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n5-n71 | 0.5 | 0.3 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

Proposal for CA_n5-n71 1UL cross-band MSD: see Table 4 below.

Table 4: CA_n5-n71 1UL cross-band MSD

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n5 | 688 | 20 | 15 | 20 (RBstart=86) | 871.5 | 5 | 1.3 | >ACLR2 |
| n5 | n71 | 834 | 20 | 15 | 20 (RBstart=0) | 649.5 | 5 | 1.1 | >ACLR2 |

# References

- R4-2306486 TP for TR 38.718-02-01 to include CA_n5A-n71A, Ericsson, Rogers, RAN4#106b-e