3GPP TSG-RAN WG4 Meeting #107                                                               R4-2309325

Incheon, Korea Republic, May 22nd – 26th, 2023

Source:	Samsung

Title:	Discussion on tunnel deployment scenario for FR2 HST

Agenda item:			8.13.4.4

Document for:	Discussion

# 1. Introduction

In RAN#95e meeting, the Rel-18 RAN4-led work item on enhanced NR support for high speed train scenario in FR2 has been approved as [1]. This Rel-18 can be regarded as the continuous enhancement over the Rel-17 feature of NR support of FR2 HST, in which Rel-17 WI RAN4 has focused on train roof-mounted high-power devices for NR SA single carrier scenario in FR2, by studying the FR2 HST deployment scenario and specifying the channel modelling, RF, RRM and demodulation requirements for FR2 HST.

As the objectives of Rel-18 enhanced NR support for FR2 HST, to study on reference tunnel deployment scenario for FR2 HST is expected to be included in this WI, which is provided in WID [2] as follows:

- Study on reference tunnel deployment scenario for FR2 HST and specify the channel model and corresponding core requirements if any [RAN4]

Over past three RAN4 meetings (RAN4#105, #106 and #106-Bis-e), the WFs on tunnel deployment have been approved as [5-7], with the following WF and agreements obtained:

WF obtained in R4-2220396 ## General assumption for tunnel deployment Agreement: - For tunnel deployment scenario - Scenario #1: single-panel reception UE and DPS transmission scheme - FFS whether to consider additional scenarios ## Key parameters for tunnel deployment Agreement: Consider the key parameters below as baseline assumption for tunnel deployment feasibility study: - Ds: the distance separation between two neighboring RRH sites: - Ds = 700m - Dmin: the minimum distance between RRH site and train track: - Dmin = 1m - DRRH_height: determined/limited by tunnel height and RRH deployment method: - Priority scenario: DRRH_height = 5.3m, for single track tunnel (Option 1 for tunnel dimension in R4-2217254) - DRRH_height is in the range of [5.3m, 7.4m] for two-track tunnel (Option 2 for tunnel dimension in R4-2217254) - gNB RRH and antenna panel element assumption: - from 1 to 4 RRHs per BBU ## Reference channel model for tunnel scenario Way forward: Further analyse the channel model for tunnel scenario: - Option 1: Re-use channel model from Scenario-A as LoS propagation assumption is valid in the tunnel deployment with pathloss model, fading model and link budget the same as Scenario-A (LoS) - Option 2: Use LoS UMi street canyon channel mode for the RRM evaluations of HST FR2 tunnel deployment - Option 3: Use multi-path fading model (e.g., with up to 2nd order multi-path components) - Take int account measurement and ray-tracing analysis. - Option 4: Consider NLOS propagation condition when UE is around the RRH within 50m range for two directions. - Other options are not precluded ## Mobility issue for tunnel scenario Way forward: Further discuss possible solutions to the mobility issue when the train is travelling opposite to the serving beam orientation: - Option 1: L3 handover and L1 beam mobility configurations - Option 2: Solutions that allow network to trigger early handover - Option 3: Method in which UE initiates TCI state switch - Other options are not precluded FFS, the large and rapid RSRP degradation for tunnel scenario when multi-path fading and NLOS conditions next to RRH are considered. WF obtained in R4-2303174 Way forwards on FR2 HST tunnel deployment scenario are provided as: Topic-1: Deployment scenarios Agreement - Sceanrio#2: Two-panel simultaneous reception scenario and analysis of corresponding transmission schemes in tunnel deployment has lower priority Way forward: - FFS: feasibility of uni- and bi-directional RRH deployments in Sceanario#1 (single-panel reception UE and DPS transition schemes) - FFS: deployment assumption at the exit/entrance of the tunnel Topic-2: Channel model Agreement - Only consider LoS propagation conditions Way forward: - FFS: whether RRM session to decide propagation conditions which can be used for demodulation performance requirements: - Option 1: Reuse FR2 HST channel model in TS38.101-4 and TS38.104. - Option 2: Consider multi-path fading model (e.g., with up to 2nd order multi-path components) - Other options are not precluded Topic-3: Mobility issue Agreement - Mobility issue at HO/beam switch when CPE is travelling in the direction opposite to the serving beam is observed due to the sharp drop of the signal strength at the edge of the beam next to the RRH. Way forward: - Consider possible solutions to the mobility issue: - Option 2: Solutions that allow network to trigger early handover - Option 2a: Enabling CHO with special settings next to the RRH - Option 3: Method in which UE initiates TCI state switch as advanced capability - Option 4: Define the beam allocation regions - Option 5: No need to introduce new mechanism for mobility issue when the train is travelling opposite to the serving beam orientation - Other options are not precluded - Clarify the standardization impacts of proposed solutions WF obtained in R4-2306398 Topic #2: Tunnel deployment Issue 2-1-1: Deployment assumptions for Scenario#1 Agreement: - Consider both uni-directional and bi-directional deployments in the tunnel for Sceanrio#1 (single-panel reception UE and DPS transition scheme). Way forward: - FFS additional assumptions on tunnel deployment: - Option 1: In uni-directional scenarios assume the at least one open-space RRH is deployed close to railway, e.g., with tunnel deployment parameters, and orientations of RRH panels are the same in open-space and in the tunnel. - Option 2: No need to taking exit/entrance of the tunnel into account of tunnel scenario and channel. Issue 2-1-2: Channel model inside the tunnel Agreement: - Continue the discussion of the channel models in the Demod track - Dmin and Ds agreed in the RRM session can be used for reference [R4-2220396] Way forward: - FFS whether channel characteristics in tunnel scenario can be assumed to comparable to open space scenario. Issue 2-1-3: Solution to the mobility issue in the tunnel Way forward: - FFS possible solution to the mobility issue inside the tunnel when CPE is travelling in the direction opposite to the serving beam: - Option 2: Solutions that allow network to trigger early handover/beam switch - Option 2a: Enabling CHO with special settings next to the RRH - Option 3: UE-initiated beam selection/activation based on beam measurement - Option 5: No need to introduce new mechanism for mobility issue when the train is travelling opposite to the serving beam orientation - Option 6: Consider bi-directional deployment with simultaneous multi-panel reception in the tunnel scenario to alleviate the mobility issues.

In this contribution, we would like to share our further viewpoints on tunnel deployment related issues.

# 2. Study on Channel Modeling for Tunnel Deployment

## 2.1. Deployment assumptions for Scenario#1

In last RAN4 meeting, it is agreed that RAN4 to consider both uni-directional and bi-directional deployments in the tunnel for Sceanrio#1 (single-panel reception UE and DPS transition scheme). While it is still FFS additional assumptions on tunnel deployment as follows [7]:

- FFS additional assumptions on tunnel deployment:

  - Option 1: In uni-directional scenarios assume the at least one open-space RRH is deployed close to railway, e.g., with tunnel deployment parameters, and orientations of RRH panels are the same in open-space and in the tunnel.

  - Option 2: No need to taking exit/entrance of the tunnel into account of tunnel scenario and channel.

Based on our understanding, the identified issue comes from the particular configuration/assumption used in system level simulation, which is related to deployment which shall not be considered in a general 3GPP channel modeling discussion

Proposal 1: For additional assumptions on tunnel deployment, no need to taking exit/entrance of the tunnel into account of tunnel scenario and channel.

## 2.2 Channel modelling inside the tunnel

<The following content in Section 2.2 has been contained in R4-2305211, which has not yet been fully discussed in last meeting. >

In this contribution, by employing the reconstructed three-dimensional (3D) environment models with calibrated electromagnetic (EM) parameters, we have conducted extensive simulations for HSR scenarios based upon ray-tracing (RT) channel modeling, thus obtaining the fundamental knowledge of HST channel propagation properties for channel deployment scenario. The fundamental channel characteristics including pathloss, time and angular spread, Doppler and dominant multipath components are thoroughly studied for tunnel scenario for DRRH_height = 5.3m, for single track tunnel.

The ray-tracing simulation configurations are summarized in Table 1. In the RT simulation, the direct, reflection and scattering propagation mechanisms are considered. Concrete is the material of the buildings and tunnel wall, and metal is the material of the train body and the railway infrastructure. Besides, the omni-directional antenna element is configured in the simulation, which is used to capture signals from all directions to reach the characterization in spatial domain. A well-calibrated RT model is used in this work to conduct ray-tracing simulation, with the electromagnetic parameters of the materials are calibrated and validated.

Table 1. Ray-Tracing simulation configuration

| Freq./BW | 30 GHz, 100MHz |  |
| --- | --- | --- |
| Scenarios | Tunnel |  |
| Antenna | Tx: 1 antenna element Rx: 1 antenna element Omni-directional antenna element |  |
| Propagation mechanism | Direct | Friis equation |
|  | Reflection | Fresnel eq. (to 7th order) |
|  | Scattering | Directive scattering |
| Material | Building/Tunnel | Concrete |
|  | Ground | Concrete |
|  | Train/Rail | Metal |

#### 2.2.1 Evaluation on Pathloss for Tunnel Deployment

Fig. 1 compares the pathloss trends between these two scenarios. It is demonstrated that the pathloss characteristics in the urban open space and tunnel scenarios are almost the same in most locations. However, the pathloss trends of two scenarios diverge as the transceivers get closer, that is, pathloss in the urban scenario becomes much larger than the counterpart in tunnel scenario. Particularly, when Rx moves to the location beneath the Tx point, the pathloss difference can reach over 20 dB. This can be explained by the narrow shape of tunnel resulting in positive accumulation of multipath components (MPCs).

Fig. 1. Pathloss in HST scenarios: Urban open space scenario (left); Tunnel scenario (right).

Observation 1: Similar pathloss trends are observed for FR2 HST tunnel and urban open space scenarios, because of FR2 panel rather than leaky cable used for FR2 deployment.

#### 2.2.2 Evaluation on Angular spread for Tunnel Deployment

The RMS angular spreads in tunnel and urban open space deployment scenario are shown in Fig. 2. ASA, ASD, ESA, ESD are the terms used to represent the angular spreads of the azimuth angle of arrival (to UE), the azimuth angle of departure (from gNB RRH), the elevation angle of arrival (to UE) and elevation angle of departure (from gNB RRH), respectively.

- ASA

- ESA

- ASD

- ESD

Fig. 2. CDF of Angular spread in HST scenarios of urban open space and tunnel scenarios: (a) ASA; (b) ESA; (c) ASD; (d) ESD.

For FR2 HST deployment in tunnel scenario, because the heights of the transmitter and receiver are comparable (i.e., DRRH_height = 5.3m and DUE_height= 5m), and thus the objects are more diverse in the horizontal plane than elevation plane, and the azimuth angular spreads are significantly larger than the elevation angular spreads. Accordingly, the below observation can be obtained.

Observation 2: By assuming omni-directional antenna for ray-tracing based channel fading evaluation, tunnel deployment demonstrates the availability of propagation paths restricted in a very limited range of elevation angle (averaged as ~ 10 degrees), but much wider azimuth angle range (averaged as ~ 78 degrees), from UE perspective.

By comparing the HST channel fading results of urban open space and tunnel scenarios, the angular spread for tunnel scenario is comparable and even more concentrated than urban open space scenario, if omni-directional antenna is considered.

Observation 3: The angular spread for tunnel scenario is comparable and even more concentrated than urban open space scenario.

Based on the above Ray-tracing based evaluation results, we would like to confirm the following proposal:

Proposal 2: Channel characteristics in tunnel scenario can be assumed to comparable to open space scenario.

# 4. Conclusion

In this contribution, we provided our viewpoints on the RF requirements for reference tunnel deployment for FR2 HST enhancement for this work item, accordingly the following observations and proposals are obtained:

Proposal 1: For additional assumptions on tunnel deployment, no need to taking exit/entrance of the tunnel into account of tunnel scenario and channel.

Observation 1: Similar pathloss trends are observed for FR2 HST tunnel and urban open space scenarios, because of FR2 panel rather than leaky cable used for FR2 deployment.

Observation 2: By assuming omni-directional antenna for ray-tracing based channel fading evaluation, tunnel deployment demonstrates the availability of propagation paths restricted in a very limited range of elevation angle (averaged as ~ 10 degrees), but much wider azimuth angle range (averaged as ~ 78 degrees), from UE perspective.

Observation 3: The angular spread for tunnel scenario is comparable and even more concentrated than urban open space scenario.

Proposal 2: Channel characteristics in tunnel scenario can be assumed to comparable to open space scenario.

# 5. Reference

[1] RP-220985, “New WID on enhanced NR support for high speed train scenario in frequency range 2 (FR2)”, Samsung.

[2]	RP-222272, “Revised WID on enhanced NR support for high speed train scenario in frequency range 2 (FR2)”, Samsung.

[3]	TR 38.854 v0.4.0, NR support for high speed train scenario in frequency range 2 (FR2).

[4] R4-2217254, “WF on tunnel deployment and UL timing adjustment for FR2 HST enhancement”, Samsung.

[5] R4-2220396, “WF on NR FR2 HST Tunnel deployment and UL timing adjustment”, Nokia.

[6] R4-2303174, “WF on NR FR2 HST Tunnel deployment”, Nokia.

[7] R4-2306398, “WF on NR FR2 HST UL Timing Adjustment Solutions and Tunnel Deployment”, Nokia.