3GPP TSG-RAN WG4 Meeting #107 	R4-2309329

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On NCR Demodulation Performance Requirements

Agenda item:	8.29.6

Document for:	Discussion

Introduction

In RAN4#106bis-e, the first meeting on the demodulation performance of the C-link of Network Controlled Repeater (NCR) was held. There were already some agreements made with several remaining open issues with few options including some FFSs. In this paper, we will first summarize the agreements made in RAN4#106bis-e meeting as documented in the WF [1] and, afterwards, we provide our views on the remaining open issues.

The agreements in [1] covered both RAN4 Demodulation Requirements and CSI Requirements topics. For Demodulation Requirements topic, there were 5 subtopics of discussions: General, PDSCH Requirements, PDCCH Requirements, PBCH Requirements and SDR Requirements. Concerning PBCH and SDR subtopics, it was already agreed in RAN#4106bis-e to not define any requirements for them.

For CSI Requirements topic, three subtopics were discussed in RAN4#106bis-e meeting: CQI Requirements, PMI Requirements and RI Requirements. It was already agreed in [1] that RAN4 will not define requirements for RI reporting. Furthermore, for CQI Requirements, all open issues treated in RAN4#106bis-e discussion have been agreed. Hence, the remaining open issue is concerning the PMI reporting.

Among all the remaining open issues, which will be discussed in this paper, three of them can already be highlighted here, namely:

- Issue 1-1-3: Whether define new requirements for FR2 Agreement: - Reuse IAB-MT requirements for NCR-MT demodulation and only consider Rank 1 with 100MHz/120kHz - FFS new requirements on PDCCH/PDSCH for signaling of Access link beam change indication - Issue 1-2-1: How to define PDSCH requirements Agreements: - Both 70% and 30% throughput cases agreed as baseline assumption for the test cases reused from IAB-MT requirements - Other values higher than 70% not precluded - FFS for the test metric of new test case for specifically for MAC-CE Access link beam indication transmitted over PDSCH if introduced - Issue 2-2-1:Whether define PMI requirements Agreements: FFS whether need to define PMI reporting requirements

Discussion

In this Chapter, we provide our views on the remaining open issues and FFSs topics. We will first discuss the open issues on Demodulation Requirements and, afterwards, on CSI Requirements.

Open Issues in Demodulation Requirements

General

There are two remaining open issues in General subtopic from RAN4#106bis-e. The first open issue is whether RAN4 considers the additional Carrier Bandwidth (CBW) of 5MHz for 15kHz SCS:

Issue 1-1-2: Whether define new requirements for FR1 FFS whether need to consider additional CHBW 5MHz/15kHz

Given the low expected throughput (TPUT) of the NCR control link, we expect the CBW allocation for such control links to be the smallest possible, which leads us to asking for the inclusion of 5MHz/15kHz requirements. Furthermore, it is expected that NCR will operate in all NR frequency bands (both in FR1 and FR2). Hence, it will need to operate in different sizes of Carrier Bandwidths (CBWs). As it is needed to operate in different CBW sizes, it would be good to have requirements for one CBW that can be extended/reused to any supported CBW.

C-link NCR-MT will have low TPUT and, hence, smallest CBW can be allocated for such a link.

NCR will operate in different CBW sizes. Requirements for minimum CBW could be extended/reused to any supported CBW.

Based on these observations we propose to define minimum performance requirements for 5MHz CBW:

RAN4 shall define new FR1 requirements for 5 MHz/15 KHz.

The second open issue in General subtopic is:

Issue 1-1-3: Whether define new requirements for FR2 FFS new requirements on PDCCH/PDSCH for signaling of Access link beam change indication

Compared to RF Repeater, NCR has the advantage of having the C-link NCR-MT to carry side control information that can be used to control and manage the beam to be used by NCR-Fwd. Due to the mobility of the UE, adaptation of the beam in Access link will be important, especially, in FR2.

The operation of NCR in FR2 will depend on the Access link beam used by the NCR-Fwd. It is unlikely that a FR2 Fwd link NCR will implement FR1 for the backhaul link only, as such we need requirements to cover the FR2 only case.

New FR2 requirements shall be defined on PDCCH/PDSCH for signaling of the indication of changing the Access link beam.

PDSCH Requirements

There is only one remaining open issue for PDSCH Requirements subtopic, namely:

Issue 1-2-1: How to define PDSCH requirements Both 70% and 30% throughput cases agreed as baseline assumption for the test cases reused from IAB-MT requirements - Other values higher than 70% not precluded FFS for the test metric of new test case for specifically for MAC-CE Access link beam indication transmitted over PDSCH if introduced

Due to delay of beam changing commands having “exponential” impact on the access link performance, requirements without retransmission (reTX) for control link shall be introduced. For no reTX consideration, the BLER KPI is more suitable than TPUT. Furthermore, control link needs to be more reliable than access link for not becoming the bottleneck (as analogous to PDCCH and PXSCH). Hence, the BLER requirements for control link needs to be tighter than for PDSCH (without reTX).

C-link of NCR-MT must be decoded correctly with as few HARQ retransmission as possible to ensure the best Access link beam is set properly for the NCR-Fwd within the appropriate time (i.e., not outdated and not introducing further latency).

RAN4 shall consider no less than 70% throughput in the requirements.

With the low data rate transmission on the C-link NCR-MT and with the requirements of having high reliability in decoding the Access link beam indication MAC CEs, Block Error Rate (BLER) can be considered as a metric for C-link NCR-MT

Additionally, RAN4 shall specify requirements having BLER <1% for the PDSCH C-link NCR-MT without HARQ retransmission.

PDCCH Requirements

Subtopic PDCCH Requirements still has two remaining open issues from RAN4#106bis-e. The first one is:

Issue 1-3-1: How to define PDCCH requirements FFS for adapt PDCCH requirement for testing of DCI type 5_0 used for access link beam change indication.

In the current requirements, the receiver characteristics of the PDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg). Corrodingly, for IAB-MT, DCI formats 1_0 and 1_1 are defined in the fixed reference channels (FRC) for PDCCH in Appendix A of [3].

However, for NCR-MTs, the scheduling grants may be used rarely, because the support of user-plane traffic is optional.

On the other hand, it is agreed in RAN1 [4][5] that the access link beam change indication will be carried by DCI type 5_0 (or DCI format 2_8). While the minimum requirements criteria of 1% of probability of misdetection can be used, necessary adaptation in PDCCH test parameters should be done for DCI type 5_0. Some of the corresponding L1 agreements are listed below:

RAN1#112 [4] Agreement The DCI Format 5_0 carrying the side control information is monitored by the NCR-MT at least in the UE specific search space. - Note: The existing configuration of UE specific search space will be reused. Agreement For periodic and semi-persistent beam indication, - The maximum number of periodic beam indication is 32. - The maximum supported number of beam is 64 (to determine the value range of beam index). - The maximum number of duration of time resource is 112 symbols. For periodic beam indication, The maximum number of forwarding resources in one periodic beam indication is 1024. RAN1#112-bis-e [5] Agreement (NR network-controlled repeaters) For the aperiodic beam indication via DCI 5-0, the following option is supported: - Option-4: - The DCI size of DCI Format 5-0 is implicitly determined by the RRC configuration with the maximum value as 128. - The [maximum] number of fields for time resource indication (Tmax) is explicitly configured by dedicated RRC parameter with the maximum value as [16] or [32]. - FFS: How to support and address an actual number of fields for time resource indication is smaller than the configured maximum number.

In some configurations, DCI 5_0 size can be pretty large, because it can carry access beam configuration for multiple time resources and beams.

For NCR-MT, scheduling grant (i.e., DCI format 1_0/1_1) is not typical, whereas DCI type 5_0/2_8 can be used for access link beam change indication. The DCI size can have the maximum value of 128.

RAN4 shall adapt test parameters for NCR PDCCH requirements following DCI format 5_0/2_8, e.g., FRC, DCI format, payload size, probability of misdetection.

The second open issue in PDCCH requirements is related to the coreset duration for FDD:

Issue 1-3-2: Coreset for FDD - Option 1: - 1TX: coreset duration 2 - 2TX: coreset duration 1 - Option 2: - 1TX: coreset duration 1 - 2TX: coreset duration 1

Most test cases in 38.101-4 [2] for PDCCH in FDD are having Coreset duration of 2 for 1TX. There is only one test case with Coreset duration of 1. Both Option 1 and Option 2 differs only on the Coreset duration for 1TX. This means that with either option, RAN4 will down select the test cases in 38.101-4.

In 38.101-4, for FDD, the majority test cases have Coreset duration of 2 for 1TX.

RAN4 to down select the test cases in 38.101-4 for NCR-MT by considering Coreset duration 2 for 1TX and Coreset duration 1 for 2TX.

Open Issues in CSI Requirements

As mentioned in the Introduction section, almost all open issues for CSI Requirements have been agreed. The remaining open issue for discussion is in subtopic PMI reporting.

Issue 2-2-1:Whether define PMI requirements. FFS whether need to define PMI reporting requirements

In RAN4#106bis-e discussions, we have provided our views on this issue. Our views are summarized in the following:

As NCR is part of the network device, in the network planning process, NCR will be deployed in a location with a relatively good channel condition to the gNB. Furthermore, as both NCR and gNB are in a fixed location and not mobile, the channel between them will be considerably stable and not change frequently.

In a usual direct communication between gNB and UE, PMI is intended to maximize the throughput based on the channel condition.  Nonetheless, it is expected that the C-link in NCR-MT will only carry low data rate for carrying only side control information.

The arguments to have PMI reporting is the change of the channel condition. Nonetheless, the changing of the channel condition can already be adapted by adapting the CQI, hence, the MCS. In RAN4#106bis-e it is already agreed to define CQI requirements.

NCRs are part of the network and can profit from network planning. Good channel conditions and known spatial transmission/receptions environments can be assumed. Overall low data rates are expected for NCR control information, hence fixed PMI or even no-PMI will be feasible in deployment. Link adaptation is covered in CQI requirements.

RAN4 shall not consider defining requirements for PMI reporting.

Conclusion

In this paper, the following Observations and Proposals for NCR-MT based on the remaining issues were made:

Observation 1: C-link NCR-MT will have low TPUT and, hence, smallest CBW can be allocated for such a link.

Observation 2: NCR will operate in different CBW sizes. Requirements for minimum CBW could be extended/reused to any supported CBW.

Proposal 1: RAN4 shall define new FR1 requirements for 5 MHz/15 KHz.

Observation 3: The operation of NCR in FR2 will depend on the Access link beam used by the NCR-Fwd. It is unlikely that a FR2 Fwd link NCR will implement FR1 for the backhaul link only, as such we need requirements to cover the FR2 only case.

Proposal 2: New FR2 requirements shall be defined on PDCCH/PDSCH for signaling of the indication of changing the Access link beam.

Observation 4: C-link of NCR-MT must be decoded correctly with as few HARQ retransmission as possible to ensure the best Access link beam is set properly for the NCR-Fwd within the appropriate time (i.e., not outdated and not introducing further latency).

Proposal 3: RAN4 shall consider no less than 70% throughput in the requirements.

Observation 5: With the low data rate transmission on the C-link NCR-MT and with the requirements of having high reliability in decoding the Access link beam indication MAC CEs, Block Error Rate (BLER) can be considered as a metric for C-link NCR-MT

Proposal 4: Additionally, RAN4 shall specify requirements having BLER <1% for the PDSCH C-link NCR-MT without HARQ retransmission.

Observation 6: For NCR-MT, scheduling grant (i.e., DCI format 1_0/1_1) is not typical, whereas DCI type 5_0/2_8 can be used for access link beam change indication. The DCI size can have the maximum value of 128.

Proposal 5: RAN4 shall adapt test parameters for NCR PDCCH requirements following DCI format 5_0/2_8, e.g., FRC, DCI format, payload size, probability of misdetection.

Observation 7: In 38.101-4, for FDD, the majority test cases have Coreset duration of 2 for 1TX.

Proposal 6: RAN4 to down select the test cases in 38.101-4 for NCR-MT by considering Coreset duration 2 for 1TX and Coreset duration 1 for 2TX.

Observation 8: NCRs are part of the network and can profit from network planning. Good channel conditions and known spatial transmission/receptions environments can be assumed. Overall low data rates are expected for NCR control information, hence fixed PMI or even no-PMI will be feasible in deployment. Link adaptation is covered in CQI requirements.

Proposal 7: RAN4 shall not consider defining requirements for PMI reporting.

- References

- R4-2305934, WF on NCR-MT demodulation requirements, ZTE Corporation, RAN4#106bis-e, Electronic Meeting, April 17 –26, 2023.

- 3GPP TS 38.101-4, User Equipment (UE) radio transmission and reception; Part 4: Performance requirements, V17.7.0, 12-2022

- 3GPP TS 38.174, Integrated access and backhaul radio transmission and reception, V17.2.0, 12-2022

- R1-2302256, Final Report of 3GPP TSG RAN WG1 #112 v1.0.0, MCC Support, Athens, Greece, 27th February – 3rd March 2023

- R1-2304301, Final Report of 3GPP TSG RAN WG1 #112bis-e v1.0.0, MCC Support, Online, April 17th - 26th, 2023