3GPP TSG-RAN WG4 Meeting #107 	R4-2309330

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	HST FR2 Enhanced UE Demodulation PDSCH Requirements with CA

Agenda item:	8.13.5.2

Document for:	Discussion

Introduction

In RAN4#106bis-e meeting, the first discussion on HST FR2 UE Demodulation was taking place. There were already some agreements made, which are documented in the WF. There were three topics discussed in the meeting, namely, HST FR2 with Carrier Aggregation (CA), HST FR2 with simultaneous multi-panel reception and HST FR2 with both simultaneous multi-panel reception and CA. It is already agreed in RAN4#106bis-e meeting that RAN4 will not define requirements for HST FR2 with both simultaneous multi-panel reception and CA.

In this paper, we will discuss HST FR2 with CA UE Demodulation Requirements. The early agreements are documented in [1]. Furthermore, simulation assumption for PDSCH requirement with CA has been proposed in [2] as the baseline assumption for companies to run simulation on CA.

The remaining open issues for HST FR2 with CA from RAN4#106bis-e are summarized below:

Issue 2-1-3: SCS & BW Agreement - Specify CA PDSCH requirements for FR2 HST with component carrier configuration as - {50, 100, 200, 400} Hz for 120KHz SCS - Companies are encouraged to check whether existing SNR requirement for 200MHz CBW defined in single carrier can be reused for CA case with 200MHz in the next meeting pending on the collected impairment results - FFS on the carrier combinations considered for requirement Issue 2-1-6: Others Way forward: - FFS on the necessity of considering additional margin for CA requirement based on single carrier simulation with considering imperfection model - FFS on how to consider the imperfection model if considered

Regarding the simulation for PDSCH requirements with CA, interested companies are encouraged to provide their simulation results in RAN4#107 meeting for the following test cases proposed in [2]:

Table 1 Simulation Test Cases for PDSCH Requirements with CA [2]

| Test number | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | FRC 1-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | TBD |
| 1-2 | FRC 1-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | TBD |
| 1-3 | FRC 1-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | TBD |
| 1-4 (For information) | R.PDSCH.5-12.2 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 13.9 |
| 2-1 | FRC 2-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | TBD |
| 2-2 | FRC 2-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | TBD |
| 2-3 | FRC 2-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | TBD |
| 2-4 (For information) | R.PDSCH.5-12.1 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 13.7 |

Discussion

In this Section we will address our views on the remaining open issues. There are two remaining open issues left from RAN4#106bis-e meeting. The first issue is about SCS and BW:

Issue 2-1-3: SCS & BW Agreement - Companies are encouraged to check whether existing SNR requirement for 200MHz CBW defined in single carrier can be reused for CA case with 200MHz in the next meeting pending on the collected impairment results - FFS on the carrier combinations considered for requirement

In RAN4#106bis-e meeting, the above-mentioned open issues were part of discussions in the first and second round offline discussions.

There are already existing minimum requirements for 200 MHZ CBW for HST FR2 as tabulated in 38.101-4 Table 7.2.2.2.4-3: Minimum performance for HST-DPS.

There is still an open issue from RAN4#106bis-e related to the possibility of adding noise margin and/or imperfection model to be discussed.

In case there is no additional noise margin or imperfection model being introduced to the single carrier link level simulation, the minimum requirements for 200 MHz CBW can be reused.

In [3] the minimum performance for multiple CA configurations is tabulated in Table 7.2A.2.1-3 as below:

Table 7.2A.2.1-3: Minimum performance for multiple CA configurations [3] Test number; CA duplex mode; Minimum performance requirements. 1; TDD 120 kHz + TDD 120 kHz; As defined in Table 7.2A.2.1-2. Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 7.1.1.5..

Table 7.1.1.5.2-1 in [3] provide further information on the applicability and test rules for CA UE demodulation test as in below:

Table 7.1.1.5.2-1: Applicability and test rules for CA UE demodulation tests [3] Tests; CA capability where the tests apply; CA configuration from the selected CA capability where the tests apply; CA Bandwidth combination to be tested in priority order; PCell CC configuration. Test 1 in Clause 7.2A.2.1; CA_C, CA_N, CA_AX; Table 7.1.1.5.2-2; Largest aggregated CA bandwidth combination; Any of CCs.

For UE demodulation, only TDD 120 KHz + TDD 120 KHz is considered in 38.101-4.

For testing in UE demodulation, largest aggregated CA bandwidth combination should be the priority.

For HST FR2 with CA, TDD 120 KHz + TDD 120 KHz shall be considered, with the largest aggregated CA bandwidth combination shall be the highest priority.

The second open issue is related to Others topic as follows:

Issue 2-1-6: Others Way forward: - FFS on the necessity of considering additional margin for CA requirement based on single carrier simulation with considering imperfection model - FFS on how to consider the imperfection model if considered

This issue has been discussed in round 1 and round 2 offline discussions in RAN4#106bis-e meeting [4]. In [6], we have provided our views on the issue.

There is a definition for poise power level being specified for CA in section 4.5.3.2 of [3], as follows:

For CA case, the Noc power level (NocCA) shall increase by a relaxation factor defined in TS 38.101-2 [5] Table 7.3A.2.1-1: NocCA = NocSC + ΔRIB - NocSC is derived by assuming UE supports single carrier. - ΔRIB values are specified in TS 38.101-2 [5].

The noise power level in CA case is coming from the noise level of single carrier with some additional noise from ΔRIB, which is the allowed reference sensitivity relaxation due to support for inter-band CA operation.

In defining PDSCH performance requirements for HST FR2, when using single carrier link level simulation, RAN4 to introduce additional noise to the single carrier noise in a similar fashion as defined in section 4.5.3.2 of 38.101-4.

If neither additional noise nor imperfection model is introduced in the single carrier link level simulation, RAN4 to define HST FR2 requirements for single carrier for the whole considered CBW in FR2. The requirements for HST FR2 with CA can then simply refer to the requirements for single carrier.

RAN4 to consider imperfection model to be introduced in single carrier link level simulation if there is a proper model proposed by any company.

Simulation

As proposed in the WF [1], interested companies are encouraged to provide simulation results based on the agreed baseline simulation assumption in [2]. The test cases 1-4 and 2-4 as in Table 1 [2] are used as the baseline and reference to all companies’ early simulation results. Nonetheless, the SNR values for test cases 1-4 and 2-4 in Table 1 were defined in 38.101-4 [3] for Rel-17 HST FR2 after several iterations of simulation alignment process between companies and further addition of impairments as documented in R4-2208074 [7]. Hence, the early simulation results for Rel 18 HST FR2 with CA from interested companies should better be compared to the original simulation alignment in R4-2208074 [7] rather than the values in Table 1 (i.e., [3]).

The reference values for test cases 1-4 and 2-4 as documented in R4-2305893 are taken from 38.101-4 for single carrier Rel-17 HST FR2.

The final values as specified in 38.101-4 for single carrier Rel-17 HST FR2 were defined after some iterations of simulation alignment process and the introduction of impairments.

In the simulation alignment for Rel-18 HST FR2 with CA, RAN4 shall consider the reference values as in R4-2208074 rather than the already specified values in 38.101-4.

Simulation Results

The simulation results are tabulated in Table 2. In our simulation results for the test cases 1-4 and 2-4, there are no impairments introduced in the single carrier link level simulation. They are reasonably aligned with the R4-2208074 [7], which were used for defining Rel-17 HST FR2 requirements before the introduction of impairments.

We note here that we will deliver our simulation results for the remaining cases on the first day of RAN4#107 meeting, by merging them into the summary of simulation results spreadsheet.

Table 2. Simulation Results for Different Test Cases for HST FR2 with CA (without impairments)

| Test number | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | FRC 1-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | n.A |
| 1-2 | FRC 1-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | n.A |
| 1-3 | FRC 1-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | n.A |
| 1-4 (For information) | R.PDSCH.5-12.2 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 11.4 |
| 2-1 | FRC 2-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | n.A |
| 2-2 | FRC 2-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | n.A |
| 2-3 | FRC 2-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | n.A |
| 2-4 (For information) | R.PDSCH.5-12.1 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 11.4 |

Conclusion

In this paper, we provide our responses to the remaining open issues on HST FR2 with CA. Furthermore, we also provide our simulation results based on the agreed simulation assumption.

The following Observations and Proposals were made:

Observation 1: There are already existing minimum requirements for 200 MHZ CBW for HST FR2 as tabulated in 38.101-4 Table 7.2.2.2.4-3: Minimum performance for HST-DPS.

Observation 2: There is still an open issue from RAN4#106bis-e related to the possibility of adding noise margin and/or imperfection model to be discussed.

Proposal 1: In case there is no additional noise margin or imperfection model being introduced to the single carrier link level simulation, the minimum requirements for 200 MHz CBW can be reused.

Observation 3: For UE demodulation, only TDD 120 KHz + TDD 120 KHz is considered in 38.101-4.

Observation 4: For testing in UE demodulation, largest aggregated CA bandwidth combination should be the priority.

Proposal 2: For HST FR2 with CA, TDD 120 KHz + TDD 120 KHz shall be considered, with the largest aggregated CA bandwidth combination shall be the highest priority.

Observation 5: The noise power level in CA case is coming from the noise level of single carrier with some additional noise from ΔRIB, which is the allowed reference sensitivity relaxation due to support for inter-band CA operation.

Proposal 3: In defining PDSCH performance requirements for HST FR2, when using single carrier link level simulation, RAN4 to introduce additional noise to the single carrier noise in a similar fashion as defined in section 4.5.3.2 of 38.101-4.

Proposal 4: If neither additional noise nor imperfection model is introduced in the single carrier link level simulation, RAN4 to define HST FR2 requirements for single carrier for the whole considered CBW in FR2. The requirements for HST FR2 with CA can then simply refer to the requirements for single carrier.

Proposal 5: RAN4 to consider imperfection model to be introduced in single carrier link level simulation if there is a proper model proposed by any company.

Observation 6: The reference values for test cases 1-4 and 2-4 as documented in R4-2305893 are taken from 38.101-4 for single carrier Rel-17 HST FR2.

Observation 7: The final values as specified in 38.101-4 for single carrier Rel-17 HST FR2 were defined after some iterations of simulation alignment process and the introduction of impairments.

Proposal 6: In the simulation alignment for Rel-18 HST FR2 with CA, RAN4 shall consider the reference values as in R4-2208074 rather than the already specified values in 38.101-4.

- References

- R4-2305892, WF on demodulation requirements for Rel-18 FR2 HST, Samsung, RAN4 #106bis-e, Electronic Meeting, April 17-26, 2023.

- R4-2305893, Simulation assumption for PDSCH requirement with CA, Ericsson, RAN4 #106bis-e, Electronic Meeting, April 17-26, 2023.

- 38.101-4 - User Equipment (UE) radio transmission and reception; Part 4: Performance requirements v17.6.0

- R4-2305990, Topic summary for [106-bis-e][321] NR_HST_FR2_enh_Demod, Samsung, RAN4 #106bis-e, Electronic Meeting, April 17-26, 2023.

- 3GPP TS 38.101-2, User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone, V18.0.0, 12-2022

- R4-2304458, HST FR2 Enhanced UE Demodulation Requirements, Nokia, Nokia Shanghai-Bell, RAN4 #106bis-e, Electronic Meeting, April 17-26, 2023.

- R4-2208074, Simulation results summary for Rel-17 FR2 HST UE demod, Samsung, RAN4#103-e, Electronic meeting, May 9th – 20th 2022.