3GPP TSG-RAN WG4 Meeting # 107	R4-2309351

# Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.19.1.1

Source: 	Qualcomm Incorporated

Title: 	MU-MIMO enhancement simulation assumptions and network assistant signaling

Document for:	Approval

# Introduction

In this contribution, we provide our view on the R18 demod enhancement on MU-MIMO.

# Discussion

## Implementation feasibility of R-ML receiver for MU-MIMO and network assistant signaling

The necessary information to enable R-ML receiver is discussed in the previous meeting. Considering the overhead, we believe the MCS index table via RRC signaling is a beneficial signaling which can reduce the search space for modulation order detection with a very low overhead.

Proposal 1: Introduce the RRC based signaling to inform UE the MCS index table(s) used for PDSCH of the co-scheduled UEs, or signal the maximum MCS index table (64QAM, 256QAM or 1024QAM table) used among all the (potential) co-scheduled UEs.

For DMRS sequence alignment, note that when all the co-scheduled UEs are with different DMRS sequence than the target UE, the target UE can’t run MU-MIMO advanced receiver algorithm on any of the co-scheduled UEs. Therefore, this scenario is almost the same as no co-scheduled UEs, and we propose the following:

Proposal 2: If “no co-schedule UEs are presented in the allocated resource to the target UE” is signaled in DCI, combine “all the co-scheduled UEs are with different DMRS sequence than the target UE” in the same signaling without additional bits, i.e., have a code point representing “no co-schedule UEs are presented in the allocated resource to the target UE, or all the co-scheduled UEs are with different DMRS sequence than the target UE”

For co-scheduled UEs in different CDM groups, detecting their existence and modulation orders becomes prohibitively expensive when the PRG allocation misalignment across different CDM groups. We have the following observation:

Observation 1: When PRG allocation is aligned across different CDM groups, the complexity of co-scheduled UE detection and the modulation order detection is reduced by at least x times, x is the PRG size.

Therefore, to enable R-ML receiver in the real deployment, we have the following proposal:

Proposal 3: Introduce the signaling to inform UE that whether the PRG sizes and boundaries of the DMRS aligned co-scheduled UEs in different CDM groups with the target UE are aligned with the target UE. The signaling format depends on how static this information is in the real deployment. DMRS aligned co-scheduled UE refers to the co-scheduled UEs with the same DMRS sequence as the target UE.

We also consider DMRS power boosting for the co-scheduled UE is a necessary information, and therefore we have the following proposal:

Proposal 4: Introduce the signaling to inform UE numbers of CDM groups without data of the DMRS aligned co-scheduled UEs, or at least inform UE whether numbers of CDM groups without data are aligned across all DMRS aligned co-scheduled UEs. The signaling can be RRC based.

PDSCH allocation alignment across co-scheduled UEs has a significant impact on the R-ML receiver:

Observation 2: The co-scheduled UE detection can’t be performed in a per-symbol basis due to the availability of DMRS and complexity constraints, and therefore, the interference in R-ML receiver applied uniformly to all the PDSCH symbols. When the interference presence is not uniform in a slot, R-ML receiver tries to cancel a non-existent interference, and end up introduce additional noise to fail the decoding of the symbol.

Therefore, we have the following proposal:

Proposal 5: Introduce the following network assistant signaling to enable the correct interference cancellation when implementing R-ML receiver:

- Whether all the serving PDSCH symbols are interfered by the same set of the DMRS aligned co-scheduled UEs. RRC signaling is applicable if the condition is satisfied in most cases.

Regarding the modulation order, in the previous meeting, many network vendors and operators mentioned the flexibility of scheduling more than one co-scheduled UEs (besides target UE) or having different modulation orders on multiple co-scheduled UEs is inevitable and may happen frequently. In our opinion, aligned modulation order on the (interfering) co-scheduled UE is beneficial to UE, but if this is not a common case that can be ensure by network vendors and operators, we need to discuss how beneficial is the signaling.

For the options captured in the WF:

WF Proposal 1: it doesn’t cover the case in which there are multiple FDMed co-scheduled UE, but with different modulation orders (e.g., one with 16QAM and one with 64QAM).

| Signalling Overhead |  | DCI signalling Info. | Antenna Port = 1000 + DMRS Port (P) |
| --- | --- | --- | --- |
| 2 bits | maxMIMO-Layers = 2 | 00: No interference presence 01: Interference with QPSK 10: Interference with 16QAM 11: Interference with 64QAM or 256 QAM | P = {0, 1} |
| 6 bits | maxMIMO-Layers = 4 | For 3 interfering ports in cyclic order from desired layer, each 2 bits represent as 00: No interference presence 01: Interference with QPSK 10: Interference with 16QAM 11: Interference with 64QAM or 256 QAM | P = {0, 1, 2, 3} |

WF Proposal 2: If network can algin the modulation order to the co-scheduling allocation, it should do it for all the co-scheduled UEs, and therefore the aligned signaling should be “all the co-scheduled UEs have the same modulation order as the target UE”.

| Bitfield | coUE modulation order | UE behavior |
| --- | --- | --- |
| 000 | No UE which has same DMRS sequence with target paired | SU-MIMO (MMSE-IRC) |
| 001 | PRB aligned (Single coUE modulation per PRB) | UE fallback to MMSE-IRC or R-ML with blind detection with only 4 hypotheses (UE dependent) |
| 010 | Bandwidth aligned, QPSK | R-ML without modulation order detection |
| 011 | Bandwidth aligned, 16-QAM | R-ML without modulation order detection |
| 100 | Bandwidth aligned, 64-QAM | R-ML without modulation order detection |
| 101 | Bandwidth aligned, 256-QAM | R-ML without modulation order detection |
| 110 | Bandwidth aligned, 1024-QAM | R-ML without modulation order detection |
| 111 | PRB not aligned (Multiple coUE modulations per PRB) | UE fallback to MMSE-IRC or R-ML with blind detection (UE dependent) |

We have the following proposal:

Proposal 6: Consider the following modulation order assistant signaling:

| Bitfield | coUE modulation order |
| --- | --- |
| 00 | No co-schedule UEs are presented in the allocated resource to the target UE, or all the co-scheduled UEs are with different DMRS sequence than the target UE |
| 01 | PRB aligned (Single coUE modulation per PRB or PRG) |
| 10 | All the co-scheduled UE has the same modulation order with the target UE |
| 11 | Other cases |

Alternatively, if we want to reduce the DCI bits, we can keep only the most critical information:

0: no co-schedule UEs are presented in the allocated resource to the target UE, or all the co-scheduled UEs are with different DMRS sequence than the target UE

1: other cases.

## Analysis for comparison between E-LMMSE-IRC receiver and LMMSE-IRC receiver

Consider the MU-MIMO system model:

rk, l= H1k,ld1k,l+j=2NUEHjk,ldjk,l+n(k,l)

The LMMSE-IRC receiver (baseline) detect signal d1k,l by first estimating the noise including interfering signal and the Gaussian noise:

rIRCk,l=rk, l-H1k,ld1k,l=j=2NUEHjk,ldjk,l+n(k,l)

rIRCk,lrIRCk,lH=rk, l-H1k,ld1k,lrk, l-H1k,ld1k,lH=rk, l-j=1NUEHjk,ldjk,l+j=2NUEHjk,ldjk,lrk, l-j=1NUEHjk,ldjk,l+j=2NUEHjk,ldjk,lH=rk, l-j=1NUEHjk,ldjk,lrk, l-j=1NUEHjk,ldjk,lH+2rk, l-j=1NUEHjk,ldjk,lj=2NUEHjk,ldjk,lH+j=2NUEHjk,ldjk,lj=2NUEHjk,ldjk,lH

Assume the signal vectors across different UEs are independent, then when we average over signal vector realizations,

Ej=2NUEHjk,ldjk,lj=2NUEHjk,ldjk,lH=j=2NUEPjHjk,lHjHk,l

Where Pj=trace(Edjk,ldjk,lH), i.e., the average power of transmission to UE j.

Assume the signal vectors of all UEs are independent with noise, then when we average over signal vector and noise realizations

Erk, l-j=1NUEHjk,ldjk,lj=2NUEHjk,ldjk,lH=En(k,l)j=2NUEHjk,ldjk,lH=Ej=2NUEn(k,l)djk,lHHjk,lH=0

Therefore, by averaging over signal vector and noise realizations, we have (use ~ to denote the averaging)

rIRCk,lrIRCk,lH~rk, l-j=1NUEHjk,ldjk,lrk, l-j=1NUEHjk,ldjk,lH+j=2NUEPjHjk,lHjHk,l

The receiver weight matrix is

RIRC= P1H1k,lH1Hk,l+1Nrek,l∈DMRS of NUErIRCk,lrIRCk,lH~ P1H1k,lH1Hk,l+1Nrek,l∈DMRS of NUErk, l-j=1NUEHjk,ldjk,lrk, l-j=1NUEHjk,ldjk,lH=P1H1k,lH1Hk,l+j=2NUEPjHjk,lHjHk,l+1Nrek,l∈DMRS of NUEr(k,l)r(k,l)H=RE-MMSE

Where RE-MMSE is from the formulation agreed in the previous meeting.

Therefore, we conclude:

Observation 3: LMMSE-IRC and E-LMMSE-IRC are statistically equivalent and can be proved mathematically. Given that the channel estimation is done across multiple REs and the simulations are ran in a long enough duration to have converged throughput, we expect the E-LMMSE-IRC receiver to have exactly the same performance as LMMSE-IRC.

Proposal 7: Do not consider additional requirement for E-LMMSE-IRC receiver.

## Advanced MU-MIMO Receiver Simulations and Requirement Recommendations

Proposal 7: When defining the requirement, the precoding matrices across co-scheduled UEs should be orthogonal given that it is a simple enhancement from the network to achieve a better performance in MU-MIMO scenarios.

Proposal 8: Based on our results, MCS 13 2+2 in TDL-A channel with 16QAM interference is a good representative scenario to verify performance gain by R-ML receiver.

# Conclusion

Proposal 1: Introduce the RRC based signaling to inform UE the MCS index table(s) used for PDSCH of the co-scheduled UEs, or signal the maximum MCS index table (64QAM, 256QAM or 1024QAM table) used among all the (potential) co-scheduled UEs.

Proposal 2: If “no co-schedule UEs are presented in the allocated resource to the target UE” is signaled in DCI, combine “all the co-scheduled UEs are with different DMRS sequence than the target UE” in the same signaling without additional bits, i.e., have a code point representing “no co-schedule UEs are presented in the allocated resource to the target UE, or all the co-scheduled UEs are with different DMRS sequence than the target UE”

Observation 1: When PRG allocation is aligned across different CDM groups, the complexity of co-scheduled UE detection and the modulation order detection is reduced by at least x times, x is the PRG size.

Proposal 3: Introduce the signaling to inform UE that whether the PRG sizes and boundaries of the DMRS aligned co-scheduled UEs in different CDM groups with the target UE are aligned with the target UE. The signaling format depends on how static this information is in the real deployment. DMRS aligned co-scheduled UE refers to the co-scheduled UEs with the same DMRS sequence as the target UE.

Proposal 4: Introduce the signaling to inform UE numbers of CDM groups without data of the DMRS aligned co-scheduled UEs, or at least inform UE whether numbers of CDM groups without data are aligned across all DMRS aligned co-scheduled UEs. The signaling can be RRC based.

Observation 2: The co-scheduled UE detection can’t be performed in a per-symbol basis due to the availability of DMRS and complexity constraints, and therefore, the interference in R-ML receiver applied uniformly to all the PDSCH symbols. When the interference presence is not uniform in a slot, R-ML receiver tries to cancel a non-existent interference, and end up introduce additional noise to fail the decoding of the symbol.

Proposal 5: Introduce the following network assistant signaling to enable the correct interference cancellation when implementing R-ML receiver:

- Whether all the serving PDSCH symbols are interfered by the same set of the DMRS aligned co-scheduled UEs. RRC signaling is applicable if the condition is satisfied in most cases.

Proposal 6: Consider the following modulation order assistant signaling:

| Bitfield | coUE modulation order |
| --- | --- |
| 00 | No co-schedule UEs are presented in the allocated resource to the target UE, or all the co-scheduled UEs are with different DMRS sequence than the target UE |
| 01 | PRB aligned (Single coUE modulation per PRB or PRG) |
| 10 | All the co-scheduled UE has the same modulation order with the target UE |
| 11 | Other cases |

Alternatively, if we want to reduce the DCI bits, we can keep only the most critical information:

- 0: no co-schedule UEs are presented in the allocated resource to the target UE, or all the co-scheduled UEs are with different DMRS sequence than the target UE

- 1: other cases.

Observation 3: LMMSE-IRC and E-LMMSE-IRC are statistically equivalent and can be proved mathematically. Given that the channel estimation is done across multiple REs and the simulations are ran in a long enough duration to have converged throughput, we expect the E-LMMSE-IRC receiver to have exactly the same performance as LMMSE-IRC.

Proposal 7: Do not consider additional requirement for E-LMMSE-IRC receiver.

Proposal 8: When defining the requirement, the precoding matrices across co-scheduled UEs should be orthogonal given that it is a simple enhancement from the network to achieve a better performance in MU-MIMO scenarios.

Proposal 9: Based on our results, MCS 13 2+2 in TDL-A channel with 16QAM interference is a good representative scenario to verify performance gain by R-ML receiver.