3GPP TSG-RAN WG4#107	R4-2309353

Incheon, Korea, May 22nd – 26, 2023

Agenda item:	8.5.1.2

Source: 	Qualcomm Incorporated

Title: 	8RX UE RF requirements

Document for:	Approval

# Introduction

Considerations and proposals on 8RX UE RF requirements are provided in this contribution.

# Discussion

## Objectives of the WI

The WID [1] has the following objectives listed for 8RX:

Enable 8Rx for CPE/FWA/vehicle/industrial devices [RAN4]

- Example bands:

  - TDD bands: n41, n77/ n78

  - FDD bands: n7

    - Note 1: the total number of example band should be limited to 3. n77/n78 are considered as one band during the study.

    - Note 2: other bands to be introduced in the release independent way later on from Rel-18

    - Note 3: specifying requirements for TDD bands has first priority

- Specify the UE RF requirements to support 8Rx

- Study and specify the requirements to support SRS antenna switching for t1r8, t2r8, t4r8

  - Discussion on t4r8 shall start after at least one PC for 4Tx is completed

- NOTE: Requirements are specified with phase approach. Objectives with 1st priority are considered first.

## WF from RAN4#106bis-e

# Topic #1: ΔRIB for 8Rx

## Sub-topic 1-1 PDCCH aggregation level (AL)

Issue 1-1-1: Value of PDCCH AL in RAN4 assumption

<Agreement in GTW>

- No consensus on PDCCH aggregation level for 8Rx in RAN4, and RAN4 does not specify PDCCH aggregation level in RAN4 specification.

Issue 1-1-2: Information to RAN5

Moderator’s comments:

It is suggested to discuss with issue 1-2-1.

## Sub-topic 1-2 ΔRIB for TDD bands

Issue 1-2-1: ΔRIB for TDD bands

<Way forward/Agreement>:

RAN4 stop the discussion on the AL issue and directly discuss ΔRIB.

Further discuss ΔRIB for TDD bands with the following values as a baseline:

- - [4.0-4.5] dB for n41

- - [4.0] dB for n77/n78/n79

## Sub-topic 1-3 ΔRIB for FDD band

Issue 1-3-1: ΔRIB for FDD band n7

<Way forward/Agreement>:

- [4.5] dB for n7

# Topic #2: ΔTRxSRS

## Sub-topic 2-1 Values of ΔTRxSRS for n79

Issue 2-1-1: Values of ΔTRxSRS for n79

<Way forward/Agreement>:

For Values of ΔTRxSRS for n79, apply 5.5dB, 5.5dB, 6.0dB for 1T8R, 2T8R, 1T8R/2T8R respectively

Issue 2-1-2: General principle for ΔTRxSRS for n79

<Way forward/Agreement>:

Apply the same “+3Db”-principle as already agreed for bands up to n77 between different power classes/TX Diversity

<AH Agreement in last meeting>

- The same value with PC3 ∆TRxSRS applies

  - when the device is capable of power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 Db, or when UE indicating txDiversity-r16

- The value 3Db larger than PC3 ∆TRxSRS applies

  - during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is capable of power class 2 in the band and ΔPPowerClass = 0 Db and not indicating txDiversity-r16

## Sub-topic 2-2 Indication of ΔTRxSRS

Issue 2-2-1: Further analysis for indication of ΔTRxSRS

<Way forward/Agreement>:

No decision and agreements are made in this meeting, future discussion in RAN4 will depend on RAN1 feedback if any.

Issue 2-2-2: Applicability for 2Rx/4Rx

<Way forward/Agreement>:

No decision and agreements are made in this meeting, future discussion in RAN4 will depend on RAN1 feedback if any.

# Topic #3: Others

## Sub-topic 3-1 ΔPpowerclass for PCMAX_H,f,c

Issue 3-1-1: Whether to remove ΔPpowerclass for PCMAX_H,f,c

<Way forward/Agreement>:

Further discuss whether to remove the ΔPPowerClass=3dB applied for PCMAX_H,f,c for a PC2 capable UE with the support of TxD, if it further indicates the support of 1TxR AS-SRS.

Reference for the future discussion:

- Background

  - In current spec, ΔPPowerClass = 3dB applies to PCMAX_H,f,c for PC2 UE supporting TxD for 1TxR cases.

- The reason of proposal:

  - If UE supports PC2 with 23+26dBm or 26+26dBm PA configuration, UE can transmit with full power PA for 1TxR. So, applying ΔPPowerClass to PCMAX_H,f,c is not necessary for such Ues.

- The reason of objection:

  - If UE supports PC2 and indicates TxD capability, UE may do antenna virtualization and transmit 23+23dBm while it shouldn’t (no matter the PA configuration). To prevent this antenna virtualization and avoid 3dB uncertainty, applying  ΔPPowerClass to PCMAX_H,f,c is necessary.

## Sub-topic 3-2 Whether to remove or not the guard period between two SRS resources transmitted in different symbols of the same slot belonging to the same SRS resource set with ‘antennaSwitching’ usage

Issue 3-2-1: whether to remove or not the guard period between two SRS resources transmitted in different symbols of the same slot belonging to the same SRS resource set with ‘antennaSwitching’ usage

<Way forward/Agreement>:

Open issue needs further discussion

-	Option 1: Do not remove

-	Option 2: Remove

## Sub-topic 3-3 Release independent

Issue 3-3-1: Which release 8Rx can be release independent from.

<Way forward/Agreement>:

FFS the following approach can be agreeable in next meeting.

- 8Rx without SRS-AS can be release independent from Rel-15.

- 8Rx with SRS-AS can be release independent from Rel-X.

  - Further discuss “Rel-X” by clarifying which SRS-AS patterns were introduced from which release.

- Further discuss how to describe it in TS 38.307.

## Sub-topic 3-4 Which RF requirements to specify for 8Rx

Issue 3-4-1: Other RF Rx single carrier requirements (Proposal 1).

<Way forward/Agreement>

Further discuss the following options:

For UE equipped with 8Rx, the UE RF Rx requirements other than REFSENS are

- Option 1: Verified with 4Rx

  - “For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports, and additional requirements for eight Rx ports shall be verified in operating bands where the UE is equipped with eight Rx antenna ports

  - For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four Rx antenna ports and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four or eight Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports.”

- Option 2: Verified with 8Rx

  - “For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports, and additional requirements for eight Rx ports shall be verified in operating bands where the UE is equipped with eight Rx antenna ports

  - For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four or eight Rx antenna ports (selected based on the maximum Rx antenna port supported by the UE) and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four or eight Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports.”

- Option 3:

  - “For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four or eight Rx ports shall be verified in operating bands where the UE is equipped with four or eight Rx antenna ports (selected based on the maximum Rx antenna port supported by the UE), and additional requirements for eight Rx ports shall be verified in operating bands where the UE is equipped with eight Rx antenna ports

  - For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four or eight Rx antenna ports (selected based on the maximum Rx antenna port supported by the UE) and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four or eight Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports.”

- Option 4: Others

Issue 3-4-1: CA requirements.

<Way forward/Agreement>:

For 8Rx UE RF CA requirement, encourage interested companies to bring contributions in RAN4#107 with the following aspects:

- Expected clarification/revision on the objective in WID if needed considering whether to consider band combinations in this WI.

- For a band supporting 8Rx in single band operation, whether the band is mandated to support 8Rx in a band combination.

- For whether example CA/DC band combinations (including EN-DC) is needed or not, check if proposal 2 is agreeable in next meeting:

  - Proposal 2: 8Rx requirements should apply to the band implemented with 8Rx in all existing band combinations including intra/inter band CA and DC (including EN-DC).

- For whether additional MSD is needed, check if proposal 3 is agreeable in next meeting:

  - Proposal 3: Clarify the relationship between MSD and ΔRIB, 8R by adding the following sentence in section 7.3A.1.

    - For operations with 8 Rx antenna ports, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,8R in Table 7.3.2-x when MSD > 0.

# Discussion

## ΔRIB for 8Rx

After lots of discussions the group was finally able to find some common ground in RAN4#106bis-e on ΔRIB,8R.

Issue 1-2-1: ΔRIB for TDD bands

<Way forward/Agreement>:

RAN4 stop the discussion on the AL issue and directly discuss ΔRIB.

Further discuss ΔRIB for TDD bands with the following values as a baseline:

- - [4.0-4.5] dB for n41

- - [4.0] dB for n77/n78/n79

Issue 1-3-1: ΔRIB for FDD band n7

<Way forward/Agreement>:

- [4.5] dB for n7

Requirement for n77/n78/n79 is quite stable now, even tough 4.0dB was a compromise from our side. We also understand 4.0dB was a compromise from the companies who initially proposed higher number. Based on this, we propose to remove the brackets and specify 4.0dB for n77/n78/n79.

Requirements for n7 and n41 are differing in the WF. We are in the understanding that there will be a bunch of other bands proposed for 8RX like for any new RAN4 WI where general requirements are specified using some example bands.

Both n7 and n41 have -2.7dB ΔRIB,4R specified in 38.101-1 as illustrated above. As we know, they are overlapping in frequency as well. To settle the work, and very importantly to ensure smooth progress not only in this WID but also in future when new bands are being added we proposed to adopt a single delta for n7 and n41, which then can be used for other bands with 2.7dB ΔRIB,4R. We propose to specify -4.3dB for both n7 and n41. The rationale for this proposal is first and foremost to find a number which hopefully would address everyone’s points and concerns, and in addition would also serve as a good baseline for other MB/HB in future. In this proposal we have accounted multiple aspects and comments from the companies during the past meetings.

Proposal 1:

Specify the following ΔRIB,8R

- -4.0dB for n77/n78/n79

- -4.3dB for n7/n41

## ΔPpowerclass for PCMAX_H,f,c

Issue 3-1-1: Whether to remove ΔPpowerclass for PCMAX_H,f,c

<Way forward/Agreement>:

Further discuss whether to remove the ΔPPowerClass=3dB applied for PCMAX_H,f,c for a PC2 capable UE with the support of TxD, if it further indicates the support of 1TxR AS-SRS.

Reference for the future discussion:

- Background

  - In current spec, ΔPPowerClass = 3dB applies to PCMAX_H,f,c for PC2 UE supporting TxD for 1TxR cases.

- The reason of proposal:

  - If UE supports PC2 with 23+26dBm or 26+26dBm PA configuration, UE can transmit with full power PA for 1TxR. So, applying ΔPPowerClass to PCMAX_H,f,c is not necessary for such Ues.

- The reason of objection:

  - If UE supports PC2 and indicates TxD capability, UE may do antenna virtualization and transmit 23+23dBm while it shouldn’t (no matter the PA configuration). To prevent this antenna virtualization and avoid 3dB uncertainty, applying  ΔPPowerClass to PCMAX_H,f,c is necessary.

We don’t fully understand why UE would transmit SRS using TxD (23+23), as then the quality of the SRS would be compromised, or at least would not serve the purpose of sounding. The 3dB uncertainty concern in our understanding relates to if UE uses PC2 or PC3 PA to transmit the SRS which we do understand. We don’t have any specific proposal for this topic, but we remain positive on lifting the restriction in case a solution addressing both sides reasons is found.

## Whether to remove or not the guard period between two SRS resources transmitted in different symbols of the same slot belonging to the same SRS resource set with ‘antennaSwitching’ usage

Issue 3-2-1: whether to remove or not the guard period between two SRS resources transmitted in different symbols of the same slot belonging to the same SRS resource set with ‘antennaSwitching’ usage

<Way forward/Agreement>:

Open issue needs further discussion

-	Option 1: Do not remove

-	Option 2: Remove

This has been a kind of No/Yes -type of topic since the beginning of the WI. We don’t want to remove the guard period, so our proposal is option 1. This requirement has been in the specifications since the beginning of NR or so and has been generally applicable. With 8RX, the AS really does not get any easier than it is with 4RX devices.

We also note that his topic has been discussed under different context both in RAN1 and RAN4 earlier, without any changes to existing specifications.

Proposal 2: Option 1/Do not remove the guard period between two SRS resources transmitted in different symbols of the same slot belonging to the same SRS resource set with ‘antennaSwitching’ usage

## Release independent

Issue 3-3-1: Which release 8Rx can be release independent from.

<Way forward/Agreement>:

FFS the following approach can be agreeable in next meeting.

- 8Rx without SRS-AS can be release independent from Rel-15.

- 8Rx with SRS-AS can be release independent from Rel-X.

  - Further discuss “Rel-X” by clarifying which SRS-AS patterns were introduced from which release.

- Further discuss how to describe it in TS 38.307.

SRS-AS patterns and their capabilities in TS38.214 chapter 6.2.1.2 and in TS38.306 are specified as follows for Rel-15-Rel-17.

| Release | Capability name | SRS AS Patterns |  |
| --- | --- | --- | --- |
| 15 | supportedSRS-TxPortSwitch | t1r2. t1r4. t2r4. t2r2. t4r4. t1r4-t2r4. |  |
| 16 | supportedSRS-TxPortSwitch-v1610 | t1r1-t1r2. t1r1-t1r2-t1r4. t1r1-t1r2-t2r2-t2r4. t1r1-t2r2. t1r1-t2r2-t4r4. t1r1-t1r2-t2r2-t1r4-t2r4. |  |
| 17 | supportedSRS-TxPortSwitchBeyond4Rx-r17 | t1r6. t1r8. t2r6. t2r8. |  |

In our view there is no specific need to restrict release independence, so for instance Rel-15 8RX does not need to be restricted to be release independent without SRS AS, as from specification perspective one does not need to always sound all 8 antennas in which case tXr4 SRS AS could be used.

As shown above, the different SRS AS patterns are specified in different releases with capabilities. Each of the SRS antenna switching patterns are available from the releases they are specified, i.e. certain 4RX antenna switching patters from Rel-16 and 8RX antenna switching patters from Rel-17. This does not have impacts to release independence, as 8RX can be supported with e.g. 1T4R SRS (Rel-15) or with e.g. 1T6R or 1T8R SRS (Rel-17).

In our view 8RX could be release independent from Rel-15 or Rel-16. Looking at the NR release timelines, Rel-16 ASN.1 was completed June 2020 so likely 8RX would not need to be release independent from Rel-15 but Rel-16 instead.

Proposal 3: Specify 8RX release independent from Release 16. The SRS AS patterns are supported from the Release they were specified in

8RX can be addressed in TS38.307 as follows.

## Common UE RF requirements for 4Rx

The requirements and test cases listed in Table B.4.X-1 are specified in REL-16 version of TS 38.101-1.

Table B.4.X-1: Common UE RF requirements for 8Rx for single band in FR1

| Clause | Description |
| --- | --- |
| 7.3 | Reference sensitivity |
| 7.4 | Maximum input level |
| 7.5 | Adjacent Channel Selectivity |
| 7.6 | Blocking characteristics |
| 7.7 | Spurious response |
| 7.8 | Intermodulation characteristics |
| 7.9 | Spurious emissions |

SRS antenna patterns should not be an issue with respect to 38.307, as they are not mentioned for 2RX/4RX either

## Which RF requirements to specify for 8Rx

Issue 3-4-1: Other RF Rx single carrier requirements (Proposal 1).

<Way forward/Agreement>

Further discuss the following options:

For UE equipped with 8Rx, the UE RF Rx requirements other than REFSENS are

- Option 1: Verified with 4Rx

  - “For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports, and additional requirements for eight Rx ports shall be verified in operating bands where the UE is equipped with eight Rx antenna ports

  - For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four Rx antenna ports and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four or eight Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports.”

- Option 2: Verified with 8Rx

  - “For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports, and additional requirements for eight Rx ports shall be verified in operating bands where the UE is equipped with eight Rx antenna ports

  - For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four or eight Rx antenna ports (selected based on the maximum Rx antenna port supported by the UE) and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four or eight Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports.”

- Option 3:

  - “For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four or eight Rx ports shall be verified in operating bands where the UE is equipped with four or eight Rx antenna ports (selected based on the maximum Rx antenna port supported by the UE), and additional requirements for eight Rx ports shall be verified in operating bands where the UE is equipped with eight Rx antenna ports

  - For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four or eight Rx antenna ports (selected based on the maximum Rx antenna port supported by the UE) and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four or eight Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports.”

- Option 4: Others

For easier comparison, we collected the essence of each proposal into a single table below.

Table 1 Comparison of 8RX UE Single CC requirement options

| Option | REFSENS verified with | Other Single CC RF RX requirements verified with |
| --- | --- | --- |
| Option 1 | 2RX, 4RX, 8RX | 4RX |
| Option 2 | 2RX, 4RX, 8RX | 8RX |
| Option 3 | 2RX, 8RX | 8RX |

We note that in the context of this discussion we are only discussing single carrier (Non-CA) requirements. We want to note this, because the sentence “For Rx requirements other than single carrier REFSENS in Clause 7” includes CA requirements in addition to other single carrier in the current specification, which is not the case for 8RX as CA has not been discussed in that context yet.

Our original proposal was option 1, but after the discussion in previous meeting we are also ok to verify other single CC RF RX requirements in 8RX mode, assuming that then 2RX and 4RX mode for these will be skipped.

Proposal 4: Specify 8RX Single carrier requirements according to Option 2. To make it clear that this agreement, as written in WF is only for non-CA requirements the proposal 2 is modified for clarity (changes in red):

  - “For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports, and additional requirements for eight Rx ports shall be verified in operating bands where the UE is equipped with eight Rx antenna ports

  - For single carrier Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four or eight Rx antenna ports (selected based on the maximum Rx antenna port supported by the UE) and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four or eight Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports.”

NOTE: The intention is not to change 2RX/4RX UE requirements at all, the proposal about is only for 8RX and is subject to further modifications after 8RX CA is discussed

Issue 3-4-1: CA requirements.

<Way forward/Agreement>:

For 8Rx UE RF CA requirement, encourage interested companies to bring contributions in RAN4#107 with the following aspects:

- Expected clarification/revision on the objective in WID if needed considering whether to consider band combinations in this WI.

- For a band supporting 8Rx in single band operation, whether the band is mandated to support 8Rx in a band combination.

- For whether example CA/DC band combinations (including EN-DC) is needed or not, check if proposal 2 is agreeable in next meeting:

  - Proposal 2: 8Rx requirements should apply to the band implemented with 8Rx in all existing band combinations including intra/inter band CA and DC (including EN-DC).

- For whether additional MSD is needed, check if proposal 3 is agreeable in next meeting:

  - Proposal 3: Clarify the relationship between MSD and ΔRIB, 8R by adding the following sentence in section 7.3A.1.

    - For operations with 8 Rx antenna ports, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,8R in Table 7.3.2-x when MSD > 0.

The current WID is not including CA, however the wording is a bit vague “Enable 8Rx for..”. We think it’s a good idea to discuss and revise the WID to account 8RX CA requirements. First discussions can be carried out in RAN4#107 to collect companies views. Naturally, it is RAN Plenary’s task to make decisions.

UE supporting 8RX for a band should not be mandated to support 8RX in CA configuration with that band. Mandating 8RX support in CA would limit CA support in general. Hence proposal 2 cannot be accepted because it our understanding it would mandate UE supporting 8RX in Non-CA mode to support the same band in 8RX mode also in CA/EN-DC mode.

Proposal 3 is logical and in principle agreeable. However, it should be implemented into specifications together with other 8RX CA general requirements.

Proposal 5: Discuss if 8RX CA is in the scope of 8RX WI and revise the WI in RAN#100 if needed

Proposal 6: UE supporting 8RX in non-CA mode is not mandated to support the same band with 8RX in CA mode

Proposal 7: 8RX CA generic requirements should not be agreed until the scope of 8RX WI is discussed and possibly revised in RAN#100

# Conclusion

Considerations of 8RX UE RF requirements were provided with the following observations and proposals.

Proposal 1:

Specify the following ΔRIB,8R

- -4.0dB for n77/n78/n79

- -4.3dB for n7/n41

Proposal 2: Option 1/Do not remove the guard period between two SRS resources transmitted in different symbols of the same slot belonging to the same SRS resource set with ‘antennaSwitching’ usage

Proposal 3: Specify 8RX release independent from Release 16. The SRS AS patterns are supported from the Release they were specified in

Proposal 4: Specify 8RX Single carrier requirements according to Option 2. To make it clear that this agreement, as written in WF is only for non-CA requirements the proposal 2 is modified for clarity (changes in red):

  - “For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports, and additional requirements for eight Rx ports shall be verified in operating bands where the UE is equipped with eight Rx antenna ports

  - For single carrier Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four or eight Rx antenna ports (selected based on the maximum Rx antenna port supported by the UE) and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four or eight Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports.”

NOTE: The intention is not to change 2RX/4RX UE requirements at all, the proposal about is only for 8RX and is subject to further modifications after 8RX CA is discussed

Proposal 5: Discuss if 8RX CA is in the scope of 8RX WI and revise the WI in RAN#100 if needed

Proposal 6: UE supporting 8RX in non-CA mode is not mandated to support the same band with 8RX in CA mode

Proposal 7: 8RX CA generic requirements should not be agreed until the scope of 8RX WI is discussed and possibly revised in RAN#100

# Reference

- RP-230753, “Revised WID on Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1)”, Huawei, HiSilicon

- R4-2303518, “WF on 8Rx RF requirements”, NTT DOCOMO Inc.

- R5-103288, “PDCCH Aggregation Level for RF tests”, Rohde&Schwartz

- RP-172842, “New WI proposal: UE requirements for LTE DL 8Rx antenna ports”, Huawei, Hisilicon

- R4-2231393, “Considerations on 8RX UE RF requirements”, Qualcomm Inc.