# 3GPP TSG-RAN WG4 Meeting #107                                   R4-2309355
Incheon,KR  22 May - 26 May, 2023

Agenda item:	8.16.3

Source:	CAICT

Title:	Discussion on FR1 TRP TRS requirements work

Document for:	Approval

1	Introduction

At the RAN4 #106bis-e meeting, the WF and updated working procedure for Rel-18 TRP/TRS requirements are approved [1,2]. In this contribution, we provide our further views on Rel-18 TRP TRS requirements work.

2	Discussion

2.1 workplan for Rel-18 TRP TRS WI

In the approved Workplan for Rel-18 TRP TRS WI [3], the RC harmonization activity for 1Tx should be started in RAN4#106-bis meeting and collect harmonization measurement results in RAN4#107. At the same time, the lab alignment should also start in RAN4#107. However, at present, the framework for harmonization is not concluded yet.

To provide clear guidance for labs resource organization and ensuring the smooth progress of the WI, RAN4 needs to update the work plan or provide a schedule for AC lab alignment, RC lab alignment, RC and AC harmonization and performance test campaigns based on the current progress of Rel-18 TRP TRS WI.

Proposal 1: RAN4 should update the work plan or provide a schedule for AC lab alignment, RC lab alignment, RC and AC harmonization and performance test campaigns based on the current progress of Rel-18 TRP TRS WI.

Proposal 2: RAN4 should conclude the detailed framework for RC harmonization activity before RAN4#108.

2.2 working procedure for Rel-18 TRP TRS requirements

In RAN4#106bis-e, an updated working procedure for Rel-18 TRP TRS requirements is approved [2].

According to the agreed working procedure for lab alignment and test campaign, TRP and TRS Quantities based on Clenshaw-Curtis quadrature and traditional sin(theta) weighting are both allowed during performance campaign test. Meanwhile, either traditional measurement grids (15 degrees for TRP, 30 degrees for TRS) or newly defined coarser measurement grids can be used during performance test campaign.

- Working procedure for Rel-18 AC Lab Alignment Campaign - Lab alignment criteria: e) The summation form for TRP and TRS lab alignment should keep consistent during the calculation process of TRP TRS lab alignment from each company i.e. sin weights approach. Only traditional approach (15-degrees TRP and 30-degrees TRS) should be used during lab alignment activity to reduce unnecessary uncertainty. 2.2 Working procedure for TRP TRS Performance Test Campaign - Test results submitting: - TRP and TIS Quantities based on Clenshaw-Curtis quadrature and traditional sin(theta) weighting are both allowed during Performance campaign test. This information should be provided from each test lab when submitting measurement results. - Test lab procedures - Newly defined Coarser measurement grid can be used for TRP TRS measurement

The above principles are reasonable. Within the scope of the specifications allowed, lab alignment and test campaigns should not restrict what methods voluntary labs should adopt, but rather the labs should decide for themselves. However, the lab alignment activity is aimed at ensuring that the test results from different volunteered labs are comparable, thereby ensuring the accuracy of the final submitted measurement results during performance development phase. Therefore, we suggest that volunteer labs should use the same method as the test campaign phase to carry out lab alignment activities, i.e., measurement grid, TRP TRS summation form. For example, if the lab plans to use a newly defined coarser grids or CLC summation form during the test campaign, the same method should be used in the lab alignment activities.

Proposal 3: The volunteer labs should use the same method, such as measurement grids and TRP TRS summation form, as the test campaign phase to carry out lab alignment activities.

2.3 PC3 TRP requirements (if the band support PC2)

In RAN4#106bis-e, it was agreed that RAN4 can consider specifying the PC3 requirements for the corresponding band based on a fixed offset required by PC2 requirement, as follows:

Issue 5-3-1: PC3 TRP requirements (if the band support PC2) Agreements: - After specifying the PC2 requirements based on measurement campaign, RAN4 can consider to specify the PC3 requirement for that band, based on the 3dB offset, i.e. TRP(PC3) = TRP(PC2) – [3] dB. Encourage measurement results to further confirm this 3dB PC2~PC3 gap. Other value is not precluded. - The above PC3 requirement is applicable for UE only support PC3 at that band.

Considering that the gap in maximum output power allowed between PC2 and PC3 is 3dB, the fixed offset required by the performance requirement is considered as [3]dB, i.e. TRP(PC3) = TRP(PC2) – [3] dB.

However, as shown in table below, when UE is transmitted at maximum output power, an additional tolerance of +2/-3dB is allowed for PC2 and ±2dB for PC3. After years of development, the PC3 RF front-end link has become very mature, which means that PC3 UE can achieve an output power exceeding 24dBm to ensure better uplink coverage. However, due to factors such as the ability of PA, it is difficult for PC2 handheld UEs to transmit stably at power levels above 26dBm, especially in the FDD bands. Therefore, considering additional tolerances, the maximum transmission power difference between PC2 and PC3 UEs is smaller than 3dB.

| NR band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| n1 | 26 | +2/-3 | 23 | ±2 |
| n3 | 26 | +2/-33 | 23 | ±23 |
| n41 | 26 | +2/-33 | 23 | ±23 |
| n77 | 26 | +2/-3 | 23 | ±2 |
| n78 | 26 | +2/-3 | 23 | ±2 |
| n79 | 26 | +2/-3 | 23 | ±2 |
| NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. |  |  |  |  |

Observation 1: The gap between the maximum output power that PC2 and PC3 UEs can achieve is less than 3dB.

If defining a reasonable performance requirement for PC3 UEs based on the PC2 requirements, the actual implementation of UE should be considered instead of just relying on the power classes level without tolerance. Therefore, it is recommended to specify the fixed offset less than 3dB, e.g., 2dB. Further refinement based on measurement results is not precluded.

Proposal 4: After specifying the PC2 requirements based on measurement campaign, RAN4 can consider to specify the PC3 requirement for that band, based on the [2]dB offset. Further refinement based on measurement results is not precluded.

3	Conclusion

This contribution makes the following observations and proposals.

Proposal 1: RAN4 should update the work plan or provide a schedule for AC lab alignment, RC lab alignment, RC and AC harmonization and performance test campaigns based on the current progress of Rel-18 TRP TRS WI.

Proposal 2: RAN4 should conclude the detailed framework for RC harmonization activity before RAN4#108.

Proposal 3: The volunteer labs should use the same method, such as measurement grids and TRP TRS summation form, as the test campaign phase to carry out lab alignment activities.

Observation 1: The gap between the maximum output power that PC2 and PC3 UEs can achieve is less than 3dB.

Proposal 4: After specifying the PC2 requirements based on measurement campaign, RAN4 can consider to specify the PC3 requirement for that band, based on the [2]dB offset. Further refinement based on measurement results is not precluded.

References

- R4-2305904, WF for Rel-18 TRP TRS requirements, vivo, 3GPP RAN4 #106bis-e, April 2023

- R4-2305907, Updated working procedure for Rel-18 TRP TRS requirements, vivo, 3GPP RAN4 #106bis-e, April 2023

- R4-2217455, Workplan of Rel-18 TRP TRS WI, vivo, 3GPP RAN4#104bis-e, October 2022.