3GPP TSG-RAN4 Meeting #107	R4-2309427

Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Source: 	Qualcomm Incorporated

Title: 	Discussion on Enhanced CHO configuraitons

Agenda item:	8.25.5

Document for:	Discussion

# Introduction

In this paper, we would like to share our view for remaining issues from the last RAN4 #106bis-e meeting[1].

# Discussion

Issue 3-3-1: PCell handover delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4) - Candidate solutions: - Option 1: same as that in obj. 3. - Option 2: DCHOwithCPAC = Max [DCHO, DCPAC] - DCHO = TRRC + TEvent_DU + Tmeasure + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin - where all detail delay components shall follow TS38.133 clause 6.1.4.2 legacy definition - DCPAC = TRRC_delay + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms - where all detail delay components shall follow TS38.133 clause 8.9A.2 legacy definition - Option 3: same as CHO - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution and Tinterrupt = Tprocessing + TIU + T∆ + Tmargin, except that - Tprocessing = 25 ms - TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC. Issue 3-3-2: PSCell handover delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4) - Candidate solutions: - Option 1: - if conditions for CPC are met before conditions for CHO are met, follow objective 3 requirements. - If conditions for CPC are met after CHO is complete, reuse existing CPC requirements. - Option 2: DCHOwithCPAC = Max [DCHO, DCPAC] - DCHO = TRRC + TEvent_DU + Tmeasure + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin - where all detail delay components shall follow TS38.133 clause 6.1.4.2 legacy definition - DCPAC = TRRC_delay + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms - where all detail delay components shall follow TS38.133 clause 8.9A.2 legacy definition - Option 3: - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms, where - The definitions of TRRC, TEvent_DU, Tmeasure, TCHO_execution, Tprocessing are the same as the definitions in the delay requirements for Pcell - TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

From RAN2 agreement, interruption due to different ending point of TeventDU is not expected.

Baseline: The UE waits until both CHO and CPC conditions are met (always). (furthermore, it is assumed that if needed the network can provide a complementary CHO-only configuration, to avoid failures in deployments where failure would otherwise be likely to happen). Alternative: FFS if When CHO condition is met, but CPC condition is not met, CHO execution is triggered (and somehow source SCG can be released). IF allowed in the new configuration the UE may continue evaluation of CPC/CPA conditions.

Observation: when UE waits until both CHO and CPC conditions are met, the CHO with CPC/CPA is similar to HO with PSCell.

Proposal: CHO with CPC/CPA can follow same mechanism as R17 HO with PScell when UE waits until both CHO and CPC conditions are met. Tevent_DU definition needs to be updated to capture UE to wait until both CHO and CPC conditions are met.

    - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution and Tinterrupt = Tprocessing + TIU + T∆ + Tmargin, except that

    - Tprocessing = 25 ms

    - TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

    - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms, where

    - The definitions of TRRC, TEvent_DU, Tmeasure, TCHO_execution, Tprocessing are the same as the definitions in the delay requirements for Pcell

    - TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

# Conclusion

Observation: when UE waits until both CHO and CPC conditions are met, the CHO with CPC/CPA is similar to HO with PSCell.

Proposal : CHO with CPC/CPA can follow same mechanism as R17 HO with PScell when UE waits until both CHO and CPC conditions are met. Tevent_DU definition needs to be updated to capture UE to wait until both CHO and CPC conditions are met.

    - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution and Tinterrupt = Tprocessing + TIU + T∆ + Tmargin, except that

    - Tprocessing = 25 ms

    - TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

    - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms, where

    - The definitions of TRRC, TEvent_DU, Tmeasure, TCHO_execution, Tprocessing are the same as the definitions in the delay requirements for Pcell

    - TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

# References

[1] R4-2306356, WF on NR Mobility Enhancements RRM requirements (part 2), RAN4 #106bis-e