3GPP TSG-RAN WG4 Meeting #107	R4-2309448
Incheon, KR, May 22 – May 26, 2023

Agenda item:	7.29.2

Source:	Huawei, Hisilicon

Title:	R18 4Rx Handheld UE requirements

Document for:	Approval

# Introduction

In the RAN4 106-bis meeting the following points were left for the further discussion in the WF[1]

Issue 1-2-1: How to specify requirements for low band 4Rx (<1GHz) Agreement: - Requirements is needed for 4Rx handheld UE, if the feasibility is confirmed - FFS whether the new requirements will be specified or the existing requirements will be reused. Issue 1-2-2: ΔRIB,4R - Proposals - Option 1: -2.7dB - Option 2: -2.2 dB - Option 3: Middle value between -2.7dB and -2.2dB. WF: - Further discuss in next meeting.

In this tdoc we express our view on these two issues.

# How to specify requirements for low band 4Rx (<1GHz)

Note 1 in Table 7.3.2-2 of TS 38.101-1 as depicted below, gives a clear indication that those bands are designated for 4Rx in FWA. If the idea was not to preclude handheld UEs from those bands, the presence of Note 1 is making the requirements more confusing and its removal could be helpful, in case the final decision is not to specify specific requirements for handheld UEs.

As the handheld UEs have more challenges for having 4Rx in a smaller form-factor than FWA, they should have a different set of specific requirements, if the feasibility study is confirmed.

Observation 1: Remove Note 1 from Table 7.3.2-2, if the final decision is not to specify specific requirements for handheld UEs

Proposal 1: As the handheld UEs have more challenges for having 4Rx in a smaller form-factor than FWA, they should have a different set of specific requirements

# ΔRIB,4R

Theoretically when the number of receivers changes from 2 to 4, there should be a gain of 3dB in REFSENS. The simulations, during the LTE evaluation of 4RX, showed the difference between 2RX and 4RX REFSENS could of 2.5 dB[2]. Moreover the REFSENS is, also, related to other parameters including implementation margin. As REFSENS is a conducted receiver parameter, the number of antennas does not play a role in its value directly. However the more antennas we have, in a limited space (handled UE), the more coupling effects we might have between their RF paths (implementation margin increases). Moreover as these antennas are not equidistant from the LNA, there could be imbalance between the receiver RF chains. Therefore based on the simulated 4Rx simulations and the implementation margin, it would be better to include these degradations into the REFSENS value, hence we suggest a ΔRIB,4R = -2.2dB for handheld UEs. For FWA UEs, ΔRIB,4R = -2.5dB should be enough

Proposal 2: We suggest a ΔRIB,4R = -2.2dB for handheld UEs and ΔRIB,4R = -2.5dB for FWA UEs

# Conclusions

Observation 1: Remove Note 1 from Table 7.3.2-2, if the final decision is not to specify specific requirements for handheld UEs

Proposal 1: As the handheld UEs have more challenges for having 4Rx in a smaller form-factor than FWA, they should have a different set of specific requirements

Proposal 2:   We suggest a ΔRIB,4R = -2.2dB for handheld UEs and ΔRIB,4R = -2.5dB for FWA UEs

# References

[1]	“R4-2306553, WF on low band 4Rx, vivo.”

[2]	“R4-156287, UE reference sensitivity for 4AP UE, Ericsson.”