3GPP TSG-RAN WG4 Meeting #107	R4-2309449
Incheon, KR, May 22 – May 26, 2023

Agenda item:	7.29.3

Source:	Huawei, Hisilicon

Title:	R18 3Tx NR CA SAR compliance and configured output power

Document for:	Approval

# Introduction

In the RAN4 106-bis meeting the following points were left for the further discussion in the WF[1]

Issue 2-2-4: Pcmax changes - Proposals - Option 1: Define configured transmitted power by considering up to PC1.5 (as below figure) for inter-band CA/EN-DC PC1.5 with 3Tx. Issue 2-5-1: PC1.5 inter-band CA/EN-DC SAR compliance - Proposals - Option 1: Use 0.5*maxUplinkDutyCycle-interBandCA-PC2 if indicated as the threshold for PC1.5 UL duty cycle, below which PC1.5 requirements apply. If absent, UE shall work on power class PC1.5 regardless of UL duty cycle and may use P-MPRc as defined in 6.2.4 in TS 38.101-1 or other means if necessary. - Option 2: For PC1.5 band combination, SAR compliance with the possible new signalling for the duty cycle mechanism need to be further studied. - Option 3: Study whether the existing UE capabilities (PC2-related maxUplinkDutyCycle) can be applied or new UE capabilities are needed.

In this tdoc we express our view on these two issues.

# PC1.5 inter-band CA/EN-DC SAR compliance

For single band operations the maximum output power adaptation with UL duty cycle for PC2 and PC1.5 are explained in TS 38.101-1 section 6.2.1. Its explanatory flow chart is available in slide 6 of [2]. For PC1.5 there is no special UE capability for the maximum UL duty cycle, and it is set as 0.5* maxUplinkDutyCycle-PC2-FR1 where its value can be 0.5*[50%, 60%, 70%, 80%, 90%, 100%]. The choice of coefficient 0.5 is because the maximum power of PC1.5 3dB greater than PC2 hence by dividing its duty cycle by a factor of 2, one can find the same SAR limitations. Based on the TS 38.306 in case of absence of maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 (which is a parameter about the maximum permissible exposure specific for FWA UEs) a maximum duty cycle of 50% and 25% shall be considered, by default, for PC2 and PC1.5, respectively.

As a specific UE capability for PC1.5 maximum UL duty cycle was not needed for a single band operator, it is preferable to keep the same methodology for the inter band CA.

In the case of inter band CA a UE capability for the maximum duty cycle in PC2 is considered, maxUplinkDutyCycle-interBandCA-PC2-r17 where its value can be [50%, 60%, 70%, 80%, 90%,100%]. In its absence, the UE will operate as PC2 regardless of the UL duty cycle and emitted power is controlled by P-MPRc as defined in TS 38.101-1 section 6.2.4.  In the case of PC1.5 inter-band CA, the maximum output power is 3dB higher than PC2 inter-band CA, hence it is logical to divide its UL duty cycle by a factor of 2 to reach the same SAR limits. Therefore the maximum UL duty cycle is set as 0.5* maxUplinkDutyCycle-interBandCA-PC2-r17 where its value can be 0.5*[50%, 60%, 70%, 80%, 90%, 100%].

Regarding PC1.5 inter-band EN-DC SAR requirements, as the WI does not include any EN-DC band combination, there is no need to discuss it at the time being.

Observation 1:   for PC1.5 there is no special UE capability for the maximum UL duty cycle. It is set as 0.5*maxUplinkDutyCycle-PC2-FR1 where its value can be 0.5*[50%, 60%, 70%, 80%, 90%, 100%].

Observation 2:   In single band operation, in case of absence of maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16, a maximum duty cycle of 50% and 25% shall be considered, by default, for PC2 and PC1.5, respectively

Observation 3:   In inter-band CA operation, In case of absence of maxUplinkDutyCycle-interBandCA-PC2-r17, the emitted power is controlled by P-MPRc, regardless of the duty cycle.

Proposal 1:   For PC1.5 in inter band CA configuration, the maximum UL duty cycle is set as 0.5* maxUplinkDutyCycle-interBandCA-PC2-r17 where its value can be 0.5*[50%, 60%, 70%, 80%, 90%, 100%].

Proposal 2:   As the WID does not include any PC1.5 EN-DC band combination, there is no need to discuss its SAR requirements at the time being.

# Pcmax changes

In section 6.2A.4.1.3 of TS 38.101-1, ΔPPowerClass,CA is used for the case where due to SAR limitation, the UE would need to control its output power and fall back to the default power class. Currently the default UE power class is PC3, so a ΔPPowerClass,CA =3dB was considered for PC2 UEs. In case of PC1.5, as the default power class is still PC3 then a ΔPPowerClass,CA= 6dB would be needed to reach down to 23 dBm.

Proposal 3:   For PC1.5,  ΔPPowerClass,EN-DC = 6 dB in section 6.2A.4.1.3 of Ts38.101-1 to ensure a fall back to the default power class

# Conclusions

Observation 1:   For PC1.5 there is no special UE capability for the maximum UL duty cycle. It is set as 0.5*maxUplinkDutyCycle-PC2-FR1 where its value can be 0.5*[50%, 60%, 70%, 80%, 90%, 100%].

Observation 2:   In single band operation, in case of absence of maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16, a maximum duty cycle of 50% and 25% shall be considered, by default, for PC2 and PC1.5, respectively

Observation 3:   In inter-band CA operation, In case of absence of maxUplinkDutyCycle-interBandCA-PC2-r17, the emitted power is controlled by P-MPRc, regardless of the duty cycle.

Proposal 1:   For PC1.5 in inter band CA configuration, the maximum UL duty cycle is set as 0.5* maxUplinkDutyCycle-interBandCA-PC2-r17 where its value can be 0.5*[50%, 60%, 70%, 80%, 90%, 100%].

Proposal 2:   As the WID does not include any PC1.5 EN-DC band combination, there is no need to discuss its SAR requirements at the time being.

Proposal 3:   For PC1.5,  ΔPPowerClass,EN-DC = 6 dB in section 6.2A.4.1.3 of Ts38.101-1

# References

[1]	“R4-2306554, WF on Tx requirements for 3Tx UE, OPPO.”

[2]	“R4-2220002, Proposal to clarify Power Class control logic, T-Mobile USA et al.”