3GPP TSG-RAN WG4 Meeting #107 	R4-2309494

Incheon, Korea, May 22 – 26, 2023

Title:	Draft LS on enhanced FR2 measurements at connection setup/resume

Response to:

Release:	Rel-18

Work Item:	Further Mobility Enhancements (NR_Mob_enh2)

Source:	RAN4 #107

To:	RAN2

Cc:	RAN

Contact person:	Jani-Pekka Kainulainen

jani-pekka.kainulainen@nokia.com,

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN4 has been working on WID objective 7 on NR mobility enhancements, which (as per RP-223520) is defined as per the following

- To study and specify how to reuse the IDLE/INACTIVE mode measurement results which are to be reported during and/or after RRC connection setup/resume in order to improve SCell/SCG setup delay [RAN4, RAN2], including: - Availability and validation of the IDLE/INACTIVE mode measurement results to be reported [RAN4]; and - Definition of corresponding RRM requirements [RAN4]; and - If necessary based on RAN4 outcome, definition of corresponding signalling support [RAN2]. Note 5: RAN4 will coordinate in due course with RAN2 to start the work. Note 6: R4-2220415 serves as baseline for future work in RAN4 Note 7: With exception of the above scenarios, enhancements on IDLE/INACTIVE mode measurements and on UE behavior in IDLE/INACTIVE mode are not in scope.

As the objective states, what RAN4 aims to do is improve the availability of UE measurements to allow for faster FR2 SCG/SCell setup at/after RRC connection setup/resume, with RAN2 to define the corresponding signalling support based on RAN4 input. To this end, RAN4 has been investigating means to allow UE to indicate the status of its FR2 candidate cell measurements to network, and has made the following agreements:

Basic procedure: UE (operating under FR1 cell) can be configured to provide network with the status of measurements of inter-band FR2 cell(s) on configured frequency (or frequencies) during RRC connection setup. UE may also perform additional measurements starting from RRC connection setup/resume procedure.

Measurement procedure: Once UE initiates RRC connection procedure, it may perform additional measurements and/or validate the existing measurements in order to indicate the availability and/or validity of the FR2 candidate cell(s).

- The number of frequencies for such measurements is reduced compared to the number of EMR measurements defined in Rel-16.

- Enhancements on UE behaviour in IDLE/INACTIVE mode are not in the scope, i.e. UE is not required (but is allowed) to perform additional measurements during IDLE/INACTIVE before the RRC connection is initiated.

- DRX is not in use during RRC connection setup/resume procedure for enhanced measurement and UE may reuse measurements done for cell reselection and/or EMR for these measurements.

- FFS: how to select the frequency layers for improved measurement, how many frequency layers to measure and which scenario that is applicable, and how long the measurement procedure may continue after the RRC connection setup procedure.

- It is FFS whether the same procedures as defined for inter-band FR2 candidate cells can also be used for FR1 candidate cells (the WI scope is restricted to FR2 candidate cells, so RAN4 focuses on inter-band target cell in FR2, but if final solution to be agreed can cover intra-band and FR1 without extra standardization effort, it is unnecessary to exclude these two scenarios).

Measurement reporting procedure: UE reports the status of the FR2 candidate cell measurement procedure at the end of the RRC connection setup/resume (i.e. at RRCSetupComplete or RRCResumeComplete).

- The exact signalling details of reporting measurement results from UE to network is up to RAN2 to define.

- FFS whether to introduce indication of availability and/or validity status of measurement results from UE to NW into the [existing] EMR reports.

UE capabilities: The required UE capabilities are FFS.

- RAN4 expects to have at least different behaviours for UEs capable of EMR and UEs not capable of EMR.

- Solution based on existing measurement and solution based on enhanced measurement are not mutual exclusive.

The precise wording of the agreements per meeting can also be found in the documents as listed in the below table:

| RAN4 #106-bis-e: R4-2306396 | <Agreement>: - Solution based on existing measurement and solution based on enhanced measurement are not mutual exclusive. The two solutions are be discussed in parallel. <Agreement>: - FFS whether to introduce indication of availability and/or validity status of measurement results from UE to NW into the [existing] EMR reports. |
| --- | --- |
| RAN4 #106: R4-2303176 | <Agreement> - RAN4 shall focus on inter-band target cell in FR2. If final solution to be agreed can cover intra-band and FR1 without extra standardization effort, it is unnecessary to exclude these two scenarios. <Agreement> - UE is allowed to reuse existing measurement, including legacy measurement for cell re-selection and EMR. - UE is allowed to perform addition measurement starting from RRC connection setup/resume procedure. - RAN4 can continue discussion on the feasibility of doing additional measurement starting from RRC setup/resume, and requirements shall be defined if feasible solution is agreed. |
| RAN4 #105 R4-2220415 | · <Agreement>: - Enhancements on UE behavior in IDLE/INACTIVE mode are not in the scope - The following aspects can be further studied - Using measurement results obtained during IDLE/INACTIVE mode for measurements during RRC connection procedure - Note: enhancements on IDLE/INACTIVE mode measurement are not in the scope |
| RAN4 104-bis-e R4-2217260 | No Agreements |
| RAN4 #104-e R4-2214348 | <Agreement >: - Reduce the number of EMR carriers to be measured for improved measurement. - FFS: how to select the frequency layers for improved measurement, how many frequency layers to measure and which scenario that is applicable. < Agreement>: - Further discuss the feasibility of improvement in FR2 SCell/SCG setup delay. < Agreement>: - DRX is not in use during RRC connection setup/resume procedure for enhanced measurement. |

While the RAN4 discussion on some of the solution details is still ongoing, RAN4 has found that a solution is feasible so RAN4 would request RAN2 to start creating the necessary signalling and procedures based on the above agreements.

# 2	Actions

To RAN2

ACTION: RAN4 respectfully asks RAN2 to take RAN4 agreements and the information provided above into account.

# 3	Dates of the next TSG RAN4 meetings

RAN WG4 Meeting #108		Aug 21 – Aug 25, 2023,			Toulouse, France

RAN WG4 Meeting #108		Nov 13 – Nov 17, 2023,			Chicago, US