3GPP TSG-RAN WG4 #107	R4-2309501

May 22nd ‒ 26th, 2023

Incheon, KR

Agenda item:	5.2.8.3

Source: 	Qualcomm Incorporated

Title: 	NTN UE maximum input power

Document for:	Discussion

# Introduction

The maximum input power is improperly specified in the NTN specifications.  It represents a condition that can never be observed in the field since the path loss between the UE and the satellite is so large, yet imposes design constraints on the UE that negatively impact cost and complexity.

# Discussion

## Maximum input power

The UE’s receive dynamic range is lower bounded by its reference sensitivity and upper bounded by the maximum input power.  This represents a very wide range of operation that is implemented in the UE by multiple discrete gain stages in the receive chain.  The current maximum input power requirement for the NTN UE in 38.101-5 mandates the UE to receive a signal at -40 dBm.  Additionally, this requirement is specified with 64QAM R=3/4 modulation requiring excellent SNR so very high linearity and low noise floor in the UE’s receive chain.

Table 7.4-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 5, 10, 15, 20 |
| Power in Transmission Bandwidth Configuration3 | dBm | -402 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-2 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Power in transmission bandwidth configuration value is rounded to the nearest 0.5dB value. |  |  |

It has already been studied and recognized that the highest received power at the UE is expected to be -77.2 dBm (clause 7.4.3.2.3 of TR 38.863) and used as a justification to relax the maximum input power from the TN specified value of -25 dBm to -40 dBm for NTN.  However, -40 dBm is still nearly 40 dB higher than what the UE would ever receive from its NTN serving cell.  As opposed to a TN UE, the NTN UE will never be able to approach the SAN (unless it’s onboard a rocket or spaceship or the SAN is falling out of the sky but those extreme scenarios can be safely ignored) so will never have the luxury of receiving such a strong signal.  Given the limited off-axis and adjacent channel suppression available for the downlink, if an on-axis signal were actually to be received at -40 dBm on the ground, the interference to other users on adjacent channels or other orbital slots would be unacceptably high.  For a LEO-600 satellite at S-band frequencies, the free space path loss is 154 dB.  To meet the -40 dBm maximum input power, the Tx EIRP from the satellite would need to be 84 dBW to maintain 34 dBW/MHz power density.  Additionally, since the modulation is 64QAM, the transmitter at the satellite would need to be backed off in power to deliver the necessary EVM so the required PA power is even higher.  There is insufficient power and size at the satellite to accommodate such numbers.  Therefore, a maximum input power requirement of -40 dBm is entirely unrealistic and does not provide any benefit to an NTN deployment.

While it is clear that there is zero probability to receive a satellite signal at -40 dBm on the Earth, it is also worth mentioning that imposing such a requirement negatively impacts the UE design and cost.  The UE’s front-end must be designed to tolerate and even demodulate with >95% throughput a signal of this magnitude.  The dynamic range of the front-end, the number of gain stages in the LNA, the RF power detector and switching logic, and the linearity requirement are all impacted.  The design and cost of the UE will be needlessly increased for no benefit.  The opportunity to produce a lower cost design targeted for example to IoT applications will be lost.

Proposal 1:  Correct the UE maximum input power to -70 dBm or lower.  Adjust the RMC to QPSK R=1/3.

## Adjacent channel selectivity

The ACS specifications are written to accommodate two cases.  ACS case 1 is specified for the serving cell Rx power near sensitivity while ACS case 2 is specified for maximum input power.  As described above, requirements for the UE at maximum input power of -40 dBm are not beneficial for NTN.  Due to the nature of an NTN network with the relatively fixed path loss between the NTN UE and the SAN, there is no strong need for an ACS case 2 requirement since the case 2 condition does not occur in the field.  Moreover, the expected use case for NTN is when the the UE out of coverage of the terrestrial network.  In this case, it is not expected that the UE is receiving a very strong (relatively speaking) signal from the satellite while at the same time as very strong interfering signal from a terrestrial network in the adjacent channel.

Proposal 2:  Modify the ACS case 2 requirement

Option 1:  Remove the ACS case 2 requirement

Option 2:  Adjust the ACS case 2 wanted signal power to -70 dBm or lower.

# Conclusion

This contribution has reiterated the already studied conclusion that the maximum input power for an NTN UE on the ground received from even the LEO-600 orbiting satellite is expected to be no higher than -77.2 dBm.  Therefore, specifying a requirement of -40 dBm for the UE’s maximum input power is unnecessary and provides no benefit.  However, it does impose additional design challenges to the UE to meet the additional dynamic range adding to its cost and complexity.  The design focus for the NTN UE should be in the low power regime.  It is therefore proposed in this contribution to modify the maximum input power to -70 dBm which still provides 7 dB of margin.  Similarly, the ACS case 2 is not appropriate since it is intended to test the ACS performance of the UE at a very high received power.  Such a condition will never be observed in the field for an NTN network.

Proposal 1:  Correct the UE maximum input power to -70 dBm or lower.  Adjust the RMC to QPSK R=1/3.

Proposal 2:  Modify the ACS case 2 requirement

Option 1:  Remove the ACS case 2 requirement

Option 2:  Adjust the ACS case 2 wanted signal power to -70 dBm or lower.

For proposal 2 on ACS, we prefer option 1 as the case 2 condition is not necessary for NTN.

A CR [2] has been provided to implement both of the above proposals.

# Reference

- TR 38.863

- R4-2309502, “Correction to maximum input power and ACS,” Qualcomm Incorporated