3GPP TSG-RAN WG4 #107	R4-2309506

May 22nd ‒ 26th, 2023

Incheon, KR

Agenda item:	9.5.2

Source: 	Qualcomm Incorporated

Title: 	Guard band discussion for the extended L band

Document for:	Approval

# Introduction

A work item [1] has been agreed to define new band for LTE NB-IoT in the extended L band frequency range.  The initial scope of the work item is limited to Region 1 suggesting that the band may be expanded to include other regions of the world in a future work item.  This contribution reviews the UE coexistence requirements associated with this new band.  In particular, as a follow-up to [3] the ECC Report 263 [2] is examined more closely to formulate a proposal for the band plan in 3GPP.

# Discussion

A summary of the outcome of the ECC Report 263 [2] was previously provided in [3] where it was proposed to define the extended L band incorporating a 3 MHz guard band between the NTN band and the closest IMT band.  This meant the extended L band’s downlink would be truncated by 3 MHz

Proposal:  Define the Extended L-band downlink range lower edge at 1521 MHz to provide 3 MHz guard band with TN bands

which has the obvious disadvantage of un-utilized spectrum.  Given that the extended L band, assuming the L band (Band n255) is not available, is only 2x7 MHz to being with, truncating 3 MHz of the downlink is a severe penalty.

To review, the ECC Report 263 concludes the following

Based on the final results of its compatibility studies, it is concluded that:

The minimum in-band blocking characteristic for land mobile earth stations receivers from a 5 MHz broadband signal interferer (LTE) operating below 1518 MHz shall be −30dBm above 1520 MHz;

The base station unwanted emission limits e.i.r.p. for a broadband signal interferer (LTE) operating below 1518 MHz shall be −30dBm/MHz above 1520 MHz. This figure is 10 dB more stringent than ECC Decision (13)03 due to a different service in the adjacent band.

It is noted that the IMT block ends at 1517 MHz.

when the MES operates above 1520 MHz

Our interpretation from this conclusion but also the body of the report is the ECC recommends a 3 MHz frequency separation between the upper edge of the IMT (which is assumed to be 1517 MHz for Europe but potentially up to 1518 MHz in other countries in the Middle East) and the lower edge of the MSS band which is assumed to operate above 1520 MHz.  Moreover, the ECC concludes that the MES receiver should be resilient to an LTE blocking signal at -30 dBm and the IMT OOBE should be at least -30 dBm/MHz with the 3 MHz guard band.  In fact, Inmarsat [4] also recognizes the potential for “significant potential interference risk to MESs” from MBB in the 1427 – 1518 MHz band.  Inmarsat endorses the ECC recommended 3 MHz guard band between MBB and MSS to mitigate the interference, but proposes to place the guard band inside the MBB (IMT) band instead of the MSS band.

Based on the above, the utility of a 3 MHz guard band between the IMT and MSS band appears to be justified.  The following options are listed below for discussion.

- The guard band is placed 1515 – 1518 MHz (proposal in [4]) and UE blocking requirements for MSS band are specified according to ECC Report 263,

- The guard band is placed 1518 – 1521 MHz (proposal in [3]) and UE blocking requirements for MSS band are specified according to ECC Report 263,

- The need for a guard band, its width, the IMT BS OOBE, and the UE blocking should be independently studied by 3GPP rather than following the outcome of ECC Report 263,

- The guard band is not “hardcoded” into the 3GPP band definitions but rather left to the deployment of channels either in the IMT band and/or the MSS band to enable a 3 MHz guard band, UE blocking requirements for the MSS band are specified according to ECC Report 263

- Option 5.  Define two extended L bands – one with 3 MHz guard band and one without.  UE blocking is specified according to ECC report 263 for the band with 3 MHz guard band.  In this way, the operator can choose which band to deploy depending on whether coexistence with IMT is required.

Option 1.  Since 3GPP bands 74/n74, 75/n75, n92, and n94 have already been defined up to 1517 MHz or 1518 MHz, equipment has already been developed, operators may have already deployed networks, redefining these bands to enable a guard band at their upper edge is not recommended.

Option 2.  This is the most straightforward option because it creates the necessary guard band, this is the newest band to be defined without any prior 3GPP deployment (i.e., it is not a legacy 3GPP band), and defining the band in this manner allows the UE Rx filter to provide protection against blocking from the IMT band.  However, it is recognized that this option robs 43% of the downlink bandwidth away from the extended L band.

Option 3.  An independent coexistence study in 3GPP can be expected to be time intensive and very likely to produce a similar outcome to the comprehensive study already performed by the ECC.  This option is not recommended.

Option 4.  This option is redirecting the problem to deployment.  It requires the NTN and TN operators in the same geographical area to coordinate their deployments to ensure a 3 MHz guard band.  Since neither the NTN operator nor the TN operator is likely to willingly carve out a guard band in their own deployment, this option has a low probability of success.  But the option does enable full use of the spectrum for deployments where there is no coexistence concern; i.e., if a TN network is not deployed.  Without embedding the guard band in the formal definition of the band range, the NTN UE Rx filter will not be able to reject the blocker.  Hence, blocking performance is diminished for this option compared to options 1 and 2.  However, since the Rx filter is not able to provide meaningful rejection (additional confirmation is welcomed) with only 3 MHz separation, the blocker advantage from the filter is available perhaps only from the second IMT channel.

Option 5.  With this option for deployments where there is no coexistence, the full band is available since the full 1518 – 1525 MHz band could be broadcast.  For deployments where there is coexistence, blocking performance of the NTN UE is enhanced and IMT OOBE is reduced with a guard band according to ECC Report 263.  However, since the coverage area of the satellite is so vast and TN deployments can evolve over time, it may be challenging to identify which downlink beams must broadcast the truncated band and which ones can broadcast the full band.  Another disadvantage of this option is the UE support.  It is unlikely for a UE to populate two separate RF filters in its front-end.  Therefore, it should be expected that the UE front-end will consist of a filter with cutoff frequency at 1518 MHz (even for the band starting at 1521 MHz) and therefore will not be able to provide rejection against the IMT blocker.  The baseband filter, however, will be able to provide some rejection but cannot provide protection to the previous Rx chain stages such as the LNA and mixer.  Also, while the UE can still take advantage of the guard band to reduce the IMT BS OOBE, but UE blocking is expected to be degraded compared to option 1 or 2.

From the discussion of the above-listed options, there does not appear to be an ideal solution.  Each option carries with it significant disadvantages whether that be the loss in spectrum or the possibility of interference.  In an interference scenario, the spectrum is anyways not useful for service so whether its loss comes from carving out a guard band or from interference is immaterial.  Our recommendation is option 5.  With option 5, the NTN operator is provided a choice to deploy with a guard band (but lose 3 MHz of spectrum) to mitigate the probability of interference, even if the TN operator deploys later in time and without the need to specifically identify which areas might be vulnerable to interference, etc.  The NTN operator can also choose to deploy the full band, but with the understanding that interference leading to service outage may be present and may not be easily predictable.  Since 3GPP should not publish specifications where service outage is expected or unpredictable, a note of warning should be included in the specification illustrating the risk of harmful interference if the full band is deployed.  The NTN operator then takes full responsibility if deploying without protection mechanisms offered by the truncated band.  For example, the NTN operator may choose to take this risk to maximize available spectrum if it has other means and willingness to contain the risk, for example, other channels within the beam footprint, bilateral agreements with TN operators, etc.  The problem of UE filtering remains.  With a desire to minimize UE fragmentation and UE designs that only support one of the two bands, it should be assumed the UE will carry a common RF front-end between the two bands – this implies that Rx filter will be designed to cover the full band.

Proposal:  Option 5 is selected.  Two bands are defined where one has interference protection by including a 3 MHz guard band in the extended L band and enhanced UE blocking (FFS).  The second band does not include the interference mitigation mechanisms but enables usage of the full extended L band.  A note is included in the 3GPP specification to indicate the risk for harmful interference, but the NTN operator is free to choose which band to deploy depending on needs and its ability to otherwise contain the risk.

UE blocking performance is still to be studied.  Depending on the outcome of the study and the feasibility of UE blocking, the appropriate specifications should be defined no matter which band option is selected.  UE blocking is the subjected for a separate paper [5].

# Conclusion

The extended L band is in close proximity to a number of TN bands and may therefore cause or experience interference when deployed in the same geographical area.  The scope of the work item is limited to Region 1, so the TN bands of potential interference have been identified as Bands 74/n74, 75/n75, n92, and n94.  The ECC CEPT in Europe conducted a study on compatibility between IMT adjacent to the extended L band and produced ECC Report 263 with a set of recommendations.  The recommendations include frequency separation (guard band), IMT BS out-of-band emissions, and enhanced MES UE blocking.  This contribution focuses on the guard band and offers five options for consideration of how this guard band could be implemented in 3GPP specifications.  After a discussion of the advantages and disadvantages, a recommendation is provided to define two bands to enable the NTN operator to make an choice based on its specific needs and ability to otherwise circumvent interference.

Proposal:  Option 5 is selected.  Two bands are defined where one has interference protection by including a 3 MHz guard band in the extended L band and enhanced UE blocking (FFS).  The second band does not include the interference mitigation mechanisms but enables usage of the full extended L band.  A note is included in the 3GPP specification to indicate the risk for harmful interference, but the NTN operator is free to choose which band to deploy depending on needs and its ability to otherwise contain the risk.

# Reference

- RP-230790, “New WID on Introduction of the Extended L-band (UL 1668-1675, DL 1518-1525) for IoT NTN,” Inmarsat

- ECC Report 263, “Adjacent band compatibility studies between IMT operating in the frequency band 1492-1518 MHz and the MSS operating in the frequency band 1518-1552 MHz,” Approved 03 March 2017.  https://docdb.cept.org/document/967

- R4-2305825, “Extended L band for LTE NB-IoT,” Qualcomm Incorporated

- International Satellite Symposium 2017, “Part 1: From WRC-15 to WRC-19 – Setting the basis,” Inmarsat https://www.itu.int/en/ITU-D/Regional-Presence/AsiaPacific/Documents/Events/2017/Aug-ISS2017/PPT_S1_Bob_Horton.pdf

- R4-2309507, “UE blocking requirements for the Extended L band,” Qualcomm Incorporated