3GPP TSG RAN WG4 Meeting #107	R4-2309592

Incheon, 22 May 2023 – 26 May 2023

Agenda Item:	8.15.5

Source: 	Ericsson

Title:	Discussion on RRM requirements for NR in less than 5 MHz bandwidth

Document for:	Discussion

# Introduction

In this contribution, we look at the RRM requirements that may be impacted due to introduction of NR in less than 5 MHz bandwidth.

# Discussion

Following issues are open after last meeting discussions.

## Open issues on impact to RRM requirements

### Impact on UE L1-RSRP Requirements

In last meeting there was no potential impact observed for SSB based L1-RSRP requirements, but it was agreed that there could be potential impact to CSI-RS based L1-RSRP requirements. However, there was open issue about only prioritising SSB based L1-RSRP.  Following WF is agreed.

Way forward:

Add ‘Prioritize SSB based L1 measurements’ for CSI-RS L1-RSRP measurements:

- Proposals

  - Option 1: Agree with adding ‘Prioritize SSB based L1 measurements’ for CSI-RS L1-RSRP measurements

  - Option 2: Disagree with adding ‘Prioritize SSB based L1 measurements’ for CSI-RS L1-RSRP measurements (include reasoning)

We think many NW implementation prefer configuring CSI RS based L1-RSRP than SSB based L1-RSRP. To give configuration flexibility to those implementations, we prefer to define CSI-RS based L1-RSRP requirements.

- RAN4 to define CSI-RS based L1-RSRP measurement requirements

### Impact on UE L1-SINR Requirements

In last meeting following WF is agreed.

Way forward:

- Proposals

  - Firstly discuss

    - Option 1: Define L1-SINR requirements

    - Option 2: Do not define L1-SINR requirements

  - Secondly discuss

    - Option 3: Define requirements for 3MHz CBW

    - Option 4: Do not define requirements for 3MHz CBW

    - Option 5: Define requirements for 5MHz CBW

    - Option 6: Do not define requirements for 5MHz CBW

In our understanding L1-SINR is defined as part of eMIMO WI in Rel-16 and we do not see a strong need to define requirements for L1-SINR.

- RAN4 to agree not to define L1-SINR requirements

In RAN4#106-bis-e, RAN4 agreed that there could be potential impact on following requirements and agreed to perform further analysis on the impact. In the following table we list the agreed reason for impact.

| Impacted requirement | Reason for impact |
| --- | --- |
| Idle mode requirements | SSB index reading Evaluation period |
| Inactive mode requirements | SSB index reading Evaluation period |
| Handover requirements | SSB Index reading MIB reading delay |
| RRC Re-establishment | SSB Index reading MIB reading delay |
| RRC Connection Release with Redirection Requirements | SSB Index reading MIB reading delay |
| UE RLM Requirements | SSB: reduced BW (Current hypothetical BW for SSB is 24PRBs (4.32MHz)) CSI-RS: reduced BW (Current hypothetical BW for CSI-RS is 48PRBs(8.64MHz)) |
| Link Recovery Procedures (SSB based BFD and CBD) Link Recovery Procedures (CSI-RS based BFD and CBD) | Current hypothetical BW for SSB is 24PRBs (4.32MHz)) Current hypothetical BW for CSI-RS is 48PRBs(8.64MHz)) |
| SSB based Intra-frequency measurements (cell detection and measurements) |  |
| SSB based Inter-frequency measurements (cell detection and measurements) |  |

From the above table we can observe that most of the requirements are impacted due to SSB index reading delay and MIB reading delay. We provide simulation results for analyse the potential impact.

## Impact on SSB Index reading

We consider following simulation parameters as baseline and simulated changing different parameters and we present our observations in these sections.

| Parameter | Unit | Value |
| --- | --- | --- |
| Number of PRBs for PBCH |  | Baseline: 12 PRBs for 3MHz CBW Optional: 15 PRBs for 3MHz CBW |
| Carrier frequency | GHz | 900MHz |
| Subcarrier spacing | kHz | 15 kHz |
| Number of Tx antennas | - | Baseline: 1 Optional: 2 |
| Number of Rx antennas | - | 2 |
| DMRS | - | 3GPP NR PBCH DMRS |
| Other assumptions |  | Tx BW and SSB puncturing are known at the Rx side |
| CP Length | - | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 |
| SS burst set periodicity | ms | 20 |
| Frequency Offset relative to UE frequency reference | Hz | 0 |
| PBCH symbols within the SS block |  | PSS-PBCH-SSS-PBCH |
| Data and Control Power offset with respect to PSS and SSS | dB | Baseline 0 |
| PBCH power offset with respect to PBCH-DMRS | dB | 0 |
| PBCH-DMRS power offset with respect to PSS and SSS | dB | 0 |
| PSS and SSS sequences | - | No changes expected |
| PBCH-DMRS sequences | - | No changes expected |
| PBCH-DMRS RE positions within the PBCH resource | - | No changes expected except for puncturing impact |
| PBCH Channel coding |  | No changes expected to actual Channel coding (Polar code with 512 length and 24bit CRC) |
| PBCH Modulation | - | QPSK |
| PBCH Payload (including the CRC) | bits | 56bit (CRC 24bit) |
| PBCH SNR | dB | -10 : 0 dB, with 1 dB spacing |
| Propagation Condition / Channel models | - | For 3 km/h UE speed - AWGN - TDL-A 30ns - TDL-B 100ns - TDL-C 300ns For 500km/h UE speed - Single tap model Additional scenarios can be considered in future meetings |
| UE speed |  | 3 km/h, 500 km/h |
| Detection Method |  | Baseline: One shot detection (i.e. no combination for different PBCHs) Optional: - Non coherent combining for SSB index reading - Soft combining for MIB reading FFS which method(s) will be used to define the requirement(s) |
| Metrics |  | See below |
| NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned. |  |  |

Simulation assumption: PRB 12, SNR -10:0db; Speed: 3km/h; Channel model: AWGN

Results:

Figure 1 SSB index reading error rate for different number of SSB transmissions

Observations:

- For AWGN with 12 PRB punctured SSB, at least one SSB is required to achieve 99% successful decoding of the PBCH SSB Index at SNR = -9 dB.

Simulation assumption: PRB 15, SNR -10:0dB; Speed: 3km/h; Channel model: TDL-A 30ns

Results:

Figure 2 SSB index reading error rate for different number of SSB transmissions

Observations:

- For TDL-A 30 ns, with 15 PRB punctured SSB, at least two SSBs are required to achieve 99% successful decoding of the PBCH SSB Index at SNR = -6 dB.

Simulation assumption: PRB 15, SNR -10:0dB; Speed: 3km/h; Channel model: TDL-B 100ns

Results:

Figure 3 SSB index reading error rate for different number of SSB transmissions

Observations:

- For TDL-B 100 ns, with 15 PRB punctured SSB, at least two SSBs are required to achieve 99% successful decoding of the PBCH SSB Index at SNR = -6 dB.

Simulation assumption: PRB 15, SNR -10:0dB; Speed: 3km/h; Channel model: TDL-C 300ns

Results:

Figure 4 SSB index reading error rate for different number of SSB transmissions

Observations:

- For TDL-C 300 ns, with 15 PRB punctured SSB, at least two SSBs are required to achieve 99% successful decoding of the PBCH SSB Index at SNR = -6 dB.

Based on the observations 2 to 4, we make following proposal.

- For 15 PRB, at least 2 SSB are required to achieve 99% successful decoding of the SSB Index reading under SNR side condition of -6dB.

Simulation assumption: PRB 12, SNR -10:0db; Speed: 500km/h; Channel model: HST single tap with 2*416.67Hz constant Doppler shift

Results:

Figure 5 SSB index reading error rate for different number of SSB transmissions

Observations:

- For HST single tap model with 2*416.67Hz constant Doppler shift, with 12 PRB punctured SSB, at least one SSB is required to achieve 99% successful decoding of the PBCH SSB Index at SNR = -8 dB.

Simulation assumption: PRB 15, SNR -10:0dB; Speed: 500km/h; Channel model: HST single tap with 2*416.67Hz constant Doppler shift

Results:

Figure 6 SSB index reading error rate for different number of SSB transmissions

Observations:

- For HST single tap model with 2*416.67Hz constant Doppler shift, with 15 PRB punctured SSB, at least one SSB is required to achieve 99% successful decoding of the PBCH SSB Index at SNR = -8 dB.

Based on the observations 5 to 6, we make following proposal.

- For 12 and 15 PRB for HST scenario, at least 1 SSB is required to achieve 99% successful decoding of the SSB Index reading under SNR side condition of -8dB.

## Impact on MIB reading delay

We consider following simulation parameters as baseline and simulated changing different parameters and we present our observations in these sections.

| Parameter | Unit | Value |
| --- | --- | --- |
| Number of PRBs for PBCH |  | Baseline: 12 PRBs for 3MHz CBW Optional: 15 PRBs for 3MHz CBW |
| Carrier frequency | GHz | 900MHz |
| Subcarrier spacing | kHz | 15 kHz |
| Number of Tx antennas | - | Baseline: 1 Optional: 2 |
| Number of Rx antennas | - | 2 |
| DMRS | - | 3GPP NR PBCH DMRS |
| Other assumptions |  | Tx BW and SSB puncturing are known at the Rx side |
| CP Length | - | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 |
| SS burst set periodicity | ms | 20 |
| Frequency Offset relative to UE frequency reference | Hz | 0 |
| PBCH symbols within the SS block |  | PSS-PBCH-SSS-PBCH |
| Data and Control Power offset with respect to PSS and SSS | dB | Baseline 0 |
| PBCH power offset with respect to PBCH-DMRS | dB | 0 |
| PBCH-DMRS power offset with respect to PSS and SSS | dB | 0 |
| PSS and SSS sequences | - | No changes expected |
| PBCH-DMRS sequences | - | No changes expected |
| PBCH-DMRS RE positions within the PBCH resource | - | No changes expected except for puncturing impact |
| PBCH Channel coding |  | No changes expected to actual Channel coding (Polar code with 512 length and 24bit CRC) |
| PBCH Modulation | - | QPSK |
| PBCH Payload (including the CRC) | bits | 56bit (CRC 24bit) |
| PBCH SNR | dB | -10 : 0 dB, with 1 dB spacing |
| Propagation Condition / Channel models | - | For 3 km/h UE speed - AWGN - TDL-A 30ns - TDL-B 100ns - TDL-C 300ns For 500km/h UE speed - Single tap model Additional scenarios can be considered in future meetings |
| UE speed |  | 3 km/h, 500 km/h |
| Detection Method |  | Baseline: One shot detection (i.e. no combination for different PBCHs) Optional: - Non coherent combining for SSB index reading - Soft combining for MIB reading FFS which method(s) will be used to define the requirement(s) |
| Metrics |  | See below |
| NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned. |  |  |

Simulation assumption: PRB 12, SNR -10:0db; Speed: 3km/h; Channel model: AWGN

Results:

Figure 7 PBCH BLER for different number of SSB transmissions

Observations:

- For AWGN, with 12 PRB punctured SSB, at least two SSBs are required to achieve 99% successful decoding of the MIB at SNR = -6 dB.

Simulation assumption: PRB 15, SNR -10:0dB; Speed: 3km/h; Channel model: TDL-A 30ns

Results:

Figure 8 PBCH BLER for different number of SSB transmissions

Observations:

- For TDL-A 30ns, with 15 PRB punctured SSB, at least 6 SSBs are required to achieve 99% successful decoding of the MIB at SNR = -4 dB.

Simulation assumption: PRB 15, SNR -10:0dB; Speed: 3km/h; Channel model: TDL-B 100ns

Results:

Figure 9 PBCH BLER for different number of SSB transmissions

Observations:

- For TDL-B 100ns, with 15 PRB punctured SSB, at least 5 SSBs are required to achieve 99% successful decoding of the MIB at SNR = -4 dB.

Simulation assumption: PRB 15, SNR -10:0dB; Speed: 3km/h; Channel model: TDL-C 300ns

Results:

Figure 10 PBCH BLER for different number of SSB transmissions

Observations:

- For TDL-C 300ns, with 15 PRB punctured SSB, at least 4 SSBs are required to achieve 99% successful decoding of the MIB at SNR = -4 dB.

Based on the observations 8 to 10, we make following proposal.

- For 15 PRB punctured SSB, 6 SSB are required to achieve 99% successful decoding of the MIB under SNR side condition of -4dB. Number of SSB required for SNR side condition of -6dB is FFS.

Simulation assumption: PRB 12, SNR -10:0db; Speed: 500km/h; Channel model: HST single tap with 2*416.67Hz constant Doppler shift

Results:

Figure 11 PBCH BLER for different number of SSB transmissions

Observations:

- For HST single tap with 2*416.67Hz constant Doppler shift, with 12 PRB punctured SSB, at least 2 SSBs are required to achieve 99% successful decoding of the MIB at SNR = -6 dB.

Simulation assumption: PRB 15, SNR -10:0dB; Speed: 500km/h; Channel model: HST single tap with 2*416.67Hz constant Doppler shift

Results:

Figure 12 Figure 11 PBCH BLER for different number of SSB transmissions

Observations:

- For HST single tap with 2*416.67Hz constant Doppler shift, with 15 PRB punctured SSB, at least one SSB is required to achieve 99% successful decoding of the MIB at SNR = -6 dB.

Based on the observations 11 to 12, we make following proposal.

- For 12 and 15 PRB punctured SSB, 2 and 1 SSB are required respectively to achieve 99% successful decoding of the MIB under SNR side condition of -6dB.

## Impact on RLM

For the radio link monitoring out-of-sync, RAN4 specified the evaluation period to determine the radio link quality is worse than the threshold Qout and indicate the out-of-sync to the higher layer. The threshold Qout corresponding to hypothetical PDCCH BLER of 10% with the transmission parameters shown in Table 1. The link quality is evaluated for SSB or CSI-RS according to the network configuration. When we look the PDCCH transmission parameters, it is assumed the PDCCH BW is 24RB and aggregation level is 8 with the power boosting of 4dB.

For the radio link monitoring in-sync, RAN4 specified the evaluation period to determine the radio link quality is better than the threshold Qin and indicate the in-sync to the higher layer. The threshold Qin corresponding to hypothetical PDCCH BLER of 2% with the transmission parameters shown in Table 1. The link quality is evaluated for SSB or CSI-RS according to the network configuration. When we look the PDCCH transmission parameters, it is assumed the PDCCH BW is 24RB and aggregation level is 4.

Table 2 shows the evaluation period for CSI-RS based BFD specified in TS38.133 8.1.3. For CSI-RS based RLM, the evaluation period is based on the number of CSI-RS samples (Mout  for out-of-sync and Min for in-sync), measurement gap factor (P), CSI-RS transmission periodicity (TCSI-RS), and DRX cycle length (TDRX). According to TS38.133, Mout = 20 and Min = 10, are derived with the assumption CSI-RS is Density = 3 and over the bandwidth ≥ 24 PRBs.

- For RLM OOS, Qout is derived based on the assumption PDCCH is transmitted with AL8 with power boosting of 4dB.

- For RLM IS, Qin is derived based on the assumption PDCCH is transmitted with AL4.

- CSI-RS based RLM evaluation period is derived based on the assumption CSI-RS is transmitted with 24RB.

Table 1	PDCCH transmission parameters for RLM Out-of-sync and In-sync.

| Attribute | Out-of-sync | In-sync |
| --- | --- | --- |
| DCI format | 1-0 | 1-0 |
| Number of control OFDM symbols | 2 | 2 |
| Aggregation level (CCE) | 8 | 4 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB | 0dB |
| Bandwidth (PRBs) | 24 | 24 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size | REG bundle size |
| REG bundle size | 6 | 6 |
| CP length | Normal | Normal |
| Mapping from REG to CCE | Distributed | Distributed |

Table 2	CSI-RS based RLM evaluation period for FR1

Table 8.1.3.2-1: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR1 Configuration; TEvaluate_out_CSI-RS (ms); TEvaluate_in_CSI-RS (ms). no DRX; Max(200, Ceil(Mout×P)×TCSI-RS); Max(100, Ceil(Min×P) × TCSI-RS). DRX ≤ 320ms; Max(200, Ceil(1.5×Mout×P)× Max(TDRX, TCSI-RS)); Max(100, Ceil(1.5×Min×P)× Max(TDRX, TCSI-RS)). DRX > 320ms; Ceil(Mout×P) × TDRX; Ceil(Min×P) × TDRX. NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10ms, 20 ms or 40 ms. TDRX is the DRX cycle length.. The values of Mout and Min used in Table 8.1.3.2-1, Table 8.1.3.2-2, Table 8.1.3.2-3 and Table 8.1.3.2-4 are defined as: - Mout = 20 and Min = 10, if the CSI-RS resource configured for RLM is transmitted with higher layer CSI-RS parameter density [6, clause 7.4.1] set to 3 and over the bandwidth ≥ 24 PRBs.

According to RAN1 agreements, when the transmission channel bandwidth is 3MHz, the available RB size is 15 or 16, which is not enough to transmit assumed PDCCH symbols for OOS (i.e., AL8) and CSI-RS bandwidth (24RB). Also PDCCH power boosting assumption 4dB is the maximum value according to TS38.104 Table 6.3.2.2-1. On the other hand, it is possible to transmit PDCCH for IS even if the transmission CBW is 15RB.

The thresholds, Qout and Qin, are based on the hypothetical PDCCH BLER. This means gNB does not need to configure them in the real network. We therefore propose RAN4 does not modify the transmission parameters but the UE capable of 3MHz CBW should derive Qout and Qin with the assumption the PDCCH is transmitted with AL8 and AL4, respectively. With the setup UE can keep the same Qout and Qin regardless of the transmission BW.

- For RLM OOS and IS for CBW less than 5MHz, thresholds Qout and Qin should be derived with the assumption the PDCCH is transmitted with the same parameters as TS38.133 8.1.2.1 and 8.1.3.1 even if the PDCCH transmission bandwidth exceeds the transmission channel bandwidth.

Smaller CSI-RS bandwidth may affect to the channel quality measurement accuracy. One option is to extend the evaluation period by about 1.5 (≈ 24RB / 15RB) to keep the same measurement accuracy. On the other hand, in our understanding, SSB-based RLM evaluation is based on SSS. Since SSS is not punctured, we can apply the same evaluation period for SSB-based RLM.

- RAN4 can keep the existing SSB based RLM evaluation periods TEvaluate_out_SSB and TEvaluate_in_SSB for CBW less then 5MHz.

- RAN4 should extend the CSI-RS based RLM evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for CBW less then 5MHz, where Mout = [30] (=1.5*20) for OOS and Mout = [15] (=1.5*10) for IS for CSI-RS within the channel bandwidth below 24RB.

## Link recovery procedures

Link recovery procedures mainly consists of beam failure detection and candidate beam detection.

### Beam failure detection (BFD)

For the beam failure detection, RAN4 specified the evaluation period to determine the radio link quality is worse than the threshold Qout_LR and indicate the beam failure to the higher layer. The threshold Qout_LR corresponding to hypothetical PDCCH BLER of 10% with the transmission parameters shown in Table 3. The link quality is evaluated for SSB or CSI-RS according to the network configuration. When we look the PDCCH transmission parameters, it is assumed the PDCCH BW is 24RB and aggregation level is 8.

Table 4 shows the evaluation period for CSI-RS based BFD specified in TS38.133 8.5.3. For CSI-RS based BFD, the evaluation period is based on the number of CSI-RS samples (MBFD), measurement gap factor (P), CSI-RS transmission periodicity (TCSI-RS), DRX cycle length (TDRX), and sharing factor (PBFD). According to TS38.133, MBFD = 10, is derived with the assumption CSI-RS is Density = 3 and over the bandwidth ≥ 24 PRBs.

- For BFD, Qout_LR is derived based on the assumption PDCCH is transmitted with AL8.

- CSI-RS based BFD evaluation period is derived based on the assumption CSI-RS is transmitted with 24RB.

Table 3	PDCCH transmission parameters for beam failure instance

| Attribute | Value for BLER |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 8 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (PRBs) | 24 |
| Sub-carrier spacing (kHz) | Same as the SCS of RMSI CORESET |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

Table 4	CSI-RS based BFD evaluation period for FR1

Table 8.5.3.2-1: Evaluation period TEvaluate_BFD_CSI-RS for FR1 Configuration; TEvaluate_BFD_CSI-RS (ms). no DRX; Max(50, Ceil(MBFD P PBFD) TCSI-RS). DRX cycle ≤ 320ms; Max(50, Ceil(1.5 × MBFD P PBFD) Max(TDRX, TCSI-RS)). DRX cycle > 320ms; Ceil(MBFD P PBFD) TDRX. Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length.. The values of MBFD used in Table 8.5.3.2-1 and Table 8.5.3.2-2 are defined as - MBFD = 10, if the CSI-RS resource(s) in set used for BFD is transmitted with Density = 3 and over the bandwidth ≥ 24 PRBs.

According to RAN1 agreements, when the transmission channel bandwidth is 3MHz, the available RB size is 15 or 16, which is not enough to transmit assumed PDCCH symbols (i.e., AL8) and CSI-RS bandwidth (24RB). For the threshold, Qout_LR, it is based on the hypothetical PDCCH BLER. We therefore propose RAN4 does not modify the transmission parameters but the UE capable of 3MHz CBW should derive Qout_LR with the assumption the PDCCH is transmitted with AL8. With the setup UE can keep the same Qout_LR regardless of the transmitted CBW.

- For BFD for CBW less than 5MHz, threshold Qout_LR, should be derived with the assumption the PDCCH is transmitted with the same parameters as TS38.133 Table 8.5.2.1-1 even if the PDCCH transmission bandwidth exceeds the transmission channel bandwidth.

Smaller CSI-RS bandwidth may affect to the channel quality measurement accuracy. One option is to extend the evaluation period by about 1.5 (≈ 24RB / 15RB) to keep the same measurement accuracy. On the other hand, in our understanding, SSB-based BFD evaluation is based on SSS. Since SSS is not punctured, we can apply the same evaluation period for SSB-based BFD.

- RAN4 apply the existing SSB based BFD evaluation period TEvaluate_BFD_SSB for CBW less then 5MHz.

- RAN4 should extend the CSI-RS based BFD evaluation period TEvaluate_BFD_CSI-RS for CBW less then 5MHz, where MBFD = [15] (=1.5*10) for CSI-RS within the channel bandwidth below 24RB.

### Candidate beam detection (CBD)

For the candidate beam detection, RAN4 specified the evaluation period to detect one SSB or CSI-RS based beam whose L1-RSRP exceeds the threshold rsrp-ThresholdSSB signalled by the network.

Table 5 shows the evaluation period for CSI-RS based CBD specified in TS38.133 8.5.6. For CSI-RS based CBD, the evaluation period is based on the number of CSI-RS samples (MCBD), measurement gap factor (P), CSI-RS transmission periodicity (TCSI-RS), DRX cycle length (TDRX), and sharing factor (PCBD). According to TS38.133, MCBD = 3, is derived with the assumption CSI-RS is Density = 3 and over the bandwidth ≥ 24 PRBs.

Table 5	CSI-RS based CBD evaluation period for FR1

Table 8.5.6.2-1: Evaluation period TEvaluate_CBD_CSI-RS for FR1 Configuration; TEvaluateC_CBD_CSI-RS (ms). non-DRX, DRX cycle ≤ 320ms; Max(25, Ceil(MCBD P PCBD) TCSI-RS). DRX cycle > 320ms; Ceil(MCBD P PCBD) TDRX. Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length.. The values of MCBD used in Table 8.5.6.2-1 and Table 8.5.6.2-2 are defined as - MCBD = 3, if the CSI-RS resource configured in the set is transmitted with Density = 3 and over the bandwidth ≥ 24 PRBs.

- CSI-RS based CBD evaluation period is derived based on the assumption CSI-RS is transmitted with 24RB.

Smaller CSI-RS bandwidth may affect to the L1-RSRP measurement accuracy used for CBD. One option is to extend the evaluation period by about 1.5 (≈ 24RB / 15RB) to keep the same measurement accuracy. On the other hand, in our understanding, SSB-based CBD evaluation is based on SSS. Since SSS is not punctured, we can apply the same evaluation period for SSB-based CBD.

- RAN4 apply the existing SSB based CBD evaluation period TEvaluate_CBD_SSB for CBW less then 5MHz.

- RAN4 should extend the CSI-RS based CBD evaluation period TEvaluate_CBD_CSI-RS for CBW less then 5MHz, where MBFD = [5] (= 1.5*3) for CSI-RS within the channel bandwidth below 24RB.

## L1 measurement requirements

As a part of CSI reporting, UE shall report the L1-RSRP measurement results for the configured SSB and/or CSI-RS resources. The network configures the L1-RSRP reporting with periodic reporting with PUCCH, semi-persistent reporting with PUCCH/PUSCH, or aperiodic reporting with PUSCH. For L1-RSRP reporting, RAN4 specified the measurement period.

Table 6 shows the measurement period of CSI-RS based L1-RSRP for FR1 used for beam reporting specified in TS38.133 9.5.4. For CSI-RS based L1-RSRP measurement, the evaluation period is based on the number of CSI-RS samples (M), measurement gap factor (P), CSI-RS transmission periodicity (TCSI-RS), DRX cycle length (TDRX), high speed relaxation factor (K), and the minimum measurement period of TReport (configured periodicity of L1-RSRP reporting). The number of CSI-RS samples, M, depends on the configuration; it is set to 1 if timeRestrictionForChannelMeasurement is configured and set to 3 otherwise.

Unlike CSI-RS based RLM/BFD/CBD, CSI-RS based L1-RSRP measurement period does not limit the CSI-RS bandwidth. Therefore we propose RAN4 reuse the existing CSI-RS based L1-RSRP measurement period requirements for the CBW less than 5MHz.

For SSB based L1-RSRP measurement, like SSB based RLM/BFD/CBD, UE uses SSS for L1-RSRP estimation. Since RAN1 agreed SSS is not punctured even if the transmission CBW is less than 5MHz, we propose RAN4 reuse the existing SSB based L1-RSRP measurement period requirements for the CBW less than 5MHz.

- RAN4 apply the existing SSB based L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB for CBW less then 5MHz.

- RAN4 apply the existing CSI-RS based L1-RSRP measurement period TL1-RSRP_Measurement_Period_CSI-RS for CBW less then 5MHz.

Table 6	CSI-RS based L1-RSRP measurement period for FR1

Table 9.5.4.2-1: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR1 Configuration; TL1-RSRP_Measurement_Period_CSI-RS (ms). non-DRX; max(TReport, ceil(M*P)*TCSI-RS). DRX cycle ≤ 320ms; max(TReport, ceil(K *M*P)*max(TDRX,TCSI-RS)). DRX cycle > 320ms; ceil(M*P)*TDRX. Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. Note 3: K = 1 when TCSI-RS ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 4: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 or measurementEnhancementCA-r17.. The value of TL1-RSRP_Measurement_Period_CSI-RS is defined in Table 9.5.4.2-1 for FR1 and in Table 9.5.4.2-2 for FR2, where - For periodic and semi-persistent CSI-RS resources, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise - For aperiodic CSI-RS resources M=1

RAN4 also specified the L1-RSRP measurement accuracy requirements based on the measurement period above. The clause below is CSI-RS based L1-RSRP absolute accuracy in FR1 specified in TS38.133 10.1.19. Unlike L1-RSRP measurement period, the existing CSI-RS based L1-RSRP accuracy requirements are set based on the assumption CSI-RS bandwidth is 48, which is much larger than the assumption.

- CSI-RS based L1-RSRP measurement accuracy requirements are valid for CSI-RS bandwidth of 48 or more.

Since most exiting frequency bands can support 10MHz CBW, it is feasible to consider 48RB to specify the measurement accuracy requirements. For this WI, however, RAN4 is going to specify the requirements for CBW less than 5MHz. Therefore RAN4 need to discuss whether to define CSI-RS based L1-RSRP measurement accuracy requirements with CSI-RS is less than 24RB in the performance part. If RAN4 apply the same measurement period TL1-RSRP_Measurement_Period_CSI-RS, for example, RAN4 may need to consider to relax the accuracy requirements.

- In the performance part, RAN4 should discuss whether to define CSI-RS based L1-RSRP measurement accuracy requirements with CSI-RS is less than 24RB.

#### 10.1.19.2 CSI-RS based L1-RSRP accuracy requirements ##### 10.1.19.2.1 Absolute Accuracy Unless otherwise specified, the requirements for absolute accuracy of CSI-RS based L1-RSRP in this clause apply to all CSI-RS resources of the serving cell configured for L1-RSRP measurement. The accuracy requirements in Table 10.1.19.2.1-1 are valid under the following conditions: - Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled. - Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.2 for a corresponding Band for each relevant CSI-RS. - The bandwidth of CSI-RS is 48 PRBs and the density is 3. The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.19.2.1-1.

## Measurement performance requirements

NR measurement accuracy is derived under certain number of PRB are used for measurement. That means same measurement accuracy may or may not be usable for NR less than 5 MHz systems. Further measurement accuracy is a performance requirement we can study them during the performance stage.

- RAN4 to agree that measurement performance requirements are studied during the performance stage.

# Summary and Conclusion

In this contribution we have analysed RAN4 aspects for NR less than 5 MHz and made following proposals.

- RAN4 to define CSI-RS based L1-RSRP measurement requirements.

- RAN4 to agree not to define L1-SINR requirements.

SIB and MID reading

- For 15 PRB, at least 2 SSB are required to achieve 99% successful decoding of the SSB Index reading under SNR side condition of -6dB.

- For 12 and 15 PRB for HST scenario, at least 1 SSB is required to achieve 99% successful decoding of the SSB Index reading under SNR side condition of -8dB.

- For 15 PRB punctured SSB, 6 SSB are required to achieve 99% successful decoding of the MIB under SNR side condition of -4dB. Number of SSB required for SNR side condition of -6dB is FFS.

- For 12 and 15 PRB punctured SSB, 2 and 1 SSB are required respectively to achieve 99% successful decoding of the MIB under SNR side condition of -6dB.

RLM:

- For RLM OOS and IS for CBW less than 5MHz, thresholds Qout and Qin should be derived with the assumption the PDCCH is transmitted with the same parameters as TS38.133 8.1.2.1 and 8.1.3.1 even if the PDCCH transmission bandwidth exceeds the transmission channel bandwidth.

- RAN4 can keep the existing SSB based RLM evaluation periods TEvaluate_out_SSB and TEvaluate_in_SSB for CBW less then 5MHz.

- RAN4 should extend the CSI-RS based RLM evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for CBW less then 5MHz, where Mout = [30] (=1.5*20) for OOS and Mout = [15] (=1.5*10) for IS for CSI-RS within the channel bandwidth below 24RB.

Link recovery:

- For BFD for CBW less than 5MHz, threshold Qout_LR, should be derived with the assumption the PDCCH is transmitted with the same parameters as TS38.133 Table 8.5.2.1-1 even if the PDCCH transmission bandwidth exceeds the transmission channel bandwidth.

- RAN4 apply the existing SSB based BFD evaluation period TEvaluate_BFD_SSB for CBW less then 5MHz.

- RAN4 should extend the CSI-RS based BFD evaluation period TEvaluate_BFD_CSI-RS for CBW less then 5MHz, where MBFD = [15] (=1.5*10) for CSI-RS within the channel bandwidth below 24RB.

- RAN4 apply the existing SSB based CBD evaluation period TEvaluate_CBD_SSB for CBW less then 5MHz.

- RAN4 should extend the CSI-RS based CBD evaluation period TEvaluate_CBD_CSI-RS for CBW less then 5MHz, where MBFD = [5] (= 1.5*3) for CSI-RS within the channel bandwidth below 24RB.

Measurements:

- RAN4 apply the existing SSB based L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB for CBW less then 5MHz.

- RAN4 apply the existing CSI-RS based L1-RSRP measurement period TL1-RSRP_Measurement_Period_CSI-RS for CBW less then 5MHz.

- In the performance part, RAN4 should discuss whether to define CSI-RS based L1-RSRP measurement accuracy requirements with CSI-RS is less than 24RB.

- RAN4 to agree that measurement performance requirements are studied during the performance stage.

# References

- RP-221799 “WID on Mobility enhancements”, MediaTek

- R4-2306244

- R4-2306346

- R4-2306400