3GPP TSG RAN WG4 Meeting #107	R4-2309595

Incheon, 22 May 2023 – 26 May 2023

Agenda Item:	8.25.2.3

Source: 	Ericsson

Title:	Discussion on L1/L2 based inter-cell mobility

Document for:	Discussion

# Introduction

In this contribution, we provide our views on the delay aspects to support L1/L2 triggered inter-cell mobility (LTM) for mobility latency reduction.

# Discussion

One of the issues discussed and its respective WF in last meeting is provided below.

Procedure of cell switch

< Way forward >: FFS the following options

  - Option 1: Further discuss whether UE can perform T/F fine tracking (TΔ) if needed at first and then L1/L2/L3 processing (Tprocessing,2) to reduce the interruption time during cell switch.

  - Option 2:  Under the condition that target cell is known, UE can perform T/F fine tracking (TΔ) if needed at first and then L1/L2/L3 processing (Tprocessing,2) to reduce the interruption time during cell switch.

  - Option 3: if T/F fine tracking (TΔ) is needed after receiving cell switch command, UE is not required to perform it before L1/L2/L3 processing (Tprocessing,2).

  - Option 4: UE cannot perform T/F tracking before necessary L1 reconfiguration, which get the UE's L1 ready to receive DL of the target cell. If needed, some other L1 configuration, L2/L3 reconfiguration and L2 reset can be performed during cell switch, i.e., in some cases it can be performed after the T/F tracking.

From the options listed above, it looks like companies have different understanding on the T/F tracking. It was agreed in RAN1 that UE can perform pre-synchronisation to transmit PRACH to the target cells and if we look at the initial transmit timing error requirement for RACH, it is same as UE UL transmit timing error requirement for PUCCH/PUSCH. That means UE should be ready with T/F tracking and no need to perform T/F tracking after receiving the cells switch command.

In last meeting there it was also discussed whether T/F fine tracking involves only SSB monitoring or TRS tracking is needed. Our understanding is UE do not need to receive TRS for receiving the first PDCCH/PDSCH after HO and we do not think anywhere in RRM specification we were using TRS for T/F fine time tracking.

- If UE is configured with pre-synchronisation, UE can perform T/F fine time tracking before the cell switch command and UE do not need extra delay for T/F tracking after receiving cell switch command.

- For common understanding among companies, RAN4 to confirm that T/F fine time tracking is based on SSB and not based on TRS (at least from the requirements point of view).

## Requirements for LTM HO

In last meeting following baseline delay equations are agreed for RACH based and RACH less LTM HO.

- The baseline of RACH-based cell switch delay requirements is Tdelay = Tcmd + Tprocessing / TLTM-processing + Tsearch + T∆ + Tmargin + Tuncertainity /TIU, where Tuncertainity /TIU is the uncertainty in acquiring the first available PRACH occasion in the new cell.

  - FFS: the exact value of each component. Some components can be 0 in certain cases, if agreed.

  - FFS: add/remove/modify other component(s).

- The baseline of RACH-less cell switch delay requirements is Tdelay = Tcmd + Tprocessing,2 / TLTM-processing + Tsearch + T∆ + Tmargin + Tuncertainity/TIU,

  - FFS: the ending point

  - FFS: the exact value of each component. Some components can be 0 in certain cases, if agreed.

  - FFS: add/remove/modify other component(s).

In this contribution we look at individual components of the delay equation.

### UE processing delay (Tprocessing,2 / TLTM-processing)

Legacy processing delay consist of UE software processing (e.g., clearing configuration and loading new configuration) and HW processing (RF retuning and baseband retuning). Before discussing the UE processing delay for LTM, some of the relevant RAN2 agreements are summarised below.

On security update

No security update support in Rel-18 with L1/L2 based mobility.

On Delta Configuration

- Candidate delta configuration is applied on top of the reference configuration to form a complete candidate configuration (FFS if done at cell switch or before the cell switch)

- The complete candidate configuration is applied and replacing the current UE configuration (at the time of reconfiguration execution/cell switch), by a RRC reconfiguration procedure that makes replacements of configuration but doesn’t necessarily reset RLC or PDCP.

- To support reconfigurations that requires reset of RLC PDCP, this should be possible (in principle same a full config)

The UE may perform early decoding and early validity check. FFS whether Early validity check triggers early re-establishment. FFS the possible timing, FFS subset of cells, FFS if need to specify anything or just up to UE impl, FFS if other signalling to notify network is needed.

Following actions may have been completed when UE receive the RRC reconfiguration message including the candidate configuration (delta configuration and potentially a reference configuration)

- ASN.1 decoding and validity check of the candidate configurations. At least related to measurements and pre-synchronisation aspects.

When UE receive cell switch command, compared to legacy HO UE may not need to perform following actions.

- UE do not need to perform security update

- UE do not need to reset L2 (RLC and PDCP)

- UE may not need to reconfigure L1

In our understanding when there is no L2 reset and only L1 reconfiguration, we think this is similar to BWP switching. That means this can be as fast as 3ms. In role change scenario, since UE was receiving from the target cell before, we think the processing delay required is 0ms.

Different scenarios may have different requirements we think we can define different groups of UE processing delay and in which scenarios UE can apply which delay can be further discussed. As a starting point we think we can define 4 groups of UE processing.

For group 1 UE processing delay can be 0ms

For group 2 UE processing delay can be 5ms

For group 3 UE processing delay can be 10ms

For group 4 UE processing delay can be 20ms/40ms (legacy processing delay)

- RAN4 to define 4 groups of UE processing delays and the delay values for different groups is [0ms, 5ms, 10ms, 20ms/40ms]

- In which scenarios which delay being applied is FFS.

### Cell search (Tsearch)

If UE was configured to perform pre-sync to a candidate cell, before performing DL synchronization UE need to perform cell search. That means for a UE which can perform pre-sync, we can assume that it can perform cell search also during or before pre-synchronization procedure. That means UE do not need extra delay for cell search for this cell. If the UE has not performed pre-sync or UE is not capable of performing pre-sync, then UE need to perform cell search and legacy requirements can be reused.

- If UE has performed pre-sync to the target cell before receiving cell switch command, cell search is 0.

- If a UE has not performed pre-sync on the target cell, legacy cell search delay can be reused.

### Fine timing (T∆ + Tmargin)

Similar to the cell search, for fine timing acquisition also, if UE has performed pre-sync to a candidate cell, UE have already acquired DL synchronization to the candidate cell and UE do not need extra samples for fine timing acquisition for this cell. If the UE has not performed pre-sync or UE is not capable of performing pre-sync, then UE need to perform fine timing acquisition and legacy requirements can be reused.

- If UE has performed pre-sync to the target cell before receiving cell switch command, T∆ + Tmargin is 0.

- If a UE has not performed pre-sync on the target cell, legacy fine timing delay can be reused.

### Delay uncertainty for endpoint

If the RACH needs to be performed after receiving cell switch command, the ending point of LTM cell switch is PRACH preamble transmission like the legacy HO. If UE received TA value in the cell switch command or before cell switch command or do not need to perform RACH, ending point of the cell switch is agreed as RRCReconfigurationComplete by RAN2. How does UE receive UL grant to transmit complete message is FFS in RA1/2.

- If UE needs to perform RACH after cell switch command, TIU can be same as legacy

- If UE need not perform RACH after cell switch command, Tuncertainity is delay uncertainty to transmit RRCReconfigurationComplete.

Other issue discussed was TCI application time in HO delay. Since Ran1 is still discussing the framework it may be early to discuss the TCI application time in delay requirements.

TCI state switching time

< Way forward >: FFS the following options

  - Option 1: no need to add TCI state switching time in cell switch delay.

  - Option 2:

    - If DL TCI state switch is included in cell switch command, there is no extra delay if fine time tracking is already included in cell switch delay requirement or obtained by pre DL-sync.

    - If UL TCI state switch is included in cell switch command, possible extra delay is expected due to non-maintained PL-RS. Further discuss whether to consider non maintained PL-RS case.

  - Option 3: TCI state switching delay shall be considered depending on the scenario. The design requires LTM mobility measurements to be discussed first.

  - Option 4:

    - If TCI is indicated within cell switch command, TCI switching time is added in the cell switch delay. However, from the perspective of optimising RRM requirement, the TCI switching time can be precluded from cell switch delay if RAN4 only define requirements for the case TCI is not indicated within cell switch command.

    - Re-use the conclusions of R17 feMIMO for the case when the UL TCI is indicated in cell switch command.

  - Option 5: If SSB based fine synchronization is performed before cell switch, TCI state switch delay may not be needed.

- RAN4 to wait for further RAN1 progress for TCI application time.

# Summary and Conclusion

In this contribution we have analysed RAN4 aspects for L1/L2 based inter-cell mobility and made following proposals.

- If UE is configured with pre-synchronisation, UE can perform T/F fine time tracking before the cell switch command and UE do not need extra delay for T/F tracking after receiving cell switch command.

- For common understanding among companies, RAN4 to confirm that T/F fine time tracking is based on SSB and not based on TRS (at least from the requirements point of view).

- RAN4 to define 4 groups of UE processing delays and the delay values for different groups is [0ms, 5ms, 10ms, 20ms/40ms]

- In which scenarios which delay being applied is FFS.

- If UE has performed pre-sync to the target cell before receiving cell switch command, cell search is 0.

- If a UE has not performed pre-sync on the target cell, legacy cell search delay can be reused

- If UE has performed pre-sync to the target cell before receiving cell switch command, T∆ + Tmargin is 0.

- If a UE has not performed pre-sync on the target cell, legacy fine timing delay can be reused.

- If UE needs to perform RACH after cell switch command, TIU can be same as legacy

- If UE need not perform RACH after cell switch command, Tuncertainity is delay uncertainty to transmit RRCReconfigurationComplete.

- RAN4 to wait for further RAN1 progress for TCI application time.

# References

- RP-221799 “WID on Mobility enhancements”, MediaTek

- R1-2302194

- R1-2304276

- R4-2306395

- R4-2306249