3GPP TSG RAN WG4 Meeting #107	R4-2309599

Incheon, 22 May 2023 – 26 May 2023

Agenda Item:	8.35.4

Source: 	Ericsson

Title:	Discussion on SSB less SCell activation

Document for:	Discussion

# Introduction

In this contribution, we provide our views on SSB less SCell activation.

# Discussion

In last meeting following scenarios are agreed for study of SSB less SCell operation in FR1 inter-band collocated CA.

  - The following scenarios are considered for inter-band SSB-less SCell requirements definition

    - Scenario 1: No SSB but with TRS transmission configured on the SSB-less SCell

      - FFS whether to consider requirements based on TRS transmission during and/or after SCell activation

    - Scenario 2: No SSB and no TRS transmission configured on the SSB-less SCell

    - Scenario 2a: No DL transmission but with UL reception at the NW side on the SSB-less SCell.

    - FFS if requirements will be defined for all or a subset of scenarios subject to feasibility analysis

Further following parameters ranges are agreed to be studied for the SSB less SCell activation to work.

    - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell

    - The difference of the reception power with the FR1 inter-band active serving cell

    - TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell)

    - Frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations

## Receive time difference

In general, we can represent receive time difference (RTD) using following equation.

RTD= TAE+ ∆RF_propagation_difference +∆Channel_dispersion

Where:

- TAE is time alignment error.

- ∆RF_propagation_difference is time of arrival difference of DL signals.

- ∆Channel_dispersionis the radio channel delay dispersion of signals

As per current specification shown below, the time alignment error for inter-band CA is 3µs.

For MIMO transmission, at each carrier frequency, TAE shall not exceed 65 ns.

For intra-band contiguous carrier aggregation, with or without MIMO, TAE shall not exceed 260ns.

For intra-band non-contiguous carrier aggregation, with or without MIMO, TAE shall not exceed 3µs.

For inter-band carrier aggregation, with or without MIMO, TAE shall not exceed 3µs.

The time alignment error requirements for NB-IoT are specified in TS 36.104 [13] clause 6.5.3.

Even when the cells are co-located, the RF propagation difference could be different for different bands due to different channel characteristics and due to dependent deployment environment. RF propagation difference and radio channel delay dispersion delay may look different for the same band combination based on the deployment scenario considered.  These two factors combined can be considered as δ1. With existing TAE requirement, the expected RTD is 3µs+ δ1. The main role of RTD in SSB less SCell operation is whether UE can perform timing acquisition for a given RTD value. To analyse whether UE can perform timing acquisition or not we need to understand the timing acquisition assumption in legacy SCell activation. As per our understanding, in legacy SCell activation, UE acquire the timing in two steps called coarse timing and fine timing. With RTD of 3µs+ δ1, we look at whether SSB less SCell operation work for scenario 1, 2 and 2a. Which means we look at whether UE can acquire coarse and fine timing for different scenarios.

Scenario 1: No SSB and TRS transmission present

In last meeting, for scenario 1 is agreed as a scenario where there is no SSB transmission from SCell, but TRS can be assumed to be transmitted. However, there is no agreement on whether TRS can be used for SSB less SCell activation. We assume TRS transmission during the SCell activation for further analysis.

Since there is no timing requirement for coarse timing acquisition in legacy SCell activation, we can think of coarse timing as an assistance information for UE to acquire fine timing using TRS.   For SSB, UE can acquire timing of symbol boundary using time domain correlation but if TRS is used as a reference resource for timing acquisition, UE need to acquire timing in frequency domain. That means reference cell and the TRS for timing acquisition should be within the CP so that UE can estimate the symbol boundary containing the TRS by decoding it in frequency domain. for UE could achieve the coarse timing from the active serving cell on the other band.

For 15kHz SCS, CP is 4.7 us and unless δ1 is more than 1.6µs (RTD=3µs+ δ1), UE can determine coarse and fine timing acquisition till RTD of 4.7µs.

- SSB less SCell activation would work when RTD is less than 4.7µs for Scenario 1 when SCS of reference cell is 15kHz.

For 30kHz SCS, cyclic prefix is 2.35µs and RTD has to be within the 2.35µs for UE to estimate timing using TRS in frequency domain under current implementation.

- SSB less SCell activation would work when RTD is less than 2.35µs for Scenario 1 when SCS of reference cell is 30kHz.

Having said that, if NES feature has to be implemented in legacy deployments, RAN4 should look at potential solutions for UE to acquire fine timing when the TAE is 3µs and CP is 2.35µs.

- RAN4 to study potential solutions for UE to acquire timing when the TAE is 3µs and reference cells SSB is 30kHz.

Scenario 2 and 2a: No SSB and No TRS transmission

Before analysing under what conditions, the SSB less SCell activation would work, we look at the legacy SSB less requirements for intra-band CA case.

If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE is not provided with SSB configuration (absoluteFrequencySSB) nor SMTC configuration for the target SCell, Tactivation_time is 3 ms for UE supporting scellWithoutSSB, provided - The RTD between the target SCell and the contiguous active serving cell is within within ±260ns, and - The difference of the reception power with the contiguous active serving cell is <= 6dB, and - The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band.

Based on the highlighted sentence above, in legacy operation UE implementations may have assumed to have TRS signals activated even for SSB less SCell operation. Before studying under what conditions SSB less SCell will work, first we need to study whether UE can work without TRS being transmitted at all.

- RAN4 to study the UEs dependency on TRS for data reception and whether no TRS would work or not.

If UE do not work without any TRS transmission at all from cell at any stage of the operation, NW need to transmit TRS at least occasionally. If the NW must transmit TRS occasionally for UE to work, we do not see a reason why TRS should not be scheduled during the SCell activation. If occasional TRS transmission is allowed from the NW, we think scenario 2 and scenario 2a are not valid scenarios.

- If UE cannot work without any TRS at all during any stage of the data transmission, RAN4 to agree that scenario 2 and 2a are not a valid scenario.

Another aspect RAN4 need to study is UL timing at the BS for the SSB less SCell when UE using active PCell or other active serving cell timing to derive SCell UL timing. If UE is deriving SCell UL timing from the PCell, and if they are of different SCS, due to TA loop errors, it may have negative impact at the SCell. e.g., if PCell is at 15kHz and SCell at 30kHz, due to the smaller CP at 30kHz and larger TA errors at 15kHz, SCell.

- If SSB less SCell supports UL, RAN4 to study UL timing requirements of SSB less SCell.

## Reception power difference

When SCell do not contain SSB, UE need to derive AGC from the other active serving cell. For the UE to use SSB on one cell to determine AGC on the other cell, either both the cells should be transmitting with same transmit power or the power difference between the cells should be within certain range so that deviation or difference in power levels between the cells can be handled by the UE with single AGC setting.

For studying the power difference UE could handle, we may need to assume different UE architectures like using single RF for receiving on both the cells or using different RF chains for receiving separately on each cells. If UE is using single RF to receive on both the cells, from UE perspective maximum power difference UE may be able to handle is similar to intra-band CA power difference. However, from the NW side, NW may be using same or different RU based on the band combination.

- For a UE using single RF chain, and NW using same RU, for scenario 1, 2 or 2a, the maximum power difference is 6.0dB. How this can be captured in spec can be FFS.

In case of dual RF chain are used at the UE, UE may be capable of handling higher power differences and RAN4 should further study the maximum value UE can handle and the method to determine the AGC when UE using different RF chains. One method of obtaining AGC for dual RF chains may be using TRS transmission. RAN4 to study A-TRS being used for AGC settling at the UE.

- For a UE using dual RF chains, the maximum power difference UE can handle is FFS.

- For a UE using dual RF chains, RAN4 to study whether UE can use TRS transmission in scenario 1 for computing AGC.

Similar to the arguments in the previous section, if UE do not work without any TRS transmission at all at any stage of the operation, NW need to transmit TRS at least occasionally. If the NW must transmit TRS occasionally for UE to work, we do not see a reason why TRS should not be scheduled during the SCell activation and whether TRS can be used AGC can be further studied.

## TCI state information

In FR1, indicating spatial relation information through TCI state activation is is not supported. The type of QCL relation UE can be configured with in FR1 is QCL type A/B/C. Further, since the two cells are in different band, we do not think NW can configure QCL type A between cells of different bands. Only QCL information UE can be configured is QCL type C as the cells are collocated. However, we are not sure in what sense this QCL type C information can be used at UE for SCell activation or operation.  UE need to know which cell to be used as reference cell to derive the fine timing of the cell. Since QCL type C do not indicate the level of sync between the two cells, we prefer using other method to indicate the reference cell to be used for deriving the coarse timing (and fine timing can be derived based on the TRS transmission).

Hence, we think TCI state indication may not be needed for SSB less SCell activation.

- TCI state indication may not be needed to complete the SSB less SCell activation.

## Band combinations

SSB less operation is possible or not depends on gNB RU implementation and UE RF architecture to receive on these two bands. Whether the UE supports SSB-less operation for a target SCell activation or not mainly depends on UE’s implementation and RF architecture to receive on these bands. In other words, whether UE can derive or use other cells timing may depend on how these bands are received at the UE (i.e., using same RF or different RF). When different UEs may support SSB less operation based on their RF architecture, unless NW knows more information from the UE, NW do not know whether UE can support SSB less operation or not in the target SCell.

On the other hand, even if the UE supports or is capable of the SSB-less SCell operation for an inter-band CA combination, NW may not support it due to different Radio units (RU) design for different bands at different deployment locations. When different RU are used, based on how the RU are connected to baseband unit, transmit timing difference that can be achieved at different RU is different.  Unless NW knows more information about the UE SSB less operation, NW cannot use SSB less SCell operation at different deployment locations. Thus, RAN4 needs to study how to allow NW to understand which bands UE can support SSB less operation.

- RAN4 to study how to allow NW to understand which bands UE can support for SSB less SCell operation.

# Summary and Conclusion

In this contribution we have provided our views on SSB less SCell operation and made following proposals:

- SSB less SCell activation would work when RTD is less than 4.7µs for Scenario 1 when SCS of reference cell is 15kHz.

- SSB less SCell activation would work when RTD is less than 2.35µs for Scenario 1 when SCS of reference cell is 30kHz.

- RAN4 to study potential solutions for UE to acquire timing when the TAE is 3µs and reference cells SSB is 30kHz.

- RAN4 to study the UEs dependency on TRS for data reception and whether no TRS would work or not.

- If UE cannot work without any TRS at all during any stage of the data transmission, RAN4 to agree that scenario 2 and 2a are not a valid scenario.

- If SSB less SCell supports UL, RAN4 to study UL timing requirements of SSB less SCell.

- For a UE using single RF chain, and NW using same RU, for scenario 1, 2 or 2a, the maximum power difference is 6.0dB. How this can be captured in spec can be FFS.

- For a UE using dual RF chains, the maximum power difference UE can handle is FFS.

- For a UE using dual RF chains, RAN4 to study whether UE can use TRS transmission in scenario 1 for computing AGC.

- TCI state indication may not be needed to complete the SSB less SCell activation.

- RAN4 to study how to allow NW to understand which bands UE can support for SSB less SCell operation.

# References

- R4-2306260