3GPP TSG-RAN WG4 Meeting #107                          			R4-2309629

# Incheon, KR, May 22 – May 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR |  | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running Draft CR to TS 36.101 on additional OOBE requirements for aerial UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_UAV_enh |  |  |  |  |  |  |  |  | 2023-05-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This is a running draft CR to collect all the agreement on the implementation on the RAN4 objectives captured in the LTE_UAV_enh WI (latest WID in RP-230783). Technical content of this version of the Draft CR is the same as the one Endorsed in R4-2306644 during RAN4#106bis-e meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following aspects were captured: - Section 3 updates - 4.3A: New suffix H for the Aerial UE requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementation of regulatory ECC Decision (22)07 would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.3, 4.3A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.101-1 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply in the case of a single component carrier. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Aerial UE: TBD

Aggregated Channel Bandwidth: The RF bandwidth in which a UE transmits and receives multiple contiguously aggregated carriers.

------------------------------ Next modified section ------------------------------

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ABS	Almost Blank Subframe

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

AWGN	Additive White Gaussian Noise

BS	Base Station

CA	Carrier Aggregation

CA_X	Intra-band contiguous CA of component carriers in one sub-block within Band X where X is the applicable E-UTRA operating band

CA_X-X	Intra-band non-contiguous CA of component carriers in two sub-blocks within Band X where X is the applicable E-UTRA operating band

CA_X-X-X	Intra-band non-contiguous CA of component carriers in three sub-blocks within Band X where X is the applicable E-UTRA operating band

CA_X-X-X-X	Intra-band non-contiguous CA of component carriers in four sub-blocks within Band X where X is the applicable E-UTRA operating band

CA_X-Y	Inter-band CA of component carrier(s) in one sub-block within Band X and component carrier(s) in one sub-block within Band Y where X and Y are the applicable E-UTRA operating band

CA_X-X-Y	CA of component carriers in two sub-blocks within Band X and component carrier(s) in one sub-block within Band Y where X and Y are the applicable E-UTRA operating bands

CC	Component Carriers

CG	Carrier Group

CPE	Customer Premise Equipment

CPE_X	Customer Premise Equipment for E-UTRA operating band X

CW	Continuous Wave

DC	Dual Connectivity

DC_X-Y	Inter-band DC of component carrier(s) in one sub-block within Band X and component carrier(s) in one sub-block within Band Y where X and Y are the applicable E-UTRA operating band

DL	Downlink

DIP	Dominant Interferer Proportion

EARFCN 	E-UTRA Absolute Radio Frequency Channel Number

EIRP	Effective Isotropic Radiated Power

EPRE	Energy Per Resource Element

E-UTRA 	Evolved UMTS Terrestrial Radio Access

EUTRAN	Evolved UMTS Terrestrial Radio Access Network

EVM	Error Vector Magnitude

FDD	Frequency Division Duplex

FRC	Fixed Reference Channel

GNSS	Global Navigation Satellite Systems

HD	Half-Duplex for Sidelink Operation

HD-FDD	Half- Duplex FDD

ITS	Intelligent Transportation Systems

MCS	Modulation and Coding Scheme

MCG	Master Cell Group

MetSat	Meteorological Satellite

MOP	Maximum Output Power

MPR	Maximum Power Reduction

MSD	Maximum Sensitivity Degradation

OCNG	OFDMA Channel Noise Generator

OFDMA	Orthogonal Frequency Division Multiple Access

OOB	Out-of-band

PA	Power Amplifier

PCC	Primary Component Carrier

P-MPR	Power Management Maximum Power Reduction

ProSe	Proximity-based Services

PSBCH	Physical Sidelink Broadcast CHannel

PSCCH	Physical Sidelink Control CHannel

PSDCH	Physical Sidelink Discovery CHannel

PSS	Primary Synchronization Signal

PSS_RA	PSS-to-RS EPRE ratio for the channel PSS

SSSS	Secondary Sidelink Synchronization Signal

PSSCH	Physical Sidelink Shared CHannel

PSSS	Primary Sidelink Synchronization Signal

RAS	Radar Absorbing Structure

RE	Resource Element

REFSENS	Reference Sensitivity power level

r.m.s	Root Mean Square

SCC	Secondary Component Carrier

SCG	Secondary Cell Group

SINR	Signal-to-Interference-and-Noise Ratio

SNR	Signal-to-Noise Ratio

SSS	Secondary Synchronization Signal

SSS_RA	SSS-to-RS EPRE ratio for the channel

SSSSSSS	Secondary Sidelink Synchronization Signal

TDD	Time Division Duplex

UAV	Uncrewed Aerial Vehicle

UE	User Equipment

UL	Uplink

UL-MIMO	Up Link Multiple Antenna transmission

UMTS	Universal Mobile Telecommunications System

UTRA	UMTS Terrestrial Radio Access

UTRAN	UMTS Terrestrial Radio Access Network

V2X	Vehicle to Everything

xCH_RA	xCH-to-RS EPRE ratio for the channel xCH in all transmitted OFDM symbols not containing cell-specific RS

xCH_RB	xCH-to-RS EPRE ratio for the channel xCH in all transmitted OFDM symbols containing cell-specific RS

------------------------------ Next modified section ------------------------------

## 4.3A	Applicability of minimum requirements (CA, UL-MIMO, ProSe, Dual Connectivity, UE category 0, UE category M1, UE category M2, UE category 1bis, UE category NB1 and NB2, V2X Communication, MBMS UE, Aerial UE)

The requirements in clauses 5, 6 and 7 which are specific to CA, UL-MIMO, ProSe, Dual Connectivity,   UE category 0, UE category M1, UE category M2, UE category 1bis, UE category NB1 and NB2, V2X Communication or Aerial UE are specified as suffix A, B, C, D, E, F and G  to H, where:

a)	Suffix A additional requirements need to support CA

b)	Suffix B additional requirements need to support UL-MIMO

c)	Suffix C additional requirements need to support Dual Connectivity

d)	Suffix D additional requirements need to support ProSe

e)	Suffix E additional requirements need to support UE category 0, category M1, category M2, and category 1bis

f)	Suffix F additional requirements need to support UE category NB1 and NB2

g)	Suffix G additional requirements need to support V2X Communication

h)	Suffix H additional requirements need to support Aerial UE

A terminal which supports the above features needs to meet both the general requirements and the additional requirement applicable to the additional subclause (suffix A to H) in clauses 5, 6 and 7. Where there is a difference in requirement between the general requirements and the additional subclause requirements (suffix A to H) in clauses 5, 6 and 7, the tighter requirements are applicable unless stated otherwise in the additional subclause.

A terminal which supports more than one feature (CA, UL-MIMO, ProSe, Dual Connectivity, UE category 0, UE category M1, UE category M2, UE category 1bis, UE category NB1 and NB2, V2X Communication, or Aerial UE) in clauses 5, 6 and 7 shall meet all of the separate corresponding requirements.

For a terminal supporting CA, compliance with minimum requirements for non-contiguous intra-band carrier aggregation in any given operating band does not imply compliance with minimum requirements for contiguous intra-band carrier aggregation in the same operating band.

For a terminal supporting CA, compliance with minimum requirements for contiguous intra-band carrier aggregation in any given operating band does not imply compliance with minimum requirements for non- contiguous intra-band carrier aggregation in the same operating band.

A terminal which supports a DL CA configuration shall support all the lower order fallback DL CA combinations and it shall support at least one bandwidth combination set for each of the constituent lower order DL combinations containing all the bandwidths specified within each specific combination set of the upper order DL combination.

A terminal which supports CA, for each supported CA configuration, shall support Pcell transmissions in each of the aggregated Component Carriers unless indicated otherwise in clause 5.6A.1.

Terminal supporting Dual Connectivity configuration shall meet the minimum requirements for corresponding CA configuration (suffix A), unless otherwise specified.

For a terminal that supports ProSe Direct Communication and/or ProSe Direct Discovery, the minimum requirements are applicable when

-	the UE is associated with a serving cell on the ProSe carrier, or

-	the UE is not associated with a serving cell on the ProSe carrier and is provisioned with the preconfigured radio parameters for ProSe Direct Communications and/or ProSe Direct Discovery that are associated with known Geographical Area, or

-	the UE is associated with a serving cell on a carrier different than the ProSe carrier, and the radio parameters for ProSe Direct Discovery on the ProSe carrier are provided by the serving cell, or

-	the UE is associated with a serving cell on a carrier different than the ProSe carrier, and has a non-serving cell selected on the ProSe carrier that supports ProSe Direct Discovery and/or ProSe Direct Communication.

When the ProSe UE is not associated with a serving cell on the ProSe carrier, and the UE does not have knowledge of its geographical area, or is provisioned with preconfigured radio parameters that are not associated with any Geographical Area, ProSe transmissions are not allowed, and the requirements in Section 6.3.3D apply.

A terminal that supports simultaneous E-UTRA ProSe sidelink transmissions and E-UTRA uplink transmissions for the inter-band E-UTRA ProSe/E-UTRA bands specified in Table 5.5D-2, shall meet the minimum requirements for the corresponding inter-band UL CA configuration (suffix A), unless otherwise specified. For transmitter characteristics specified in clause 6, the terminal is required to meet the conformance tests for the corresponding inter-band UL CA configuration and is not required to be retested with simultaneous E-UTRA ProSe sidelink and E-UTRA uplink transmissions.

A terminal that supports E-UTRA V2X intra-band multi-carrier operation including carrier aggregation for the band specified in Table 5.5G-3, shall meet the corresponding transmitter characteristics requirements (in subclauses with suffix G in Section 6) only when there are multiple active transmissions on all of the configured carrier components. When there is only one active transmission on one of the configured carrier components, the corresponding requirements for V2X single carrier operation apply for the corresponding active carrier component.

A terminal which supports MBMS (including 15 kHz, 7.5 kHz ,1.25 kHz, 2.5 kHz and 0.37 kHz subcarrier spacing), shall meet the minimum requirements in clauses 5 and 7. A terminal which supports MBMS is not required to support all kinds of subcarrier spacing.

A terminal that supports multiple TTI patterns in different carriers, different TTI patterns can only be used when the carriers are aggregated in inter-band manner. For intra-band carrier aggregation, only same TTI patterns and same TAG are allowed in aggregated carriers.

------------------------------ End of modified section ------------------------------