3GPP TSG-RAN WG4 Meeting #107                          			R4-2309634

# Incheon, KR, May 22 – May 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.941 | CR | 0037 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 37.941: FR2-2 MU budget calculations implementation for RX requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Perf, TEI17 |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Implementation of the FR2-2 RX MU budget calculations, expanded uncertainty and TT values. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Implementation of the FR2-2 RX MU budget calculations - Implementation of the FR2-2 RX expanded uncertainty values Implementation of the FR2-2 RX TT values |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Derivation of FR2-2 RX MU budget calculations would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.3.4, 10.2.7, 10.2.8, 12.3.3, 17, 18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

Table 10.2.3.4-1: CATR MU value derivation for OTA sensitivity measurement, FR2

| UID | Uncertainty source | Uncertainty value |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 24.25 < f < 29.5 GHz | 37 < f < 43.5 GHz | 52.6 < f ≤ 71 GHz |  |  |  | 24.25 < f < 29.5 GHz | 37 < f < 43.5 GHz | 52.6 < f ≤ 71 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |
| B2-1a |  | Misalignment and pointing error of BS |  |  | 0.2 |  | 0.2 |  | 0.2 |  | Exp. normal |  |  |  | 2.00 |  |  | 1 |  |  | 0.10 | 0.10 |  |  | 0.10 |
| B2-2 |  | Standing wave between BS and test range antenna |  |  | 0.21 |  | 0.21 |  | 0.21 |  | U-shaped |  |  |  | 1.41 |  |  | 1 |  |  | 0.15 | 0.15 |  |  | 0.15 |
| C1-2 |  | Uncertainty of the RF signal generator |  |  | 0.9 |  | 0.9 |  | 0.98 |  | Gaussian |  |  |  | 1.00 |  |  | 1 |  |  | 0.90 | 0.90 |  |  | 0.98 |
| B2-3 |  | RF leakage & dynamic range, test range antenna cable connector terminated. |  |  | 0.01 |  | 0.01 |  | 0.01 |  | Gaussian |  |  |  | 1.00 |  |  | 1 |  |  | 0.01 | 0.01 |  |  | 0.01 |
| B2-4a |  | QZ ripple experienced by BS |  |  | 0.4 |  | 0.4 |  | 0.4 |  | Gaussian |  |  |  | 1.00 |  |  | 1 |  |  | 0.40 | 0.40 |  |  | 0.40 |
| B2-9 |  | Miscellaneous uncertainty |  |  | 0 |  | 0 |  | 0 |  | Gaussian |  |  |  | 1.00 |  |  | 1 |  |  | 0.00 | 0.00 |  |  | 0.00 |
| MU pa |  | MUPA Uncertainty due to use of PA |  |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.00 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| C1-3 |  | Uncertainty of the network analyzer |  |  | 0.3 |  | 0.3 |  | 0.85 |  | Gaussian |  |  |  | 1.00 |  |  | 1 |  |  | 0.30 | 0.30 |  |  | 0.85 |
| B2-5 |  | Mismatch of transmit chain (i.e. between transmitting measurement antenna and BS) |  |  | 0.43 |  | 0.57 |  | 0.85 |  | U-shaped |  |  |  | 1.41 |  |  | 1 |  |  | 0.30 | 0.40 |  |  | 0.60 |
| B2-6 |  | Insertion loss of transmitter chain |  |  | 0.12 |  | 0.12 |  | 0.12 |  | Rectangular |  |  |  | 1.73 |  |  | 1 |  |  | 0.07 | 0.07 |  |  | 0.07 |
| B2-7 |  | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  | 0.01 |  | 0.01 |  | 0.01 |  | Gaussian |  |  |  | 1.00 |  |  | 1 |  |  | 0.01 | 0.01 |  |  | 0.01 |
| B2-8 |  | Influence of the calibration antenna feed cable |  |  | 0.21 |  | 0.29 |  | 0.29 |  | U-shaped |  |  |  | 1.41 |  |  | 1 |  |  | 0.15 | 0.21 |  |  | 0.21 |
| C1-4 |  | Uncertainty of the absolute gain of the reference antenna |  |  | 0.52 |  | 0.52 |  | 0.52 |  | Rectangular |  |  |  | 1.73 |  |  | 1 |  |  | 0.30 | 0.30 |  |  | 0.30 |
| B2-11 |  | Misalignment positioning system |  |  | 0 |  | 0 |  | 0 |  | Exp. normal |  |  |  | 2.00 |  |  | 1 |  |  | 0.00 | 0.00 |  |  | 0.00 |
| B2-4b |  | QZ ripple experienced by calibration antenna |  |  | 0.1 |  | 0.1 |  | 0.1 |  | Gaussian |  |  |  | 1.00 |  |  | 1 |  |  | 0.10 | 0.10 |  |  | 0.10 |
| B2-10 |  | Rotary joints |  |  | 0 |  | 0 |  | 0 |  | U-shaped |  |  |  | 1.41 |  |  | 1 |  |  | 0.00 | 0.00 |  |  | 0.00 |
| B2-1b |  | Misalignment and pointing error of calibration antenna |  |  | 0 |  | 0 |  | 0 |  | Exp. normal |  |  |  | 2.00 |  |  | 1 |  |  | 0.00 | 0.00 |  |  | 0.00 |
| B2-12 |  | Standing wave between SGH and test range antenna |  |  | 0.09 |  | 0.09 |  | 0.09 |  | U-shaped |  |  |  | 1.41 |  |  | 1 |  |  | 0.06 | 0.06 |  |  | 0.06 |
| B2-13 |  | Switching uncertainty |  |  | 0.1 |  | 0.1 |  | 0.1 |  | Rectangular |  |  |  | 1.73 |  |  | 1 |  |  | 0.06 | 0.06 |  |  | 0.06 |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.15 |  |  | 1.19 |  |  | 1.55 |  |  |  |
| Expanded uncertainty (1.96σ – confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.25 |  |  | 2.33 |  |  | 3.03 |  |  |  |

------------------------------ Next modified section ------------------------------

Table 10.2.7-2: OTA test system specific measurement uncertainty values for the OTA sensitivity in Normal test conditions, FR2

|  | Expanded uncertainty (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | 24.25 GHz < f ≤ 29.5 GHz |  | 37 GHz < f ≤ 43.5 GHz | 43.5 GHz < f ≤ 48.2 GHz | 52.6 < f ≤ 71 GHz |
| Indoor Anechoic Chamber |  | 2.33 |  | 2.46 |  |  |
| Compact Antenna Test Range |  | 2.25 |  | 2.33 |  | 3.03 |
| Common maximum accepted test system uncertainty |  | 2.4 |  | 2.4 | 3.5 | 3.0 |

------------------------------ Next modified section ------------------------------

Table 10.2.8-2: Test Tolerance values for the OTA sensitivity, Normal test conditions

|  | 24.25 GHz < f ≤ 29.5 GHz | 37 GHz < f ≤ 43.5 GHz | 43.5 GHz < f ≤ 48.2 GHz | 52.6 < f ≤ 71 GHz |
| --- | --- | --- | --- | --- |
| Test Tolerance (dB) | 2.4 | 2.4 | 3.5 | 3.0 |

------------------------------ Next modified section ------------------------------

### 12.3.3	Maximum accepted test system uncertainty

The TRP MU is very similar to that for the transmitter mandatory spurious emissions. However, the receiver requirements are at a much lower power level so TRP calculation may be affected by the noise floor of the measurement system.

For FR1 the following MU are calculated.

Table 12.3.3-1: Receiver spurious emissions MU values – FR1

|  | Expanded uncertainty (dB) |  |  |
| --- | --- | --- | --- |
|  | 30 MHz< f ≤ 6 GHz | 6 GHz< f ≤ 19 GHz | 19 GHz< f ≤ 26 GHz |
| General Directional Chamber | 2.50 | 4.25 | 4.25 |
| Reverberation Chamber | - | - | - |
| Common maximum accepted test system uncertainty | 2.50 | 4.20 | 4.20 |

For FR2 the

same values are assumed for receiver spurious emissions as for transmitter spurious emissions.

This range can be split into a number of regions:

30 MHz < f ≤ 6 GHz

This region also exists in FR1, the same MU is assumed for FR2 and for FR1.

6 GHz < f ≤ 18 GHz

This is also an FR1 region however the MU values assumed in FR1 is larger than the in-band MU for FR2 which is at a higher frequency. An FR2 BS will likely be smaller than an FR1 BS and hence the chamber can be smaller and the requirements on the quiet zone can be relaxed. In addition, the test equipment is suitable for much higher frequencies (FR2 in band is above the frequency range) implying a low uncertainty. The MU in the region therefore is assumed to be the same as the FR2 in-band MU.

18 GHz < f ≤ 71 GHz

This frequency range covers the FR2 in-band region. The in-band MU budget is found in clause 11.2.

71 GHz < f ≤ 142 GHz

This frequency range is above the in-band region, the measured frequencies are above the measurement frequency of the test equipment and hence a mixer is used to down convert the measured frequency to within the range of the test equipment.

Table 12.3.3-2: Receiver spurious emissions MU values

| Frequency range | Expanded uncertainty (dB) |  |
| --- | --- | --- |
| 30 MHz < f ≤ 6 GHz | 2.5 |  |
| 6 GHz < f ≤ 43.5 GHz | 2.7 |  |
| 43.5 GHz < f ≤ 60 GHz | 5 |  |
| 71 < f ≤ 110 GHz |  | 4.7 |  |
| 110 < f ≤ 142 GHz |  | 4.9 |  |

------------------------------ Next modified section ------------------------------

Table 17-4: RX Measurement Uncertainty values derivation – FR2

| Requirement | Maximum OTA Test System uncertainty | Clause |
| --- | --- | --- |
| OTA reference sensitivity level | ±2.4 dB, 24.25 GHz < f ≤ 29.5 GHz ±2.4 dB, 37 GHz < f ≤ 43.5 GHz ±3.5 dB, 43.5 GHz < f ≤ 48.2 GHz ±3.0 dB (52.6 GHz ≤ f ≤ 71 GHz) | 10.2.7 |
| OTA adjacent channel selectivity | ±3.4 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.4 dB, 37 GHz < f ≤ 43.5 GHz ±5.1 dB, 43.5 GHz < f ≤ 48.2 GHz ±4.0 dB (52.6 GHz ≤ f ≤ 71 GHz) | 10.5.4 |
| OTA in-band blocking (General) | ±3.4 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.4 dB, 37 GHz < f ≤ 43.5 GHz ±5.1 dB, 43.5 GHz < f ≤ 48.2 GHz ±4.0 dB (52.6 GHz ≤ f ≤ 71 GHz) | 10.5.4 |
| OTA out-of-band blocking | ±3.6 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.6 dB, 37 GHz < f ≤ 43.5 GHz ±4.5 dB, 43.5 GHz < f ≤ 48.2 GHz ±4.0 dB (52.6 GHz ≤ f ≤ 142 GHz) | 14.3 |
| OTA receiver spurious emissions | ±2.3 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 43.5 GHz ±5.0 dB, 43.5 GHz < f ≤ (52.6 GHz ±4.7 dB (52.6 GHz < f ≤ 71 GHz) ±4.7 dB (71 GHz < f ≤ 110 GHz) ±5.9 dB (110 GHz < f ≤ 142 GHz) | 12.3.3 |
| OTA receiver intermodulation | ±3.9 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.9 dB, 37 GHz < f ≤ 43.5 GHz ±5.4 dB, 43.5 GHz < f ≤ 48.2 GHz ±4.5 dB (52.6 GHz ≤ f ≤ 71 GHz) | 10.6.4 |
| OTA in-channel selectivity | ±3.4 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.4 dB, 37 GHz < f ≤ 43.5 GHz ±5.1 dB, 43.5 GHz < f ≤ 48.2 GHz ±4.0 dB (52.6 GHz ≤ f ≤ 71 GHz) | 10.7.4 |
| NOTE: Test System uncertainty values are applicable for normal condition unless otherwise stated. |  |  |

------------------------------ Next modified section ------------------------------

Table 18-4: Test Tolerance values derivation, RX FR2

| Requirement | Test Tolerance (TTOTA) | Clause |
| --- | --- | --- |
| OTA reference sensitivity level | 2.4 dB, 24.25 GHz < f ≤ 29.5 GHz 2.4 dB, 37 GHz < f ≤ 43.5 GHz 3.5 dB, 43.5 GHz < f ≤ 48.2 GHz 3.0 dB, 52.6GHz f 71GHz | 10.2.8 |
| OTA adjacent channel selectivity | 0 dB | 10.5.5 |
| OTA in-band blocking (General) | 0 dB | 10.5.5 |
| OTA out-of-band blocking | 0 dB | 14.4 |
| OTA receiver spurious emissions | 0 dB | 12.3.4 |
| OTA receiver intermodulation | 0 dB | 10.6.5 |
| OTA in-channel selectivity | 3.4 dB, 24.25 GHz < f ≤ 29.5 GHz 3.4 dB, 37 GHz < f ≤ 43.5 GHz 5.1 dB, 43.5 GHz < f ≤ 48.2 GHz 4.0 dB, 52.6GHz f 71GHz | 10.7.5 |
| NOTE: TTOTA values are applicable for normal condition unless otherwise stated. |  |  |

------------------------------ End of modified section ------------------------------